3GPP TSG-SA1 Meeting #90	S1-202231

Electronic Meeting, 18 - 22 May 2020	(revision of S1-202146)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.104 | CR | 0044 | rev | 1 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correcting description of communication service status in Clause C.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Orange, Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | cyberCAV |  |  |  |  |  | Date: |  |  | 2020-05-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Communication service availability is shown incorrectly in Annex C.3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correcting Figure C.3-1 where communication service availability was presented incorrectly. Adding application recovery time to the figure. The descriptive text is corrected accodingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect representation of communication service availability in the specification, which could lead to misinterpretation of the requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Note to the editor: The existing figure C.3-1 is deleted and replaced by an udated figure (the first one). For some reason, MS Word does not always show the deleted figure with a strike-through markup. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S1-202146 |  |  |  |  |  |  |  |  |

----------------------------------------------------- Start of the 1st change --------------------------------------------------------------

# C.3	Up time and up state vs. down state and down time

The assessment of periodic deterministic communication services is based on the assessment of successful message transmission over a logical communication link. Message transmission is either:

-	successful, if it is correctly and timely received, or

-	unsuccessful, if it is incorrectly received, lost or untimely.

Up time and down time can be derived from received messages. As far as timely received messages are correct, the logical communication link status is up. If a message loss or an incorrectly or untimely received message is detected the logical communication link status is down. To denote up and down states the terms “up time interval” and “down time interval”, or alternatively “available” and “unavailable” may be used. An example of the relation between logical communication link status, communication service status and application status is presented in Figure C.3-1.

Figure C.3-1: Relation between logical communication link, communication service and application statuses (example with lost messages)

The flow of events in Figure C.3-1 is as follows:

a)	The logical communication link is up and running (blue line is UP). A source device starts sending periodic messages to a target device (orange arrows), on which an automation function (application) is running. The communication service is, from the point of view of the target application, in an up state (violet line is UP) and so is the application (green line is UP).

b)	The logical communication link status changes to down state if it no longer can support end-to-end transmission of the source device's messages to the target device in agreement with the negotiated communication requirements. Once the application on the target device senses the absence (or unsuccessful reception) of expected messages ("Deadline for expected message" in Figure C.3-1), it will wait a pre-set period before it considers the communication service to be unavailable; this is the so-called survival time. The survival time can be expressed as

-		a period or,

-	especially with cyclic traffic, as maximum number of consecutive incorrectly received or lost messages.

c)	If the survival time has been exceeded, both the communication service and the application transition into a down state (violet and green lines change to DOWN in Figure C.3-1). The application will usually take corresponding actions for handling such situations of unavailable communication services. For instance, it will commence an emergency shutdown. Note that this does not imply that the target application is "shut off"; rather it transitions into a pre-defined state, e.g. a safe state. In the safe state, the target application might still listen to incoming packets or may try to send messages to the source application.

d)	Once the logical communication link status is in the up state again (blue line in Figure C.3-1 changes to UP), the communication service state as perceived by the target application will change to the up state. The communication service is thus again perceived as available (violet line changes to UP in Figure C.3-1). The state of the application, however, depends on the counter measures taken by the application. The application might stay in down state if it is in a safe state due to an emergency shutdown. Or, the application may do a recovery and change to up state again. The time needed for the application to return to the up state after the communication service is restored is shown as “Application recovery time” in Figure C.3-1.

The availability of the communication service is calculated using the accumulated down time. For instance, in case the communication service is expected to run for a time T, the unavailability U of the communication service can be calculated as

U=i∆tiT

Where Δti is the length of the i-th downtime interval of the communication service within the time period T. The communication service availability A can then be calculated as

A = 1–U.

----------------------------------------------------- End of 1st change --------------------------------------------------------------