3GPP TSG-SA1 Meeting #91e	S1-203250

Electronic Meeting, 24 August - 2 September 2020	(revision of S1-20xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.832 | CR | 0030 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial Formatting Improvements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Siemens |  |  |  |  |  |  |  |  |  |
| Source to TSG: | WG1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_eCAV |  |  |  |  |  | Date: |  |  | 2020-08-14 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Several editorial problems are caused by wrong formatting. This is especially true for the Table of Contents (ToC). The current Table of Contents of TR 22.832 needs formatting improvements. Furthermore, some alignments in tables are needed. Change requests on formatting and its effects are difficult to describe with Word revision marks. Therefore, many changes are indicated by Word comments. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected formatting of headings and table of contents, alignments in tables. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Table of contents with missing headings and ill-formatted entries; misaligned entries in some tables. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Contents, Introduction, 5.11.6, 5.19, 6.4, C.3, D.1, E, F, Footer |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF PROPOSED CHANGES

Start of Change 1

Contents

Foreword	11

Introduction	11

1	Scope	12

2	References	12

3	Definitions and abbreviations	13

3.1	Definitions	13

3.2	Abbreviations	13

4	Overview	14

4.1	Overview of use cases	14

4.2	Discussion on network performance requirements	14

4.2.1	KPIs from TS 22.104 and TS 22.261	14

4.2.2	KPIs from TS 23.501	15

4.2.3	Relationships between KPIs for 5G system design	15

4.3	Survival time vs. consecutive message loss	16

5	Use cases	17

5.1 	Introduction	17

5.2	Integration of 5G networks with TSN networks (time synchronization)	18

5.2.1	Description	18

5.2.2	Possible locations of sync master/sync device	19

5.2.2.1	General assumption	19

5.2.2.2	Location of sync master	19

5.2.2.3	Location of sync device	19

5.2.3	Integration Scenarios	19

5.2.3.1	General assumption	19

5.2.3.2	5G network on the path of sync messages (one wireless link)	20

5.2.3.3	5G network on the path of sync messages (two wireless links)	20

5.2.3.4	5G network on the path of sync messages (multiple synchronization domains through 5G network)	21

5.2.3.5	5G UE as sync device	21

5.2.3.6	5G UE as sync master	22

5.2.4	General	22

5.2.5	Existing features partly or fully covering the use case functionality	22

5.2.6	Potential New Requirements needed to support the use case	23

5.3	Multiple Working Clock Domains in gNB	23

5.3.1	Description	23

5.3.2	Pre-conditions	24

5.3.3	Service Flows	24

5.3.4	Post-conditions	25

5.3.5	Existing features partly or fully covering the use case functionality	25

5.3.6	Potential New Requirements needed to support the use case	25

5.4	Merging of working clock domains	26

5.4.1	Description	26

5.4.2	Pre-conditions	27

5.4.3	Service Flows	27

5.4.4	Post-conditions	27

5.4.5	Existing features partly or fully covering the use case functionality	28

5.4.6	Potential New Requirements needed to support the use case	28

5.5	Communication monitoring for CAV applications	28

5.5.1	Description	28

5.5.2	Pre-conditions	28

5.5.3	Service Flows	28

5.5.4	Post-conditions	29

5.5.5	Existing features partly or fully covering the use case functionality	29

5.5.6	Potential New Requirements needed to support the use case	29

5.6	Network energy efficiency and resource optimization with application assistance	29

5.6.1	Description	29

5.6.2	Pre-conditions	29

5.6.3	Service Flows	30

5.6.4	Post-conditions	30

5.6.5	Existing features partly or fully covering the use case functionality	30

5.6.6	Potential New Requirements needed to support the use case	30

5.7	Consideration on Communication Service interface	30

5.7.1	Description	30

5.7.2	Pre-conditions	31

5.7.3	Service Flows	31

5.7.4	Post-conditions	31

5.7.5	Existing features partly or fully covering the use case functionality	31

5.7.6	Potential New Requirements needed to support the use case	31

5.8	Mobile operation panel for production lines	31

5.8.1	Description	31

5.8.2	Pre-conditions	33

5.8.3	Service Flows	33

5.8.3.1	Overview	33

5.8.3.2	Establishing connectivity for emergency stop	33

5.8.4	Post-conditions for establishing connectivity for emergency stop	34

5.8.5	Existing features partly or fully covering the use case functionality	34

5.8.6	Potential New Requirements needed to support the use case	34

5.9	Providing vertical positioning for industrial use cases	36

5.9.1	Description	36

5.9.2	Pre-condition	36

5.9.3	Service Flows	36

5.9.4	Post-conditions	36

5.9.5	Potential Requirements	36

5.10	Device-to-device communication in close proximity for CAV applications	37

5.10.1	Description	37

5.10.2	Scenarios	37

5.10.2.1	Support communication in bad & no network coverage area	37

5.10.2.2	High cooperative robots' operation	38

5.10.2.3	Using Device-to-Device communication for traffic offload in factory	39

5.10.3	Service Flows	39

5.10.4	Post-conditions	39

5.10.5	Existing features partly or fully covering the use case functionality	39

5.10.6	Potential New Requirements needed to support the use case	39

5.11	Cooperative carrying of work pieces	39

5.11.1	Description	39

5.11.2	Pre-conditions	40

5.11.3	Service Flows	41

5.11.4	Post-conditions	43

5.11.5	Existing features partly or fully covering the use case functionality	43

5.11.6	Potential New Requirements needed to support the use case	43

5.12	Providing relative positioning information of the device with no or bad network coverage	46

5.12.1	Description	46

5.12.2	Pre-condition	46

5.12.3	Service Flows	46

5.12.4	Post-conditions	46

5.12.5	Existing features partly or fully covering the use case functionality	47

5.12.6	Potential New Requirements needed to support the use case	47

5.13	Service to out-of-coverage UEs for process monitoring and plant asset management	47

5.13.1	Description	47

5.13.2	Pre-conditions	47

5.13.3	Service Flows	47

5.13.4	Post-conditions	47

5.13.5	Existing features partly or fully covering the use case functionality	47

5.13.6	Potential New Requirements needed to support the use case	47

5.14	Non-public networks as private slices	48

5.14.1	Description	48

5.14.2	Pre-conditions	48

5.14.3	Service Flows	48

5.14.4	Post-conditions	48

5.14.5	Existing features partly or fully covering the use case functionality	49

5.14.6	Potential New Requirements needed to support the use case	49

5.15	Flexible manufacturing with modular production systems and mobility	49

5.15.1	Description	49

5.15.2	Pre-conditions	50

5.15.3	Service Flows	50

5.15.4	Post-conditions	50

5.15.5	Existing features partly or fully covering the use case functionality	50

5.15.6	Potential New Requirements needed to support the use case	50

5.16	Control-to-Control communication (Wired to wireless link replacement)	51

5.16.1	Description	51

5.16.2	Pre-conditions	52

5.16.3	Service Flows	52

5.16.4	Post-conditions	52

5.16.5	Existing features partly or fully covering the use case functionality	52

5.16.6	Potential New Requirements needed to support the use case	53

5.17	Redundant Network Paths	53

5.17.1 	Description	53

5.17.2 	Pre-conditions	54

5.17.3	Service Flows	54

5.17.4	Post Conditions	54

5.17.5	Existing Features partly or fully covering the use case functionality	54

5.17.6	Potential New Requirements needed to support the use case	54

5.18	Industrial Wireless Sensors for process and asset monitoring	55

5.18.1	Description	55

5.18.2	Pre-conditions	55

5.18.3	Service Flows	56

5.18.4	Post-conditions	56

5.18.5	Existing features partly or fully covering the use case functionality	56

5.18.6	Potential New Requirements needed to support the use case	57

5.19	Use case for multi-connectivity for robotic automation	58

5.19.1	Description	58

5.19.2	Pre-conditions	58

5.19.3	Service flows	58

5.19.4	Post-conditions	60

5.19.5	Existing features partly or fully covering the use case functionality	60

5.19.6	Potential new requirements needed to support the use case	61

5.20	Time synchronisation budget for 5G system in integrated TSN networks	61

5.20.1	Description	61

5.20.1.1	Integration scenarios and clock synchronicity budget for the 5G system	61

5.20.1.2	Terminology	62

5.20.1.3	Constraints on time synchronization in wired networks	62

5.20.1.4	Determination of time synchronization budget for 5G system	63

5.20.2	Pre-conditions	63

5.20.3	Service Flows	63

5.20.4	Post-conditions	63

5.20.5	Existing features partly or fully covering the use case functionality	63

5.20.6	Potential New Requirements needed to support the use case	64

6	Merged potential service requirements	64

6.1	Industrial Ethernet integration	64

6.2	Non-public networks	65

6.3	Network operation and Maintenance	65

6.4	Network performance requirements	66

6.5	Positioning	71

6.6	Device-to-device/ProSe communication	72

6.6.1	General requirements	72

6.6.2	Related to network performance	72

6.6.3	Related to clock synchronization	72

6.6.4	Related to service continuity	72

6.6.5	Related to indirect communication	72

6.6.6	Relation between merged and use case new potential service requirements	73

7	Conclusions	73

Annex A: Considerations on communication service interface	74

A.1	Overview	74

A.2	Classification of Application Requirements	74

A.2.1	Characteristic parameters for identification of requirement classes	75

A.2.2	Descriptive parameters (Dsp)	75

A.2.3	Quality of Service Parameters (QoSp)	75

A.3	Requirement classification	75

A.3.1	Deterministic Periodic	75

A.3.2	Deterministic Aperiodic	76

A.3.3	Non-Deterministic	76

A.3.4	Real-time and Non-Real-time Classification	76

A.3.5	Other Classifications.	76

A.4	General Use Case for Communication Service Interface	76

A.4.1	General Consideration	76

A.4.2	Pre-conditions	77

A.4.3	Service Flows	77

A.4.4	Post-conditions	78

A.5	Requirements on Communication Service Interface	78

A.5.1	Output block	78

A.5.2	Input block	78

A.5.3	Communication Service Interface consideration for 5G Systems	78

A.5.4	General application & communication service interface format	79

A.6	Communication Service objectives and candidate parameters	79

A.6.1	Communication Service Disengagement	79

A.6.2	Communication Service Setup	80

A.6.3	Communication Service Modification	80

A.6.4	Communication Service Monitoring	80

Annex B: Considerations on absolute and relative positioning	82

Annex C: Industrial communication network architecture	84

C.1	Overview	84

C.2	Use Case 1: Line Controller-to-Controller (L2C) and Controller-to-Controller (C2C) communication	84

C.3	Use Case 2: Controller-to-Device (C2D) communication	85

C.4	Use Case 3: Device to Compute (D2Cmp) Communication	85

Annex D: Communication service availability vs. failure	87

D.1	Description	87

D.2	Communication system parameters and influence quantities	88

Annex E: Requirements mapping use case to merged	89

Annex F: Change history	90

End of Change 1

Start of Change 2

# Introduction

This document studies further enhancements to the 5G system for the support of cyber-physical control applications in vertical domains.

This document collects new use cases and enhanced functionality for communication in automation in vertical domains in clause 5. Potential new 5G service requirements for Rel.17 are derived for each use case.

Use cases and enhanced functionality are collected for the following topics:

-	Industrial Ethernet integration, which includes time synchronization, different time domains, integration scenarios, and support for time-sensitive networking (TSN) in clauses 5.2, 5.3, 5.4, and 5.15;

-	Non-public networks, non-public networks as private slices, and further implications on security for non-public networks in clause 5.14;

-	Network operation and Maintenance in 5G non-public networks for cyber-physical control applications in vertical domains; Enhanced QoS monitoring, communication service and networks diagnostics; Communication service interface between application and 5G systems, e.g. information to the network for setting up communication services for cyber-physical control applications and corresponding monitoring in clauses 5.5, 5.6, 5.7, 5.13, 5.17, and 5.19;

-	Network performance requirements for cyber-physical control applications in vertical domains in clauses 5.8, 5.16, and 5.18;

-	Positioning enhancements, including relative positioning information and vertical directions / dimension for Industrial IoT in clauses 5.9 and 5.12;

-	Device-to-device/ProSe communication for cyber-physical applications in vertical domains in clauses 5.10, 5.11, and 5.13.

The potential new 5G service requirements are consolidated in clause 6.

Additional information of specific concepts of communication in automation in vertical domains and of cyber-physical control applications is given in clause 4 and in several of the annexes. This information is given for clarification and in order to help in correlating the application behaviour and 5G service requirements to the 5G system.

End of Change 2

Start of Change 3

### 5.11.6	Potential New Requirements needed to support the use case

[PR-5.11.6-001]	The 5G system shall be able to support direct wireless communication (ProSe) between a group of UEs for communication with service performance requirements in Table 5.11.6-1.

[PR-5.11.6-002]	The 5G system shall be able to support 5G LAN-type service to UEs using ProSe communication.

[PR-5.11.6-003]	The 5G system shall be able to support time synchronization (working clock domain) between the UEs within the group of UEs connected by ProSe communication.

[PR-5.11.6-004]	The 5G system shall be able to support periodic deterministic communication (both unicast and multicast) with KPIs given in Table 5.11.6-1 between the UEs within the group of UEs connected by ProSe communication.

[PR-5.11.6-005]	The 5G system shall be able to support multicast communication between the UEs within the group of UEs connected by ProSe communication.

[PR-5.11.6-006]	The 5G system shall be able to support mobility of the group of UEs connected by ProSe communication with KPIs given in Table 5.11.6-1 within the 5G network.

[PR-5.11.6-007]	The 5G system shall be able to support service continuity of the group of UEs using ProSe communication with KPIs given in Table 5.11.6-1 between a non-public network and a PLMN (subject to operator policies and agreement between the operators and service providers).

[PR-5.11.6-009]	The 5G system shall be able to support indirect 3GPP communication with KPIs given in Table 5.11.6-1 between UEs out of transmission range by one intermediate UE.

[PR-5.11.6-010]	The 5G system shall be able to support ProSe communication (both unicast and multicast) with KPIs given in Table 5.11.6-1 between UEs up to 50 m distance.

Table 5.11.6-1: KPIs for ProSe communication for cyber-physical control applications

| Use case | Characteristic parameter |  |  |  |  |  | Influence quantity |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.11 – Cooperative carrying of work pieces | Communication service availability: target value in % | Communication service reliability: Mean Time Between Failure | End-to-end latency: maximum | Bit rate | Direction | Message Size [byte] | Transfer interval [ms] (note 2) | Survival time [ms] (note 2) | UE speed [km/h] | # of UEs Connection | Service Area [m³] (note 1) |
| Cooperative carrying – fragile work pieces | 99,9999 % to 99,999999 % | ~ 10 years | < 0.5 * Transfer interval | 2.5 Mbit/s | UE-UE (ProSe communication) | 250 500 with localisation information | > 5 | 0 | ≤6 | 2-8 | 10 x 10 x 5; 50 x 5 x 5 |
|  |  |  |  |  |  |  | >2.5 | Transfer interval |  |  |  |
|  |  |  |  |  |  |  | >1.7 | 2 * Transfer interval |  |  |  |
| Cooperative carrying – elastic work pieces | 99,9999 % to 99,999999 % | ~ 10 years | < 0.5 * Transfer interval | 2.5 Mbit/s | UE-UE (ProSe communication) | 250 500 with localisation information | > 5 | 0 | ≤12 | 2-8 | 10 x 10 x 5; 50 x 5 x 5 |
|  |  |  |  |  |  |  | >2.5 | Transfer interval |  |  |  |
|  |  |  |  |  |  |  | >1.7 | 2 * Transfer interval |  |  |  |
| NOTE 1: Service Area for direct communication. The group of UEs with direct communication might move throughout the whole factory site (up to several km²) NOTE 2: The first value is the application requirement, the other values are the requirement with multiple transmission of the same information (two or tree times respectively). |  |  |  |  |  |  |  |  |  |  |  |

The use case scenario “cooperative carrying – elastic work pieces” allows a larger tolerance in the precision of the movements of the collaborating mobile robots /AGV compared to the use case scenario “cooperative carrying – fragile work pieces”. The larger tolerance in the coordinated movements allows for either faster movement of the work piece or longer transfer intervals (tradeoff between UE speed and transfer interval). A longer transfer interval relaxes the requirement on the maximum end-to-end latency.

The values given in Table 5.11.6-1 are the upper boundary of the required KPIs. There is a broad variety in the actual use cases of cooperative carrying and several parameters and deployment options have some flexibility in their value range or implementation. Relaxed KPI requirements can already satisfy a substantial set of cooperative carrying use cases depending on the actual physical characteristics (e.g. carrying speed, type of carried work piece).

The actual physical use case setting, such as the kind of workpiece to be carried, determines the necessary requirements on the communication service and the possible tradeoffs. Also the employed control algorithms and recovery mechanisms may further relax the necessary requirements and KPIs. Furthermore, the careful design of the mobile robots / AGVs (e.g. placement of UEs) can further improve the reliability of the wireless communication. Tradeoffs are especially possible between the carrying speed and the transfer interval / end-to-end latency as well as the communication service reliability. A reduction of the carrying speed reduces the requirements on the transfer interval / end-to-end latency or on the packet error ratio. The latter is possible, for instance, if the increased time budget is used for more transmissions.

End of Change 3

Start of Change 4

## 5.19	Use case for multi-connectivity for robotic automation

End of Change 4

Start of Change 5

## 6.4	Network performance requirements

This clause contains merged and consolidated potential service requirements related to network performance requirements for cyber-physical control applications in vertical domains.

[MPR-22832-6.4-00a]	The 5G system shall be able to support seamless mobility with handovers between different gNodeBs in the same 5G network without any impact on the safety-critical functions (handover latency ≤ 30 ms).

[MPR-22832-6.4-00b]	The 5G system shall be able to provide periodic deterministic communication with the service performance requirements reported in Tables 6.4-1 and 6.4-2.

[MPR-22832-6.4-00c]	The 5G system shall be able to provide aperiodic deterministic communication with the service performance requirements reported in Tables 6.4-3 and 6.4-4.

[MPR-22832-6.4-00d]	The 5G system shall be able to provide mixed traffic communication with the service performance requirements reported in Table 6.4-5.

[MPR-22832-6.4-00e] 5G System shall be able to support industrial wireless sensors according to the service performance requirements listed in Table 6.4-6.

Table 6.4-1: Periodic deterministic communication service performance requirements

| Use case | Characteristic parameter |  |  |  | Influence quantity |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Communication service availability: target value in % | Communication service reliability: Mean Time Between Failure | End-to-end latency: maximum | Bit rate [bit/s] | Direction | Message Size [byte] | Transfer interval [ms] | Survival time | UE speed | # of UEs Connection | Service Area [m², m³] |
| Mobile Operation Panel: Emergency stop (connectivity availability) | 99,999999 | 1 day | <8 ms | 250 k | Uplink Downlink (note 1) | 40-250 | 8 ms | 16 ms | quasi-static; up to 10 km/h | 2 or more | 30 x 30 |
| Mobile Operation Panel: Safety data stream | 99,99999 | 1 day | <10 ms | < 1 M | Uplink (note 1) | <1 K | 10 ms | ~10 ms | quasi-static; up to 10 km/h | 2 or more | 30 x 30 |
| Mobile Operation Panel: Control to visualization | 99,999999 | 1 day | 10-100 ms | 10 k | Uplink Downlink (note 1) | 10-100 | 10-100 ms | transfer interval | stationary | 2 or more | 100-2000 |
| Mobile Operation Panel: Motion control | 99,999999 | 1 day | <1 ms | 12 M -16 M | Downlink (note 1) | 10-100 | 1 ms | ~1 ms | stationary | 2 or more | 100 |
| Mobile Operation Panel: Haptic feedback data stream | 99,999999 | 1 day | <2 ms | 16 k (UL) 2 M (DL) | Uplink Downlink (note 1) | 50 | 2 ms | ~2 ms | stationary | 2 or more | 100 |
| Cooperative carrying – fragile work pieces |  | 99,9999 to 99,999999 |  | ~ 10 years |  | < 0.5 * transfer interval |  | 2.5 M |  | UE-UE (ProSe communication) |  | 250 500 with localisation information |  | > 5 |  | 0 |  | ≤6 |  | 2-8 |  | 10 x 10 x 5; 50 x 5 x 5 (note 3) |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | >2.5 |  | transfer interval |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | >1.7 (note 2) |  | 2 * transfer interval (note 2) |  |  |  |  |  |  |  |
| Cooperative carrying – elastic work pieces |  | 99,9999 to 99,999999 |  | ~ 10 years |  | < 0.5 * transfer interval |  | 2.5 M |  | UE-UE (ProSe communication) |  | 250 500 with localisation information |  | > 5 |  | 0 |  | ≤12 |  | 2-8 |  | 10 x 10 x 5; 50 x 5 x 5 (note 3) |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | >2.5 |  | transfer interval |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | >1.7 (note 2) |  | 2 * transfer interval (note 2) |  |  |  |  |  |  |  |
| NOTE 1: The mobile operation panel is connected wirelessly to the 5G system. If the mobile robot/production line is also connected wirelessly to the 5G system, the communication includes two wireless links. NOTE 2: The first value is the application requirement, the other values are the requirement with multiple transmission of the same information (two or tree times respectively). NOTE 3: Service Area for direct communication between UEs. The group of UEs with direct communication might move throughout the whole factory site (up to several km²) |  |  |  |  |  |  |  |  |  |  |  |

The use case scenario "cooperative carrying – elastic work pieces" allows a larger tolerance in the precision of the movements of the collaborating mobile robots /AGV compared to the use case scenario "cooperative carrying – fragile work pieces". The larger tolerance in the coordinated movements allows for either faster movement of the work piece or longer transfer intervals (tradeoff between UE speed and transfer interval). A longer transfer interval relaxes the requirement on the maximum end-to-end latency.

The values given in Table 6.4-1 for use case "cooperative carrying" are the upper boundary of the required KPIs. There is a broad variety in the actual use cases of cooperative carrying and several parameters and deployment options have some flexibility in their value range or implementation. Relaxed KPI requirements can already satisfy a substantial set of cooperative carrying use cases depending on the actual physical characteristics (e.g. carrying speed, type of carried work piece).

The actual physical use case setting, such as the kind of workpiece to be carried, determines the necessary requirements on the communication service and the possible tradeoffs. Also the employed control algorithms and recovery mechanisms may further relax the necessary requirements and KPIs. Furthermore, the careful design of the mobile robots / AGVs (e.g. placement of UEs) can further improve the reliability of the wireless communication. Tradeoffs are especially possible between the carrying speed and the transfer interval / end-to-end latency as well as the communication service reliability. A reduction of the carrying speed reduces the requirements on the transfer interval / end-to-end latency or on the packet error ratio. The latter is possible, for instance, if the increased time budget is used for more transmissions.

Table 6.4-2: Periodic deterministic communication

| Use case # | Characteristic parameter |  |  | Influence quantity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Communication service availability: target value in % | Communication service reliability: mean time between failures | End-to-end latency: maximum | Data rate [Mbit/s] | Transfer interval | Survival time | UE speed | # of UEs | Service area (note 1) |
| Control-to-Control: 100 Mbit/s link replacement | 99,9999 to 99,999999 | ~ 10 years | < transfer interval value | 50 | ≤ 1 ms | 3 * transfer interval | stationary | 2 to 5 | 100 m x 30 m x 10 m |
| Control-to-Control: 1 Gbit/s link replacement | 99,9999 to 99,999999 | ~ 10 years | < transfer interval value | 250 | ≤ 1 ms | 3 * transfer interval | stationary | 2 to 5 | 100 m x 30 m x 10 m |
| NOTE 1: Length x width x height. NOTE 2: Transfer interval for scheduled aperiodic traffic |  |  |  |  |  |  |  |  |  |

Table 6.4-3: Aperiodic deterministic communication service performance requirements

| Use Case | Characteristic parameter (KPI) |  |  |  |  | Influence quantity |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Communication service availability: target value in % | Communication service reliability: Mean Time Between Failure | End-to-end latency: maximum | Bit rate [bits/s] | Direction | Message size [byte] | Survival time | UE speed | # of UEs connection | Service Area [m², m³] |
| Mobile Operation Panel: Emergency stop (emergency stop events) | 99,999999 | 1 day | <8 ms | 250 k | Uplink Downlink (note) | 40-250 | 16 ms | quasi-static; up to 10 km/h | 2 or more | 30 x 30 |
| NOTE: The mobile operation panel is connected wirelessly to the 5G system. If the mobile robot/production line is also connected wirelessly to the 5G system, the communication includes two wireless links. |  |  |  |  |  |  |  |  |  |  |

Table 6.4-4: Aperiodic deterministic communication

| Use case # | Characteristic parameter |  |  | Influence quantity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Communication service availability: target value in % | Communication service reliability: mean time between failures | End-to-end latency: maximum | Data rate [Mbit/s] | Transfer interval | Survival time | UE speed | # of UEs | Service area (note 1) |
| Control-to-Control: 100 Mbit/s link replacement | 99,9999 to 99,999999 | ~ 10 years | < transfer interval value | 25 | ≤ 1 ms (note 2) | n/a | stationary | 2 to 5 | 100 m x 30 m x 10 m |
| Control-to-Control: 1 Gbit/s link replacement | 99,9999 to 99,999999 | ~ 10 years | < transfer interval value | 500 | ≤ 1 ms (note 2) | n/a | stationary | 2 to 5 | 100 m x 30 m x 10 m |
| NOTE 1: Length x width x height. NOTE 2: Transfer interval for scheduled aperiodic traffic |  |  |  |  |  |  |  |  |  |

Table 6.4-5: Mixed traffic communication service performance requirements

| Use Case | Characteristic parameter (KPI) |  |  |  |  | Influence quantity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Communication service availability: target value in % | Communication service reliability: Mean Time Between Failure | End-to-end latency: maximum | Bit rate [bits/s] | Direction |  | Message Size [byte] | Survival time | UE speed | # of UEs connection | Service Area [m², m³] |  |
| Mobile Operation Panel: Manufacturing data stream | 99,9999 to 99,99999 | 1 day |  | 12 M | Uplink Downlink (note) |  | 250-1500 | n/a | quasi-static; up to 10 km/h | 2 or more | 30 x 30 |  |
| NOTE: The mobile operation panel is connected wirelessly to the 5G system. If the mobile robot/production line is also connected wirelessly to the 5G system, the communication includes two wireless links. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.4-6: Industrial wireless sensors service performance requirements

|  | Characteristic parameter |  |  |  |  |  |  | Influence quantity |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use case | Communication service availability: target value in % | Communication service reliability: mean time between failure | End-to-end latency | Transfer interval | Bit rate [bits/s] | Battery lifetime [year] (note 1) | Direction | Message Size [byte] | Survival time | UE speed | UE density [UE / m2] | Range [m] (note 2) |
| 1 | 99,99 | ≥ 1 week | < 100 ms | 100 ms – 60 s | ≤ 1 M | ≥5 | Mobile originated | 20 (note 3) | 3 x transfer interval | Stationary | Up to 1 | <500 |
| 2 | 99,99 | ≥ 1 week | < 100 ms | ≤ 1 s | ≤ 200 k | ≥5 | Mobile originated | 25 k | 3 x transfer interval | Stationary | Up to 0,05 | <500 |
| 3 | 99,99 | ≥ 1 week | < 100 ms | ≤ 1 s | ≤ 2 M | ≥5 | Mobile originated | 250 k | 3 x transfer interval | Stationary | Up to 0,05 | <500 |
| NOTE 1: Industrial sensors can use a wide variety of batteries depending on the use case,but in general they are highly constrained in terms of battery size. NOTE 2: Distance between the gNB and the UE. NOTE 3: The application-level messages in this use case are typically transferred over Ethernet. For small messages, the minimum Ethernet frame size of 64 bytes applies and dictates the minimum size of the PDU sent over the air interface. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.4-7: Relation between merged and new potential service requirements from use cases

| Merged Potential Service Requirement (clause 6) | Based on New Potential Service Requirements (clause 5) | Merged Potential Service Requirement (clause 6) | Based on New Potential Service Requirements (clause 5) |
| --- | --- | --- | --- |
| [MPR-22832-6.4-00a] | [PR-5.8.6-004] |  |  |
| [MPR-22832-6.4-00b] Table 6.4-1 | [PR-5.8.6-001] Table 5.8.6-1 [PR-5.11.6-001] Table 5.11.6-1 | [MPR-22832-6.4-00b] Table 6.4-2 | Table 5.16.6-1 |
| [MPR-22832-6.4-00c] Table 6.4-3 | [PR-5.8.6-001] Table 5.8.6-1 | [MPR-22832-6.4-00c] Table 6.4-4 | Table 5.16.6-1 |
| [MPR-22832-6.4-00d] Table 6.4-5 | [PR-5.8.6-001] Table 5.8.6-1 | [MPR-22832-6.4-00e] Table 6.4-6 | [PR-5.18.6-001] Table 5.18.6-1 |

End of Change 5

Start of Change 6

## C.3	Use Case 2: Controller-to-Device (C2D) communication

A machine module has typically a control unit and couple of field devices (I/O boxes). Field devices can have inputs for Sensors S and/or outputs for actuators A as process data. Typically, the machine controller (PLC or motion controller) has 1:n bidirectional communication relationships to (a set of) field device(s), e.g. sensors, actuators, drives. Smart devices include a control loop that is controlled by the control unit by set points with feedback values from the devices, e.g. a drive.

A typical configuration of a machine module has 10 to 20 field devices connecting a few hundreds of sensors and actuators. Larger entities can have 100 and more field devices within a service area of 50m x 50m x 10m.

A process automation facility may have a larger number of sensors and actuators distributed over a large area (up to several 10,000 in total) for closed-loop process control and process monitoring. In a typical DCS configuration, each distributed controller has a control over up to hundreds of sensors and actuators via 10 to 20 I/O boxes.

Within a machine network, I/O devices may need to exchange data with each other. A use case could be the connection to a (rotating) part of a machine that has so far been connected via a slip ring. A short delay is required, as the network sub-parts are logically be handled as one network of field devices. This communication can be handled as a sub-network connected to the machine controller (C2D).

End of Change 6

Start of Change 7

## D.1	Description

Communication service availability is defined in TS22.104 as "percentage value of the amount of time the end-to-end communication service is delivered according to an agreed QoS, divided by the amount of time the system is expected to deliver the end-to-end service according to the specification in a specific area". As explained further in TS22.104, this parameter indicates if the communication system works as contracted ("available"/"unavailable" state). The communication system is in the "available" state as long as the availability criteria for transmitted messages are met. The service is unavailable if the packets received at the target are impaired and/or untimely (e.g. update time > stipulated maximum).

A failure occurs when the survival time has elapsed. In a failure situation, the application has to be stopped and restarted again after the communication service has recovered. The application may stop and start automatically by itself, and it is not necessarily this has to done from outside. During such a failure situation, the communication system is in the "unavailable" state. For every failure situation the complete application has to be recovered until it is up-running again. Depending on the application this recovery time can last up to several minutes, for example a robot has to be moved to a “safe” re-starting point.

An example is illustrated in Figure D.1-1. A single failure of the communication service occurred in Figure D.1-1, which resulted from the downtime of the communication service in the network exceeding the survival time. The duration of the down state of the application depends on the application recovery time.

Figure D.1-1: Communication service status vs. application layer experience

In Figure D.1-1, the communication service and the application are in the up state (0). A failure occurs in the communication service in the network (packet loss) and from the network viewpoint theswitches to the down state (1). After the survival time has elapsed, the communication service (from the application view) also turns into the down state, as does the application (2). The failure is solved and the communication service changes to the up state (3). After the recovery time, the application is restored and switches to the up state (4).

The time needed for recovering has to be counted for each failure situation, which contributes to the “unavailability” of production application. The availability of the overall production asset varies depending on the frequency of downtime occurrences of the communication service and the recovery time required by the application. For simplicity reason we consider a communication service with 99,9999% availability, which would allow only 30 seconds downtime in a year (365 days). It is also assumed that the survival time is 10 s and that it takes 480 seconds to get the application recovered, i.e. recovery time = 480 s.

Table D.1-1 Calculation of Availability

| Survival time: 10 s Application recovery time: 480 s | Case 1 1 x 30 s downtime per year | Case 2 2 x 15 s downtime per year |
| --- | --- | --- |
| Communication service down time (application view) Communication service down time (network view) Application down time | 30 s 30 s + 10 s = 40 s 30 s + 480 s = 510 s | 2 x 15 s = 30 s 2 x (15 s + 10 s) = 50 s 2 x (15 s + 480 s) = 990 s |
| Communication service availability (application view) Communication service availability (network view) Application availability | 99,99990% 99,99987% 99,99838% | 99,99990% 99,99984% 99,99686% |

Communication service availability can also be estimated from the mean time between failures (MTBF) and the mean time to repair (MTTR) of the communication service:

communication service availability ≈ MTBF / (MTBF + MTTR)

In this context, MTBF excludes downtime, as illustrated in Figure D.1-3,  while MTTR refers to the mean time until the communication service is available after a failure, i.e., until the next valid packet has been received.

Figure D.1-3: MTBF and MTTR

Note that the time for an application to recover from a failure (recovery time) due to e.g. a robot arm having to be moved to a safe start position, is totally dependent on the application, which the communication service can hardly influence. In general, the application with a long recovery time may require very stringent communication service reliability as any occurrence of communication failure greatly impacts the application down-time and hence the overall production system availability. On the other hand, if the application recovers very fast, it can likely tolerate more frequent failures of the communication system, while achieving the same overall production system availability target. In such case the communication service reliability could be more relaxed (compared to the above case).

End of Change 7

Start of Change 8

Annex E:
Requirements mapping use case to merged

The potential new requirements of the studied use cases in clause 5 have been consolidated into the merged potential service requirements in clause 6. Table E-1 provides this mapping from the potential new requirements of the use cases to the corresponding merged and consolidated potential service requirements for all use cases. This documented mapping supports the assessment of the consolidation process (e.g. whether all potential new requirements of the use cases have been consolidated), provides transparency to the consolidation process, and provides the first part of the necessary linkage for tracing to follow up the implementation of the use case specific 5G service requirements in the 3GPP specification process.

Table E-1: Mapping of potential new requirements of use cases to merged potential service requirements

| Potential new requirement of use case (clause 5) | Corresponding merged potential service requirement (clause 6) | Potential new requirement of use case (clause 5) | Corresponding merged potential service requirement (clause 6) |
| --- | --- | --- | --- |
| [PR-5.2.6-001] | [MPR-22832-6.1-00a] | [PR-5.3.6-001] | [MPR-22832-6.1-00d] |
| [PR-5.2.6-002] | [MPR-22832-6.1-00c] | [PR-5.3.6-002] | [MPR-22832-6.1-00d] |
| [PR-5.2.6-003] | [MPR-22832-6.1-00b] |  |  |
| [PR-5.4.6-001] | [MPR-22832-6.1-00e] | [PR-5.5.6-001] | [MPR-22832-6.3-00a]; [MPR-22832-6.3-00b] |
| [PR-5.4.6-002] | [MPR-22832-6.1-00e] |  |  |
| [PR-5.6.6-001] | [MPR-22832-6.3-00c] | [PR-5.7.6-001] | [MPR-22832-6.3-00d] |
| [PR-5.8.6-001] | [MPR-22832-6.4-00b]; [MPR-22832-6.4-00c]; [MPR-22832-6.4-00d] - Mobile Operation Panel | [PR-5.9.6-001] | Table 6.5.1 |
| [PR-5.8.6-004] | [MPR-22832-6.4-00a] | [PR-5.9.6-002] | Table 6.5.1 |
|  |  | [PR-5.9.6-003] | Table 6.5.1 |
|  |  | [PR-5.9.6-004] | Table 6.5.1 |
|  |  | [PR-5.9.6-005] | Table 6.5-1 |
|  |  | [PR-5.11.6-001] | [MPR-22832-6.4-00b] Cooperative carrying; [MPR-22832-6.6-00a]; [MPR-22832-6.6-00e] |
|  |  | [PR-5.11.6-002] | [MPR-22832-6.6-00c] |
|  |  | [PR-5.11.6-003] | [MPR-22832-6.6-00h] |
|  |  | [PR-5.11.6-004] | [MPR-22832-6.6-00e] |
|  |  | [PR-5.11.6-005] | [MPR-22832-6.6-00d] |
|  |  | [PR-5.11.6-006] | [MPR-22832-6.6-00f] |
|  |  | [PR-5.11.6-007] | [MPR-22832-6.6-00i] |
|  |  | [PR-5.11.6-009] | [MPR-22832-6.6-00j] |
|  |  | [PR-5.11.6-010] | [MPR-22832-6.6-00g] |
| [PR-5.12.6-001] | [MPR-22832-6.5-00a] | [PR-5.13.6-001] | [MPR-22832-6.6-00k] |
|  |  | [PR-5.15.6-001] | [MPR-22832-6.1-00f] |
|  |  | [PR-5.15.6-002] | [MPR-22832-6.1-00g] |
|  |  | [PR-5.15.6-003] | [MPR-22832-6.1-00h] |
| Table 5.16.6-1 | Table 6.4-2 and Table 6.4-4 | [PR-5.17.6-001] | [MPR-22832-6.3-00e] |
| [PR-5.18.6-001] | [MPR-22832-6.4-00e] |  |  |

End of Change 8

Start of Change 9

Annex F: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-02 | SA1#85 |  |  |  |  | TR Skeleton for Study on further service requirements for cyber-physical control applications in vertical domains; | 0.0.0 |
| 2019-05 | SA1#86 | S1-191335 |  |  |  | FS_eCAV - Scope for 22.832 | 0.1.0 |
| 2019-05 | SA1#86 | S1-191299 |  |  |  | Open issues on network performance requirements | 0.1.0 |
| 2019-05 | SA1#86 | S1-191338 |  |  |  | Use case for ProSe Device to Device communication for cyber-physical applications | 0.1.0 |
| 2019-05 | SA1#86 | S1-191302 |  |  |  | Use case for providing out of network coverage positioning service for cyber-physical applications | 0.1.0 |
| 2019-05 | SA1#86 | S1-191339 |  |  |  | Use case for communication monitoring for CAV applications | 0.1.0 |
| 2019-05 | SA1#86 | S1-191579 |  |  |  | Use case for network energy efficiency with application assistance | 0.1.0 |
| 2019-05 | SA1#86 | S1-191341 |  |  |  | FS_eCAV - TR 22.832 - Multiple Time Domains in gNB | 0.1.0 |
| 2019-05 | SA1#86 | S1-191342 |  |  |  | FS_eCAV - TR 22.832 - Merging of working clock domains | 0.1.0 |
| 2019-05 | SA1#86 | S1-191307 |  |  |  | FS_eCAV - TR 22.832 - Mapping of sync devices in integrated 5G-TSN networks | 0.1.0 |
| 2019-05 | SA1#86 | S1-191580 |  |  |  | FS_eCAV - TR 22.832 - D2D/ProSe communication for cooperative carrying | 0.1.0 |
| 2019-05 | SA1#86 | S1-191641 |  |  |  | FS_eCAV - TR 22.832 - Mobile Operation Panel for Production Lines | 0.1.0 |
| 2019-05 | SA1#86 | S1-191310 |  |  |  | FS_eCAV - TR 22.832 - Communication Service Interface | 0.1.0 |
| 2019-05 | SA1#86 | S1-191311 |  |  |  | Use cases with an need for vertical positioning | 0.1.0 |
| 2019-05 | SA1#86 | S1-191189 |  |  |  | FS_eCAV - TR 22.832 - Structure for consolidated potential requirements | 0.1.0 |
| 2019-05 | SA1#86 | S1-191259 |  |  |  | Incorporated all agreed contributions to TR 22.832 at SA1#86 | 0.1.0 |
| 2019-05 | SA#84 | SP-190319 |  |  |  | Presentation to SA for information | 1.0.0 |
| 2019-08 | SA1#87 | S1-192415 |  |  |  | Overview section | 1.1.0 |
| 2019-08 | SA1#87 | S1-192752 |  |  |  | Resolution of questions in clause 4.1 | 1.1.0 |
| 2019-08 | SA1#87 | S1-192417 |  |  |  | Survival time vs. consecutive message loss | 1.1.0 |
| 2019-08 | SA1#87 | S1-192418 |  |  |  | Introduction subclause for use case chapter | 1.1.0 |
| 2019-08 | SA1#87 | S1-192428 |  |  |  | Update use case “Integration of 5G networks with TSN networks (time synchronization)” | 1.1.0 |
| 2019-08 | SA1#87 | S1-192757 |  |  |  | Update use case “Cooperative carrying of work pieces” | 1.1.0 |
| 2019-08 | SA1#87 | S1-192846 |  |  |  | Update Use Case Mobile Operation Panel | 1.1.0 |
| 2019-08 | SA1#87 | S1-192459 |  |  |  | Clarification for Communication Monitoring for CAV Applications and pCR | 1.1.0 |
| 2019-08 | SA1#87 | S1-192463 |  |  |  | Use of sidelink for process monitoring and plant asset management | 1.1.0 |
| 2019-08 | SA1#87 | S1-192849 |  |  |  | Non-public networks as private slices | 1.1.0 |
| 2019-08 | SA1#87 | S1-192437 |  |  |  | Use Case Flexible Manufacturing | 1.1.0 |
| 2019-08 | SA1#87 | S1-192462 |  |  |  | Wired to wireless link replacement for Industry 4.0 | 1.1.0 |
| 2019-08 | SA1#87 | S1-192758 |  |  |  | TSN Reliability enhancement | 1.1.0 |
| 2019-08 | SA1#87 | S1-192438 |  |  |  | Industrial Wireless Sensors for process and asset monitoring | 1.1.0 |
| 2019-08 | SA1#87 | S1-192848 |  |  |  | Multi-connectivity for robotic automation | 1.1.0 |
| 2019-08 | SA1#87 | S1-192852 |  |  |  | Merged potential service requirements – Industrial Ethernet integration | 1.1.0 |
| 2019-08 | SA1#87 | S1-192442 |  |  |  | Merged potential service requirements –Network operation and maintenance | 1.1.0 |
| 2019-08 | SA1#87 | S1-192853 |  |  |  | Merged potential service requirements – Network performance | 1.1.0 |
| 2019-08 | SA1#87 | S1-192464 |  |  |  | Merged potential service requirements – Positioning | 1.1.0 |
| 2019-08 | SA1#87 | S1-192445 |  |  |  | Considerations on absolute and relative positioning (Annex) | 1.1.0 |
| 2019-08 | SA1#87 | S1-192854 |  |  |  | Merged potential service requirements – ProSe Communication for CAV | 1.1.0 |
| 2019-08 | SA1#87 | S1-192447 |  |  |  | Consolidation of sidelink requirements | 1.1.0 |
| 2019-08 | SA1#87 | S1-192448 |  |  |  | Conclusion section | 1.1.0 |
| 2019-08 | SA1#87 | S1-192449 |  |  |  | Industrial communication network architecture (Annex) | 1.1.0 |
| 2019-08 | SA1#87 | S1-192450 |  |  |  | Communication service availability vs. failure (Annex) | 1.1.0 |
| 2019-08 | SA1#87 | S1-192367 |  |  |  | Incorporated all agreed contributions at SA#87 to TR 22.832 | 1.1.0 |
| 2019-09 | SA#85 | SP-190834 |  |  |  | Presentation for approval by the editor, since it could not be agreed at WG level due to lack of time | 1.1.1 |
| 2019-09 | SA#85 | SP-190834 |  |  |  | Approved by SA#85 | 17.0.0 |
| 2019-12 | SA#86 | SP-191027 | 0014 | 1 | F | FS_eCAV update to requirements consolidation for positioning | 17.1.0 |
| 2019-12 | SA#86 | SP-191027 | 0009 | 1 | F | Addition of definition and abbreviation for non-public network | 17.1.0 |
| 2019-12 | SA#86 | SP-191027 | 0010 | 1 | F | FS_eCAV update to requirements consolidation for industrial ethernet integration | 17.1.0 |
| 2019-12 | SA#86 | SP-191027 | 0012 | 1 | F | FS_eCAV update to requirements consolidation for network operation and maintenance | 17.1.0 |
| 2019-12 | SA#86 | SP-191027 | 0013 | 1 | F | FS_eCAV - Update Clause Merged Potential Service Requirements - Network Performance | 17.1.0 |
| 2019-12 | SA#86 | SP-191027 | 0015 | 1 | F | FS_eCAV update to requirements consolidation for device-to-device/ProSe communication | 17.1.0 |
| 2019-12 | SA#86 | SP-191027 | 0017 | 1 | D | Editorial and minor corrections to TR 22.832 | 17.1.0 |
| 2019-12 | SA#86 | SP-191027 | 0002 | 4 | F | Clarification on MTBF | 17.1.0 |
| 2019-12 | SA#86 | SP-191027 | 0005 | 3 | F | Update Use Case Mobile Operation Panel EN | 17.1.0 |
| 2019-12 | SA#86 | SP-191027 | 0004 | 3 | F | Clarification on KPIs | 17.1.0 |
| 2019-12 | SA#86 | SP-191027 | 0019 | 3 | F | Mapping or requirements from use case to consolidated | 17.1.0 |
| 2019-12 | SA#86 | SP-191027 | 0018 | 2 | F | Introduction clause for TR 22.832 | 17.1.0 |
| 2019-12 | SA#86 | SP-191027 | 0020 | 3 | F | Correction of clause 4.1 and clause 5.1 | 17.1.0 |
| 2019-12 | SA#86 | SP-191027 | 0016 | 3 | B | Resolution of remaining Editor's Notes | 17.1.0 |
| 2019-12 | SA#86 | SP-191027 | 0021 | 4 | C | Clock synchronization budget use case | 17.1.0 |
| 2020-07 | SA#88e | SP-200566 | 0023 |  | F | Delete the editor's note in 5.14.6 | 17.2.0 |
| 2020-07 | SA#88e | SP-200566 | 0027 |  | C | Miscellaneous values for further study | 17.2.0 |
| 2020-07 | SA#88e | SP-200566 | 0028 |  | D | 22.832 Miscellaneous editorial corrections | 17.2.0 |
| 2020-07 | SA#88e | SP-200566 | 0022 | 1 | D | Alignment and Correction to Change History | 17.2.0 |
| 2020-07 | SA#88e | SP-200566 | 0026 | 3 | D | Positioning value for further study and editorial changes | 17.2.0 |
| 2020-07 | SA#88e | SP-200566 | 0025 | 2 | F | Time synchronization budget for the 5G system | 17.2.0 |

End of Change 9

END OF PROPOSED CHANGES