3GPP TSG-SA WG1 Meeting #91e 	S1-203188

Electronic Meeting, 24 August - 2 September 2020	(revision of S1-20xxxx)

Title:	FS_AMMT – Compressed Federated Learning

Agenda Item:	7.4.1 - FS_AMMT

Source:	LG Electronics

Contact:	Sungjin (James) Kim <sj88.kim@lge.com>

Abstract: This contribution proposes to add the container use case in TR 22.874 version 0.0.0 and to identify the potential new requirements of this use case related to Compressed Federated Learning.

****First CHANGE****

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[X]	Song Han, Jeff Pool, John Tran, William J. Dally, "Learning both Weights and Connections for Efficient Neural Networks" May 2015.

****Second CHANGE (ALL NEW TEXT) ****

7.X	Compressed Federated Learning for image/video processing

7.X.1	Description

Federated learning can be used to train AI/ML models based on number of images and videos generated by cameras in mobiles devices by iteratively exchanging gradient of updating models instead of direct user images and videos. Because this method can utilize images and videos from many users, the performance of a trained AI/ML model can be significantly higher than a stand-alone case. However, the basic federated learning methods can have disadvantages by massive uplink traffics and high computational cost at a mobile device. Therefore, we propose to consider a compressed federated learning method, which allows to transfer compressed (not full) models during a learning period.

Figure 5.x.1.1-1 shows the essential procedure of compressed federated learning. Compressed federated learning (CFL) performs a set of the three operation stages iteratively. The three operation stages include the training UE selection, the sparse weight distribution, and the training result reporting stages, while the operations of the first iteration and the last iteration are different from operations in the other iterations.

Each iteration in CFL starts with the training UE selection stage, at which the CFL server selects a set of available users from the candidate users to associate with the same purpose AI/ML model. To the selected users, ready to participate in the learning process because of being an available state, the CFL server transmits the train configuration information. At the next stage, the CFL server sends the sparse global model, which could be an initial version of the AI/ML model in the first iteration. Otherwise, the sparse global model is an aggregated version based on user reporting information.

Then, each UE trains a received model after expanding the spatial model and reports a training result to the CFL server, where the training result is comprised only of significant value weight gradients for applying a model compression. By doing so, uplink throughout requirement can be significantly reduced in comparison with the federated learning method. In the last iteration, I-th iteration, the CFL Server sends ‘train stop message’ to UEs so that the UEs can not send its update any longer, and the CFL Server performs fine-tuning by pruning unnecessary nodes. Throughout these multiple iterations, the AI/ML model will be progressively enhanced based on user data in mobile networks at reduced uplink and downlink throughput.

Figure 5.x.1.1-1. Compressed Federated Learning timeline for image recognition

7.X.2	Pre-conditions

UE can have a computational hardware and algorithm capability to train an AI/ML model such as for an image and video cognition.

UE can send intermediate training results to a CFL server.

A CFL server can select training devices and determine training configuration.

A CFL server can aggregate intermediate training results and generate a sparse global model for the next learning iteration.

A CFL server can distributed a global AI/ML mode to a set of selected users.

7.X.3	Service Flows

Step 1: The CFL server selects the training users from candidate users.

Step 2: The CFL server sends the configuration information to the selected users.

Step 3: The CFL server distributes the initial (or, aggregated) sparse global model to the selected users through a 5G networks.

Step 4: Each UE expands the sparse global model and train the expanded model using its local data. Then, each UE sends only significant value weight gradients to the CFL server.

Step 5: The CFL server aggregates the training results received from the training UEs and update a global model using the aggregated results.

Step 6: Until the AI/ML model reaches saturated performance enhancement, the process runs repeatedly from step 1.

Step 7: Otherwise, the CFL server performs fine-tuning for a global model compression for a global model. This process can applied regularly so as to improve bandwidth and computation resource efficiency before the training finalization.

Finally, the CFL server distributes the new sparse global model to all users which needs the same AI/ML model.

7.X.4	Post-conditions

For a UE prospective, CFL can reduce uplink and downlink throughput requirements for the federated learning process. Also, the computational complexity in UEs can be significantly reduced because of enabling a compressed model.

7.X.5	Gap analysis

Latency analysis for gradient uploading and the global model downloading for image recognition

AI/ML model training data for CFL is a new type of traffic. Consider CFL to train a 8-bit CNN VGG16 model with 224x224x3 images. Table X.1.5-1 shows that the single GPU computation time should be larger than the addition of gradient uploading latency and global model downloading latency.

Table 5.X.1.5-1: GPU computation time for different mini-batch sizes 
for Compressed Federated Learning

| Mini-batch size (images) | GPU computation time (ms) | Required latency for trained gradient uploading (ms) | Required latency for global model distribution (ms) |
| --- | --- | --- | --- |
| 64 | 325 | <162ms | <162ms |
| 32 | 191 | <95ms | <95ms |
| 16 | 131 | <65ms | <65ms |
| 8 | 111 | <55ms | <55ms |
| 4 | 105 | <52ms | <52ms |

Data rate analysis for gradient uploading and the global model downloading for image recognition

Table 5.X.1.5-2 shows the required data rate for gradient uploading and global model downloading for the above 8-bit VGG16 model when CFL is applied. We calculated the required data rate based on Table 1 in [X], in which the pruning size of the 8-bit VGG16 model can be reduced 13 times from the original size of 138 Mbyte. It is noteworthy that 13 times model compression gives almost no accuracy degradation for the 8-bit VGG16 model. If we assume that the minibatch size is 4, the uplink required rate is compressed trained parameter size * 8 / (GPU computation time / 2) = (138 / 13) Mbyte * 8 bits / (105ms / 2) = 1.56Gbps which is same to the downlink require rate. For 7 UE cases, the total uplink required data can be 1.56Gbps * 7 = 11Gbps, which is higher than the NR UL peak data rate (10Gbps). However, we remind that it could be 143Gbps for federated learning. Therefore, a model compression approach, such as CFL should be considered for the federated learning process.

Table 5.X.1.5-2: Required data rate for gradient uploading and global model downloading
Required data rate = compressed trained parameter size * 8 / (GPU computation time / 2)

| Mini-batch size (images) | Required UL data rate for trained gradient uploading (Gbps) | Required DL data rate for global model distribution (Gbps) |
| --- | --- | --- |
| 64 | 0.50 | 0.50 |
| 32 | 0.86 | 0.86 |
| 16 | 1.25 | 1.25 |
| 8 | 1.48 | 1.48 |
| 4 | 1.56 | 1.56 |

Latency analysis for training device selection and training configuration

The CFL Server should notice training device and configuration information in [1]ms.

7.X.6	Potential New Requirements needed to support the use case

[P.R.7.x.6-001] The 5G system shall support the UE to report its trained gradient for each iteration of Compressed Federated Learning within [52~162]ms.

[P.R.7.X.6-002] The 5G system shall support the CFL server to distribute the global model for each iteration of Compressed Federated Learning within [52~162]ms.

[P.R.7.x.6-003] The 5G system shall support UL unicast or multicast transmission with [0.5]-[1.56]Gbps data rates and the communication service availability not lower than [99.999%] for reporting the trained gradients for Compressed Federated Learning.

[P.R.7.x.6-004] The 5G system shall support DL multicast transmission with [0.5]-[1.56]Gbps data rates and the communication service availability not lower than [99.999%] for distributing the global model for Compressed Federated Learning.