3GPP TSG-SA1 Meeting #92e	S1-204137

Electronic Meeting, 10 - 19 November 2020	(revision of S1-20xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.263 | CR | 0011 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support deterministic transport services in VIAPA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | AVPROD |  |  |  |  |  | Date: |  |  | 2020-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the performance requiremens part of TS 22.263, it mentions low-latency periodic deterministic audio transport service. However, there is no requirements for determinstice service in the service requirement part. In addition, the definition of deterministic service is not defined. This may results in confusion for downstream groups. This CR proposes to add requirements for deterministic transport service and define what deterministic service represents. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding requirements for deterministic transport service, and clarify the definition of deterministic. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The definition of the term deterministic in performance requirement is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the Change---

5	Service requirements

5.1	Non-public network requirements

The 5G system enables an NPN for video, imaging and audio for professional applications. The related requirements are described in 3GPP TS 22.261 [4]:

-	Generic NPN requirements can be found in clause 6.25.

-	Requirements on the subscription aspects can be found in clause 6.14.

-	Authentication requirements can be found in clause 8.3.

5.2	Application specific requirements

The 5G system shall support media flows from open standard based broadcast workflows and be agnostic to the data carried.

5.3	Clock synchronization

The 5G network shall be able to provide a time reference information to a 3rd party application acting as a master clock with an accuracy of 1 microsecond.

5.4	Network exposure requirements

The 5G system shall support mechanisms to allow 3rd party application to update information associated to UE configuration (e.g. media compression, resolution, frame rate) for a UE or group of UEs using the application.

5.5	Service continuity

The 5G system shall be able securely reconnect within 20 ms from UE starting first network connection attempt after a UE network connection loss.

The 5G system shall support uplink and downlink service continuity maintaining acceptable performance requirements while switching between co-located PLMN and NPN (e.g., due to mobility).

The 5G system shall support service continuity maintaining acceptable performance requirements: for an uplink stream while performing traffic steering, switching, and splitting among co-located PLMN(s) and NPN(s); for downlink while switching between co-located PLMN and NPN.

5.6	Multi-network connectivity and service delivery

The 3GPP system shall be able to enable a UE to receive low-latency downlink multicast traffic from one network (e.g. NPN), and paging as well as data services from another network (e.g. PLMN) simultaneously.

NOTE: 	Depending on the capabilities and configurations of the UE, limitations of data-rate and latency may be acceptable.

5.7	Deterministic transport requirements

The 5G system shall support stable delay between transmission of a message at the transmitter side and receipt of the message at the at the receiver side to ensure deterministic transport services.

## ---End of the Change---