3GPP TSG-SA WG1 Meeting #92e 	S1-204202

Electronic Meeting, 16 – 25 November 2020	(revision of S1-204xxx)

Title:	Energy Substation Surveillance

Agenda Item:	7.9.1 (FS_5GSEI)

Document for:	Approval

Source:	Samsung

Contact:	Erik Guttman <erik.guttman@samsung.com>

Abstract: Local regulations and Utility operations require physical security measures for energy substations. These measures include surveillance information that can be evaluated at any time. To meet these requirements, significant communication is required between the substation and the Utility operations center.

Proposal

It is proposed to agree to the following changes to TR 22.867.

The use case concerns the use of surveillance cameras in sensitive locations and the implications on telecommunications where that serves as the means by which the surveillance media is collected in a remote location.

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	                       3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	SunWeiqing etc.  Generalized Energy Storage Control Strategies on User Side in Power Ancillary Service Market.  Automation of Electric Power System Vol.44 No.2 Jan.25, 2020

[3]	China Southern Power Grid, China Mobile, HuaWei: "White Paper: 5G enables smart grid applications".http://www.sgcio.com/technology/anli/2018/0716/86656.html

[4]  	GB/T 1936-2018 Guidelines of cyber security protection for electric power system supervision and control http://www.howtoabc.com/book8/14078-0.html, https://www.secrss.com/articles/24761

[5]	IEEE C37.2-2008 IEEE Standard Electrical Power System Device Function Numbers, Acronyms, and Contact Designations.

[6]	IEC TR 61850-90-1:2010, Communication Networks and Systems for Power Utility automation – Part 90-1: Use of IEC61850 for the communication between substations.

[7]	IEEE 1588-2019 – IEEE Standard for a precision clock synchronization protocol for networked measurement and control systems.

[8]	IEEE Guide for Application of Digital Line Current Differential Relays Using Digital Communication.

[9]	61850-9-3-2016 – IEC/IEEE International Standard - Communication Networks and Systems for Power Utility automation – Part 90-3: Precision time protocol profile for power utility

[10]                     3GPP TR 22.804: "Study on Communication for Automation in Vertical domains (CAV)".

[11]	Sendin, A., et. al., “Telecommunication Networks for the Smart Grid,” Artech House, 2016.

[12]	Goel, S., S. F. Bush, and D. Bakken, (eds.), IEEE Vision for Smart Grid Communications: 2030 and Beyond, New York: IEEE, 2013.

[13]	US Department of Energy, “Communications Requirements of Smart Grid Technologies, 2010”, accessed 14.08.20, http://energy.gov/sites/prod/files/gcprod/documents/Smart_Grid_Communications_Requirements_Report_10-05-2010.pdf

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

[y]	Budka, K, et. al., “Communication Networks for Smart Grids”, Springer Verlag, 2014.

Second Change

## 5.x	Energy Substation Surveillance

### 5.x.1	Description

The use case concerns the use of surveillance cameras in sensitive locations and the implications on telecommunications where that serves as the means by which the surveillance media data is collected in a remote location.

Closed-circuit video (CCTV) is the most bandwidth-intensvie of all utility applications. The data rates depend on the required resolution of images, number of colors and frame rate. There are existing standards that reduce this data rate considerably while still providing excellent video quality. During incidents (such as security breaches, equipment malfunctions and power outages,) a higher resolution can be used.

Many utility substations today use fiber optic cables to provide sufficient communications capacity to transport surveillance media data. This use case explores the use of 5G instead either as the sole means for transporting surveillance media data, or as a backup in case the primary means of transport (fiber) has been compromised. This could occur inadvertently if the fixed communication network suffers damage, or intentionally as part of a coordinated local attack that compromises substation security.

While it is often necessary and prudent to transport surveillance media data continuously, there are approaches that reduce this need, e.g. triggers based on motion detectors, tampering (e.g. with cameras, locks, etc.), heat sensors, etc. In addition, video analytics can identify suspicious changes in a field of view within the secure permiter, such as people present when they are unexpected, or people present at places within the secure perimeter for too long, etc. In this case, video could be stored locally and only transmitted on demand, or when triggered by an event. The design of a secure facility is beyond the scope of this use case. In any case, there are times in which transport of all surveillance data is needed – so the capacity model is based on this situation. This capacity may only be needed intermittently, for the reasons given above, or it may be required constantly.

Some regulations however, e.g. NERC CIP in North America, require continuous live video transport from cameras monitoring the substation’s Electronic Security Perimeter.

Further, given the use of fiber optic cables, the networks are vastly over-provisioned. For this reason, even where regulation does not require constant video media transport for perimeter security, there is a tendency to transport all surveillance media, at all times.

The most common video codecs used are MPEG-2, MPEG-4 and more recently, the Motion-Joint Photographic Expert Group (M-JPEG). The frame rate commonly used is 25-30 video frames per second. The resolutions often used, per the Common Interchange Format (CIF), are QCIF (176x120), CIF (352x240), 2CIF (704x240), and 4CIF (704x480). [y] This reference is 7 years out of date, though, and the current rate used by some energy utilities is now HDTV (1280x720) resolution.

For the purposes of this use case HDTV with continual media transport will be considered as the service requirement, as this is most realistic looking forward in time. 1280x720 * 8bit resolution * 30 frames per second = 2.4 (10)7 bits per second (uncompressed.) For H.264 compression (MPEG-4), this can be transported with 3 Mbps.

### 5.x.2	Pre-conditions

The substation has the capacity to generate surveillance data. The utility operator “U” needs for this data to be sent from the substation to a centeral operation center reliably, so they arrange services with a telecommunications service provider “T” for this purpose.

Surveillance cameras use mechanisms for mechanical panning, zooming and tilting (PZT) so they do not need to be more than high definition (HD).

### 5.x.3	Service Flows

There are two potential service flows.

1) Continuously transport surveillance media from the substation site to a central operation center.

2) When triggered due to local conditions or remote control, transport surveillance media from the substation site to a central operations center.

### 5.x.4	Post-conditions

The substation security has some assurance, as the substation surveillance media data that is generated on the site can be transported reliably to a central operations center.

### 5.x.5	Existing features partly or fully covering the use case functionality

The video rates described below can be supported by 5G. In fact, 4G suffices as a ‘backup’ mechanism to carry surveillance video data in some existing substations today.

| scenario | User Experienced Data Rate (Mbps per camera) | Latency (ms) | Reliability % | Connection density | Coverage (km) |
| --- | --- | --- | --- | --- | --- |
| Surveil-lance HD camera media | 3 (up 5 if a lot is going on) | [NOTE1] | [NOTE1] | ~12/sub-station | >=30km (city range) |
| NOTE1: Media transport typically uses a streaming protocol that allows for buffering of media in the event of changes of latency (jitter) or intermittent failure. This may result in transient interruptions of the video stream, but does not result in loss of media. For this reason, no KPI target is given for these metrics. |  |  |  |  |  |

Table 5.x.5-1: Transport of Surveillance HDTV KPI Requirements

### 5.x.6	Potential New Requirements needed to support the use case

This use case is included for completeness but no new requirements have been identified.

Second Change