3GPP TSG-SA WG1 Meeting #92e	S1-20xxxx

Online, 09 November – 18 November 2020

Title:	 FS_Ranging KPI analysis on Distance based Smart Home Device Control Use Case

Agenda Item:	7.10.1

Source:	Xiaomi

Contact:	Xiaowei Jiang (jiangxiaowei@xiaomi.com)

1. Background

In last SA1 meeting, the numbers in the KPI table of distance based smart home device control were marked in square bracket due to concerns on the value. In this contribution, we try to analyze and justify these values, and suggest to remove the square brackets.

2. Discussion

The KPIs for distance based smart home device control is defined in Table 5.1.6-1 in 22.855, as cited below:

Table 5.1.6-1 – KPIs for distance based smart home control

| Ranging scenario | Ranging Accuracy (95 % confidence level) |  | Availability | Latency | Effective Ranging distance, Coverage, NLOS/LOS, Relative UE velocity, Ranging interval, Number of concurrent ranging operation for a UE, Number of concurrent ranging operation in an area |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Distance Accuracy | Direction Accuracy |  |  | Effective Ranging distance | Coverage | NLOS/LOS | Relative UE velocity | Ranging interval | Number of concurrent ranging operation for a UE | Number of concurrent ranging operation in an area |
| Distance based smart device control | [10cm] | - | [99 %] | [50ms] | [20m] | IC/PC/OOC | LOS | Static/ Moving (<1m/s) | [50ms] | [20] | - |

Distance Accuracy:

In a typical smart home scenario, user’s room may be equipped with varies of smart devices. Take smart light as example, there are Table lamp, Ceiling lamp, night lamp, etc. The size of the room may be 3*4 meter or even smaller. Given the limited movement area in this kind of room, The distance between user and ceiling lamp might be quite similar to the distance between user and table lamp/night lamp in many cases as the user moves. 10cm distance accuracy will guarantee distance based smart device control works well in this case.

Availability:

99% availability will ensure a user friendly experience for smart home control.

Latency:

Smart speaker is the typical control device in smart home. For light control, typically around 1 second latency from user giving the voice instruction to the light being turned on/off  is satisfying to user. The speech recognition and semantics understanding is usually handled in the cloud and takes on average 500ms for processing and takes around 50ms for round trip transmission. The command for performing ranging, ranging result report, and command for controlling the light would take about 400ms if only local command is involved. Then, the budget for ranging would be around 50ms. One illustration of the voice control flow is shown in the figure below:

Besides, due to user movement, service latency should also ensure that the acquired ranging result is still valid. For 1m/s movement, 50ms latency would result in 5cm distance change. Then service latency below 50ms should be ensured to keep the validity of ranging result.

Ranging Interval:

Ranging interval should ensure that the measured result is still valid (with sufficient accuracy) during this period when user moves. For 1m/s user movement, 50ms ranging interval corresponds to 5cm distance change, which is acceptable for 10cm distance accuracy.

Number of concurrent ranging operation for a UE:

To reduce the response latency, it might be sometimes useful that ranging is performed in advance, e.g. before UE giving the voice instruction (e.g. when UE arouses the smart speaker) if power consumption is less a concern. In this case, UE may need to perform ranging with all the smart devices around the UE. Typically, there might be no more than 20 smart devices within LOS distance.

3. Proposal

Proposal：The square brackets in table 5.1.6-1 in 22.855 are removed.

**************** First Change ******************

Table 5.1.6-1 – KPIs for distance based smart home control

| Ranging scenario | Ranging Accuracy (95 % confidence level) |  | Availability | Latency | Effective Ranging distance, Coverage, NLOS/LOS, Relative UE velocity, Ranging interval, Number of concurrent ranging operation for a UE, Number of concurrent ranging operation in an area |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Distance Accuracy | Direction Accuracy |  |  | Effective Ranging distance | Coverage | NLOS/LOS | Relative UE velocity | Ranging interval | Number of concurrent ranging operation for a UE | Number of concurrent ranging operation in an area |
| Distance based smart device control | 10cm | - | 99 % | 50ms | 20m | IC/PC/OOC | LOS | Static/ Moving (<1m/s) | 50ms | 20 | - |

************* End of First Change ***************