3GPP TSG-SA WG1 Meeting #92e	S1-20xxxx

Online, 09 November – 18 November 2020

Title:	 FS_Ranging KPI analysis on Smart Home TV Control Use Case

Agenda Item:	7.10.1

Source:	Xiaomi

Contact:	Xiaowei Jiang (jiangxiaowei@xiaomi.com)

1. Background

In last SA1 meeting, the numbers in the KPI table of Smart Home TV Control were marked in square bracket due to concerns on the value. In this contribution, we try to analyze and justify these values, and suggest to remove the square brackets.

2. Discussion

The KPIs for Smart Home TV Control is defined in Table 5.2.6-1 in 22.855, as cited below:

Table 5.2.6-1 – KPIs for smart home TV control

| Ranging scenario | Ranging Accuracy (95 % confidence level) |  | Availability | Latency | Effective Ranging distance, Coverage, NLOS/LOS, Relative UE velocity, Ranging interval, Number of concurrent ranging operation for a UE, Number of concurrent ranging operation in an area |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Distance Accuracy | Direction Accuracy |  |  | Effective Ranging distance | Coverage | NLOS/LOS | Relative UE velocity | Ranging interval | Number of concurrent ranging operation for a UE | Number of concurrent ranging operation in an area |
| Distance based smart device control | [10cm] up to 2 meter separation | [±2°] horizontal direction accuracy at 0.1 to 2 meter separation and AoA coverage of (-60°) to (+60°); [±2°] Elevation direction accuracy at 0.1 to 2 meter separation and AoA coverage of (-45°) to (+45°) | [99 %] | [10ms] | [10m] | IC/PC/OOC | LOS | Static/ Moving (<[10m/s]) | [50ms] | - | - |

Distance Accuracy & Direction Accuracy:

Assuming a 50 inch TV(110cm * 62 cm), a typical 3 meter distance between user and TV, 10 cm error of the measured pointed position would be less sensible, and it corresponds to roughly 2 degree angular error (3 meter * tan(2)).

Availability:

99% availability will ensure a user friendly experience for smart home control.

Latency:

Service latency depends on the sampling rate of the remoter movement. As the cursor movement on the TV is quite similar to the cursor movement on the computer screen, we can assume the same sampling rate as the mouse. Typically, the mouse sampling rate is 125 Hz. Then the service latency of Ranging is 1/125 = 8ms.

Effective ranging distance:

Typically, the distance between TV and the watcher is no more than 10 meter.

Relative UE velocity:

Typically, the speed of user hand movement is no more than 10m/s.

Ranging Interval:

Ranging interval are mainly depends on the movement. For 10m/s movement, 10ms ranging interval corresponds to 10cm distance change.

3. Proposal

Proposal：The square brackets in table 5.2.6-1 in 22.855 are removed. The ranging interval is changed from 50ms to 10ms.

**************** First Change ******************

Table 5.2.6-1 – KPIs for smart home TV control

| Ranging scenario | Ranging Accuracy (95 % confidence level) |  | Availability | Latency | Effective Ranging distance, Coverage, NLOS/LOS, Relative UE velocity, Ranging interval, Number of concurrent ranging operation for a UE, Number of concurrent ranging operation in an area |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Distance Accuracy | Direction Accuracy |  |  | Effective Ranging distance | Coverage | NLOS/LOS | Relative UE velocity | Ranging interval | Number of concurrent ranging operation for a UE | Number of concurrent ranging operation in an area |
| Distance based smart home TV control | 10cm up to 2 meter separation | ±2° horizontal direction accuracy at 0.1 to 2 meter separation and AoA coverage of (-60°) to (+60°); ±2° Elevation direction accuracy at 0.1 to 2 meter separation and AoA coverage of (-45°) to (+45°) | 99 % | 10ms | 10m | IC/PC/OOC | LOS | Static/ Moving (<10m/s) | 10ms | - | - |

************* End of First Change ***************