3GPP TSG-SA WG1 Meeting #92e	S1-20xxxx

Online, 09 November – 18 November 2020

Title:	 FS_Ranging KPI analysis on Finding items in a Supermarket Use Case

Agenda Item:	7.10.1

Source:	Spreadtrum Communications

Contact:

1. Background

In last SA1 meeting, the numbers in the KPI of Finding items in a Supermarket were marked in square bracket due to concerns on the value. In this contribution, we try to analyze and justify these values, and suggest removing the square brackets.

2. Discussion

The KPIs for Finding items in a Supermarket is defined in 5.4.6 in 22.855, as cited below:

5G system shall be able to support Ranging service a maximum of [500UEs] simultaneously with service latency less than [100ms] in the service area with radius less than [100 m]

Editor’s Note: the radius value is up to the determination of Ranging distance.

The 5G system shall be able to support Ranging service with less than [1m] distance accuracy at the UE speed up to 3km/h.

The 5G system shall be able to support Ranging service with less than [20 degree] direction accuracy at the UE speed up to 3km/h.

The 5G system shall be able to support Ranging service with service availability in reach [>95%].

The 5G system shall be able to support Ranging service at a maximum distance of [100m].

The 5G system shall be able to support a maximum of [10UEs] at the same time.

Distance Accuracy:

We assume that,

- Shelves in supermarket are 50~60 cm width.

- Aisles are 1~2 m width.

If the distance accuracy is larger than the width of the shelves, Ranging service would guide the user to a wrong aisle. So, distance accuracy of 50 cm is necessary.

Direction accuracy:

Alice is walking to the aisles with the distance of about 2 meters. She needs to choose an aisle to get her desired goods. Her desired goods are deep in the aisle, about 5 meters (r) to her, which cannot be seen. Alice needs to use ranging service to find the right aisle.

It’s assumed that the shelves are 50 cm (d) wide. If the direction mistake is larger than 5.73 degree (d/r*180/PI), Ranging service would guide to a wrong aisle.

So, direction accuracy of 5 degree would be necessary.

Latency:

1 second total latency is satisfying to user, which is from user clicking searching button to the app giving out the item’s position.

The server of supermarket takes 100 ms to give information of the item.

Round trip transmission takes about 200 ms.

The phone takes 130 ms to pair with the item.

The app takes 67 ms (2 frames of display) to display the position.

So, the time budget would be about 500 ms (1s – 100ms – 200ms – 130ms – 67ms = 503ms).

Availability:

95% availability will ensure a user friendly experience for Finding items in a Supermarket.

Maximum distance of Ranging service:

Distance of 100 meters of ranging service would ensure a user to find his/her desired item in a large supermarket.

Number of concurrent ranging operation:

A middle-sized supermarket can support 500 people at the same time. We suppose 20% of them need ranging service. There would be 100 UEs using Ranging service at the same time.

3. Proposal

Proposal：The square brackets in table 5.4.6-1 in 22.855 are removed.

**************** First Change ******************

### 5.4.6	Potential New Requirements needed to support the use case

Table 5.4.6-1 – KPIs for finding items in a supermarket

| Ranging scenario | Ranging Accuracy (95 % confidence level) |  | Availability | Latency | Effective Ranging distance, Coverage, NLOS/LOS, Relative UE velocity, Ranging interval, Number of concurrent ranging operation for a UE, Number of concurrent ranging operation in an area |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Distance Accuracy | Direction Accuracy |  |  | Effective Ranging distance | Coverage | NLOS/LOS | Relative UE velocity | Ranging interval | Number of concurrent ranging operation for a UE | Number of concurrent ranging operation in an area |
| Smart Vehicle Key | 50 cm | 5 degree | 95 % | 500ms | 100m | IC/PC/OOC | - | Static/ Moving (<1m/s) | - | - | 100 UEs/ (3.14*104m2) |

************* End of First Change ***************