3GPP TSG-SA WG1 Meeting #92e	S1-20xxxx

Online, 09 November – 18 November 2020

Title:	 FS_Ranging KPI analysis on Smart Vehicle Key Use Case

Agenda Item:	7.10.1

Source:	Spreadtrum

Contact:

1. Background

In last SA1 meeting, the numbers in the KPI table of smart vehicle key were marked in square bracket due to concerns on the value. In this contribution, we try to analyze and justify these values, and suggest removing the square brackets.

2. Discussion

The KPIs for smart vehicle key is defined in Table 5.1.6-1 in 22.855, as cited below:

Table 5.3.6-1 – KPIs for smart vehicle key

| Ranging scenario | Ranging Accuracy (95 % confidence level) |  | Availability | Latency | Effective Ranging distance, Coverage, NLOS/LOS, Relative UE velocity, Ranging interval, Number of concurrent ranging operation for a UE, Number of concurrent ranging operation in an area |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Distance Accuracy | Direction Accuracy |  |  | Effective Ranging distance | Coverage | NLOS/LOS | Relative UE velocity | Ranging interval | Number of concurrent ranging operation for a UE | Number of concurrent ranging operation in an area |
| Smart Vehicle Key | [1]m | - | [95 %] | [50ms] | [100m] | IC/PC/OOC | - | Static/ Moving (<1m/s) | - | - | [40]UEs/ (104m2) |

We assume the ranging antenna is on the top of the vehicle, which is shown in the figure below:

In lock and unlock scenario, user’s experience and security issues need to be balanced. There are several requirements need to be considered:

- The vehicle should be unlocked and keep unlocked, when the UE is within a certain distance (e.g., 1 meter) to any door of the vehicle. So that user can get friendly experience.

- The vehicle should be locked and keep locked, when the UE is out of a certain distance to all doors (e.g., 3 meters). So that the vehicle can keep safe.

- The vehicle should keep the same status, when the UE moves to a certain distance range (e.g., from 1 meter to 3 meters) to the closest door, in case the vehicle switches status to often.

- The vehicle should be able to predict the UE’s position to unlock itself at the moment the UE moves within the certain range, even when the UE is moving quickly.

Distance Accuracy:

Above requirements need Ranging service to detect the smart key’s position and measure the distance of 1 meter accurately, which need a distance accuracy of lower by one order of magnitude than 1 meter, i.e., 10 cm.

Direction Accuracy:

It’s assumed that, a door is 1.5 m to the antenna. Direction mistake of 10 degree can cause 10cm distance mistake, which can reduce user experience. So, direction accuracy of 10 degree is needed.

Availability:

99% availability will ensure a user friendly experience for predicting moving UE’s position.

Latency:

A user hopes to unlock his vehicle immediately when he’s running to his car at 2m/s. It’s set up that the car unlocks itself when the user’s phone gets within 1 meter. The car must be unlocked when the phone gets within 90 cm to ensure friendly experience. So, the budget for ranging would be 50ms.

Effective Ranging distance

We assume:

- A vehicle can be 10 meters long.

- Ranging service should measure a distance of 10 meters to the vehicle.

- Ranging service should standby for 5 seconds before unlocking the vehicle, even when the user is running at 2m/s.

Effective Ranging distance should ensure that the vehicle standby to unlock in time. So, effective ranging distance should be 30 m.

Ranging Interval:

Ranging interval should ensure that the measured result is still valid (with sufficient accuracy) during this period when user moves. For 2m/s user movement, 25ms ranging interval corresponds to 5cm distance change, which is acceptable for 10cm distance accuracy.

Number of concurrent ranging operation for in an area:

A standard parking slot is about 40 m2. A parking place of 104 m2 can support 250 vehicles. We assume 20% of all the vehicles need ranging service at the same time of rush hour. So, the number of concurrent ranging operation in this parking place should be 50.

3. Proposal

Proposal：The square brackets in table 5.3.6-1 in 22.855 are removed.

**************** First Change ******************

### 5.3.6	Potential New Requirements needed to support the use case

Table 5.3.6-1 – KPIs for smart vehicle key

| Ranging scenario | Ranging Accuracy (95 % confidence level) |  | Availability | Latency | Effective Ranging distance, Coverage, NLOS/LOS, Relative UE velocity, Ranging interval, Number of concurrent ranging operation for a UE, Number of concurrent ranging operation in an area |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Distance Accuracy | Direction Accuracy |  |  | Effective Ranging distance | Coverage | NLOS/LOS | Relative UE velocity | Ranging interval | Number of concurrent ranging operation for a UE | Number of concurrent ranging operation in an area |
| Smart Vehicle Key | 10 cm | 10 degree- | 99 % | 50ms | 30m | IC/PC/OOC | - | Static/ Moving (<2m/s) | - | - | 50 UEs/ (104m2) |

************* End of First Change ***************