3GPP TSG-SA WG1 Meeting #92e	S1-20xxxx

Online, 09 November – 18 November 2020

Title:	 FS_Ranging Consolidated Requirements

Agenda Item:	7.10.1

Source:	Xiaomi

Contact:	Xiaowei Jiang (jiangxiaowei@xiaomi.com)

Abstract: consolidated requirements section of FS_Ranging TR22.855  and will be updated based on the email discussion results in this meeting.

**************** First Change ******************

6	Consolidated requirements

6.1	Functional requirements

The 5G system shall be able to enable or disable the Ranging service.

The 5G system shall be able to support for a UE to discover other UEs supporting Ranging.

The 5G system shall allow operator to authorize each individual UE for Ranging if licensed band is used.

The 5G system shall support mutual ranging, i.e. the two UE’s involved shall be able to range each other.

The 5G system shall be able to protect the privacy of UE and its user to ensure that no identifiable information and location information can be tracked when ranging is performed.

The 5G system shall be able to protect confidentiality, integrity and authenticity of ranging related information.

6.2	Performance requirements

| Ranging scenario | Ranging Accuracy (95 % confidence level) |  | Availability | Latency | Effective Ranging distance, Coverage, NLOS/LOS, Relative UE velocity, Ranging interval, Number of concurrent ranging operation for a UE, Number of concurrent ranging operation in an area |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Distance Accuracy | Direction Accuracy |  |  | Effective Ranging distance | Coverage | NLOS/LOS | Relative UE velocity | Ranging interval | Number of concurrent ranging operation for a UE | Number of concurrent ranging operation in an area |
| Distance based smart device control | 10cm | - | 99 % | 50ms | 20m | IC/PC/OOC | LOS | Static/ Moving (<1m/s) | 50ms | 20 | - |
| Distance based smart home TV control | [10cm] up to 2 meter separation | [±2°] horizontal direction accuracy at 0.1 to 2 meter separation and AoA coverage of (-60°) to (+60°); [±2°] Elevation direction accuracy at 0.1 to 2 meter separation and AoA coverage of (-45°) to (+45°) | [99 %] | [10ms] | [10m] | IC/PC/OOC | LOS | Static/ Moving (<[10m/s]) | [50ms] | - | - |
| Smart vehicle key | [1]m | - | [95 %] | [50ms] | [100m] | IC/PC/OOC | - | Static/ Moving (<1m/s) | - | - | [40]UEs/ (104m2) |
| Finding items in a supermarket | [1]m | - | [95 %] | [50ms] | [100m] | IC/PC/OOC | - | Static/ Moving (<1m/s) | - | - | [40]UEs/ (104m2) |
| Touchless Self-checkout Machine Control |  |  |  |  |  |  |  |  |  |  |  |
| Smart Transportation Metro/Bus Validation |  |  |  |  |  |  |  |  |  |  |  |
| Distance based Intelligent Perception for Public Safety |  |  |  |  |  |  |  |  |  |  |  |
| Picture and video sharing based on ranging results | 10cm | 2° | 99 % | [50ms] | 10m | IC/PC/OOC | LOS | Static/ Moving (<1m/s) | 50ms | - | - |
| Ranging of UE’s in front of vending machine | [20cm] | [10°] | - | [150ms] | [5m] | IC/PC/OOC | LOS | Static/ Moving (<1m/s) | [50ms] | - | [10] |

************* End of First Change ***************