3GPP TSG-SA WG1 Meeting #92e	S1-204007r4

Electronic Meeting, 11-20 November 2020	(revision of S1-xxxxxx)

Title:	FS_Resident: New use case on Local control of Premises Radio Access Station connectivity for UE access to a Customer Premises Network device

Agenda Item:	7.11.1 FS_Resident

Source:	InterDigital, KPN

Contact:	Atle Monrad <atle dot monrad at interdigital dot com>

Abstract

This document proposes a new use case for the study item FS_Resident on local control of premises radio access station for UE access to the customer premises network device.

Proposal

It is proposed to include the below new use case on local control of premises radio access station for UE access to a customer premises network device in 3GPP TR 22.858 version 0.1.0.

* * * *   First Change   * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: " Service requirements for the 5G system; Stage 1".

[3]	3GPP TS 22.220: "Service requirements for Home Node B (HNB) and Home eNode B (HeNB)".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

It is preferred that the reference to 21.905 be the first in the list.

* * * *   Second Change   * * * *

## 5.x	Use case: local control of Premises Radio Access Stations connectivity for UE access to a Customer Premises Network device

### 5.x.1	Description

In many residential scenarios, indoor coverage will require deployment of multiple PRASs or Wi-Fi Access Points (APs). It is expected that both 3GPP RATs and non-3GPP RATs (e.g. Wi-Fi) offer wireless connectivity allowing UEs to connect to local as well as distant networks via the eRG, thus, as users move through the home, switching of connectivity between different PRASs or APs is required to maintain service quality.

For certain services requiring low latency and high throughput such as playing an interactive game on a VR/AR headset UE, there is a need to connect to a device within the CPN, for example a CPN device like a gaming console or a set-top-box.

A gaming console (CPN device) will provide the necessary service experience such as reactiveness when computing and rendering a new AR/VR scene displayed on the VR/AR headset (UE #1). In this use case, the CPN device can either be a 3GPP device, a non-3GPP device or support both RATs (3GPP and non-3GPP). The gaming console is part of the CPN.

The VR/AR headset (UE #1) will connect to the gaming console via one of the points of access (PRAS or AP) connected to the eRG.

As a player moves around in the home, there will be a need to handover the VR/AR headset (UE #1) connection from one PRAS or AP to another. Although the VR/AR headset (UE #1) can connect simultaneously to an AP and to a PRAS, the VR/AR headset (UE #1) is not able to take educated decisions on its own with regards to capabilities of alternative accesses in the home, e.g. if a candidate handover access is overloaded. However, a centralized application can distribute accesses with respect to relevant applicative parameters like VR/AR headset, the position of the UE and buffer level. Besides, the centralized application can know where the different players are and where they can be expected to move as the game progress.

The game application can also know the overall context for all players and can compute the best path for each UE for connecting to a particular point of access (PRAS or AP). This application can predict and adapt access to each 3GPP RAT or non-3GPP RAT for providing the best global service experience applied to each individual player at each time. The application can also learn from individual service experience resulting from past decisions applied to input parameters to improve access decisions.

Therefore, the gaming console (CPN device) needs to control which particular VR/AR headset (UE #1 or UE #2) connects to a particular point of access (PRAS #1 or PRAS #2 or AP #1 or AP #2) to reach the application. For example, the gaming console (CPN device) can disable a VR/AR headset (UE #1) to connect PRAS (PRAS #2) in order to force this VR/AR headset (UE #1) to connect to PRAS (PRAS #1).

Figure 5.x.1-1. Local control of PRASs for UE #1 to access CPN device

### 5.x.2	Pre-conditions

John and his game player friend Paul wear a 5G-enabled VR/AR headset (UE #1, UE #2). They love to play their favourite immersive game taking place throughout the whole house. The following pre-conditions and assumptions apply to this use case.

- Multiple PRASs (PRAS #1, PRAS #2) (3GPP RAT) are deployed in John’s home, connected to the eRG.

- The home is equipped with several Wi-Fi APs (AP #1, AP #2) (non-3GPP RAT), connected to the eRG.

- Game players use VR/AR headsets (UE #1, UE #2) that require low latency and high throughput.

- There is a gaming console (CPN device) in the home providing the expected gaming service experience such as reactiveness when computing and rendering a new AR/VR scene to be display on a VR/AR headset (UE #1, UE #2).

- The gaming console (CPN device) is connected to the CPN via the eRG.

- Game players VR/AR headsets (UE #1, UE #2) can communicate with the gaming console over either of the 3GPP or non-3GPP RATs and the eRG.

- The game play application knows the user locations.

### 5.x.3	Service Flows

The service flow for this use-case is as follows:

- John launches the multi-player augmented reality game from the residential gaming console (CPN device) indicating multi-RAT connectivity;

- the gaming console (CPN device) communicates with the PRASs to allow to control of the connectivity from the small base station to the gaming console;

- John and Paul start playing the game from the living-room and power their VR/AR headset (UE #1 and UE #2);

- each VR/AR headset (UE #1, UE #2) initially connects the gaming console from any PRAS or Wi-Fi AP e.g. John’s VR/AR headset (UE #2) is connected through Wi-Fi (AP #1);

- the gaming console (CPN device) updates the game play each time a player moves indoor and forces the AR/VR headset (UE #1, UE #2) to connect to a particular point of access (PRAS #1 or PRAS #2 or AP #1 or AP #2)

- when Paul moves, the gaming console (CPN device) communicates with the PRASs (enable UE #1 on PRAS #1, disable UE #1 on PRAS #2) to force the VR/AR headset (UE #1) to connect to PRAS #1 to reach the gaming console (CPN device).

### 5.x.4	Post-conditions

John and his friends can move throughout the home, experiencing no latency when their VR/AR headset displays an augmented reality scene.

### 5.x.5	Existing features partly or fully covering the use case functionality

3GPP TS 22.261 [2] clause 6.3.2.1: The 5G system shall be able to support mobility between the supported access networks (e.g. NG-RAN, WLAN, fixed broadband access network, 5G satellite access network).

3GPP TS 22.261 [2] clause 6.3.2.4: The 5G system shall support use of a home base station that supports multiple access types (e.g. 5G RAT, WLAN access, fixed broadband access).

3GPP TS 22.261 [2] clause 6.7.2: A 5G system with multiple access technologies shall be able to select the combination of access technologies to serve a UE on the basis of the targeted priority, pre-emption, QoS parameters and access technology availability.

3GPP TS 22.220 [3] clause 5.5.3: It shall be possible to support service continuity, including handover, between a H(e)NB cell and other cells and between H(e)NB cells. This includes H(e)NB cells in residential and enterprise environment.

3GPP TS 22.220 [3] clause 5.7.2: A H(e)NB subsystem shall be able to support Local IP Access in order to provide access for IP capable UEs connected via a H(e)NB subsystem (i.e. using H(e)NB radio access) to other IP capable entities in the same residential/enterprise IP network.

Editor’s Note:	3GPP TS 22.220 [3] is referred, as it is not clear whether these requirements also apply to 5G.

### 5.x.6	Potential New Requirements needed to support the use case

Editor’s Note:	The requirements are FFS.

* * * *   End of Changes   * * * *