3GPP TSG-SA WG1 Meeting #92e	S1-204012

Electronic Meeting, 11-20 November 2020	(revision of S1-20xxxx)

Title:	FS_AMMT – New use-case on local AI/ML model split on factory robots

Agenda Item:	7.4.1

Source:	InterDigital

Contact:	Atle Monrad <Atle.Monrad@InterDigital.com>

Abstract

AI-driven robots are more and more commonly used in factories to accomplish fast, complex and repetitive tasks. In a near future, mobile factory robots will operate in open environments with human operators and other robots. This will require the acquisition and treatment in real-time of a huge amount of sensing data. Those robots will not always be connected to the 5G network with as consequences to handle locally the data and to interact with other robots via proximity communication means. This perspective brings new challenges in terms of AI/ML model split for robots.

Proposal

It is proposed to include the below new use case on local AI/ML model split on factory robots in 3GPP TR 22.874 version 0.1.0.

* * * *   First Change   * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 22.891, Feasibility Study on New Services and Markets Technology Enablers

[3]	3GPP TR 22.863, Feasibility study on new services and markets technology enablers for enhanced mobile broadband

[4]	3GPP TS 22.261, Service requirements for the 5G system

[5]	3GPP TS 22.104, Service requirements for cyber-physical control applications in vertical domains

[6]	3GPP TR 37.910, Study on self evaluation towards IMT-2020 submission (Release 16)

[7]	A. Krizhevsky, I. Sutskever, and G. E. Hinton, “ImageNet classification with deep convolutional neural networks”, in Proc. NIPS, 2012, pp. 1097–1105.

[8]	K. Simonyan and A. Zisserman, “Very deep convolutional networks for large-scale image recognition,” 2014, arXiv:1409.1556. [Online]. Available: https://arxiv.org/abs/1409.1556

[9] 	C. Szegedy, et al., “Going deeper with convolutions”, in Proc. CVPR, 2015, pp. 1-9.

[10]	Zhi Zhou, Xu Chen, En Li, Liekang Zeng, Ke Luo, Junshan Zhang, “Edge intelligence: Paving the last mile of artificial intelligence with edge computing”, Proceeding of the IEEE, 2019, Volume 107, Issue 8.

[11]	Jiasi Chen, Xukan Ran, “Deep learning with edge computing: A review”, Proceeding of the IEEE, 2019, Volume 107, Issue 8.

[12]	I. Stoica et al., “A Berkeley view of systems challenges for AI”, 2017, arXiv:1712.05855. [Online]. Available: https://arxiv.org/abs/1712.05855

[13] 	Y. Kang et al., “Neurosurgeon: Collaborative intelligence between the cloud and mobile edge”, ACM SIGPLAN Notices, vol. 52, no. 4, pp. 615–629, 2017.

[14]	E. Li, Z. Zhou, and X. Chen, “Edge intelligence: On-demand deep learning model co-inference with device-edge synergy”, in Proc. Workshop Mobile Edge Commun. (MECOMM), 2018, pp. 31–36.

[15]	3GPP TR 38.913, Study on Scenarios and Requirements for Next Generation Access Technologies (Release 15)

[16]	B. Kehoe, S. Patil, P. Abbeel, and K. Goldberg, “A survey of research on cloud robotics and automation,” IEEE Transactions on automation science and engineering, vol. 12, no. 2, pp. 398–409, 2015.

[17]	Huaijiang Zhu, Manali Sharma, Kai Pfeiffer, Marco Mezzavilla, Jia Shen, Sundeep Rangan, and Ludovic Righetti, “Enabling Remote Whole-body Control with 5G Edge Computing”, to appear, in Proc. 2020 IEEE/RSJ International Conference on Intelligent Robots and Systems. Available at: https://arxiv.org/pdf/2008.08243.pdf

[18] 	K. He, X. Zhang, S. Ren, and J. Sun, “Deep residual learning for image recognition,” in Proc. IEEE CVPR, Jun. 2016, pp. 770-778.

[19] 	A. G. Howard et al., “MobileNets: Efficient convolutional neural networks for mobile vision applications,” 2017, arXiv:1704.04861. [Online]. Available: https://arxiv.org/abs/1704.04861

[20] 	B. Taylor, V. S.Marco, W. Wolff, Y. Elkhatib, and Z. Wang, “Adaptive deep learning model selection on embedded systems,” in Proc. ACM LCTES, 2018, pp. 31–43.

[21] 	G. Shu, W. Liu, X. Zheng, and J. Li, “IF-CNN: Image-aware inference framework for CNN with the collaboration of mobile devices and cloud”, IEEE Access, vol. 6, pp. 621–633, 2018.

[22] 	D. Stamoulis et al., “Designing adaptive neural networks for energy-constrained image classification”, in Proc. ACM ICCAD, 2018, Art. no. 23.

[23] 	Sergey Ioffe and Christian Szegedy. “Batch normalization: Accelerating deep network training by reducing internal covariate shift”, In ICML., 2015.

[24]	C.-J. Wu et al., “Machine learning at facebook: Understanding inference at the edge,” in Proc. IEEE Int. Symp. High Perform. Comput. Archit. (HPCA), Feb. 2019, pp. 331–344.

[25]	Vivienne Sze, Yu-Hsin Chen, Tien-Ju Yang, Joel S. Emer, “Efficient processing of deep neural networks: A tutorial and survey”, Proceeding of the IEEE, 2017, Volume 105, Issue 12.

[26]	Y. LeCun, Y. Bengio, and G. Hinton, “Deep learning,” Nature, vol. 521, no. 7553, pp. 436-444, May 2015.

[27]	“An All-Neural On-Device Speech Recognizer”, March 12, 2019, Posted by Johan Schalkwyk, https://ai.googleblog.com/2019/03/an-all-neural-on-device-speech.html

[28]	Yanzhang He, etc., “Streaming End-to-end Speech Recognition for Mobile Devices”, 2019 IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP 2019)

[29]	3GPP TS 22.243: "Speech recognition framework for automated voice services; Stage 1".

[30]	H. B. McMahan, E. Moore, D. Ramage, S. Hampson, and B. A. Y. Arcas, “Communication-efficient learning of deep networks from decentralized data”, Proc. of the International Confe rence on Artificial Intelligence and Statistics, Apr. 20 17. [Online]. Available: https://arxiv.org/abs/1602.05629

[31] 	“Federated Learning”, https://justmachinelearning.com/2019/03/10/federated-learning/

[32]	T. Nishio and R. Yonetani, “Client selection for federated learning with heterogeneous resources in mobile edge”, 2018, arXiv:1804.08333. [Online]. Available: https://arxiv.org/abs/1804.08333

[33]	E. Park et al., “Big/little deep neural network for ultra low power inference”, in Proc. 10th Int. Conf. Hardw./Softw. Codesign Syst. Synth., 2015, pp. 124–132.

[34]	Nguyen H. Tran ; Wei Bao ; Albert Zomaya ; Minh N. H. Nguyen ; Choong Seon Hong, “Federated Learning over Wireless Networks: Optimization Model Design and Analysis”, In proc. IEEE INFOCOM 2019 - IEEE Conference on Computer Communications

[35] 	Song Han, Xingyu Liu, Huizi Mao, Jing Pu, Ardavan Pedram, Mark A Horowitz, and William J Dally. “EIE: efficient inference engine on compressed deep neural network”, In 43rd International Symposium on Computer Architecture, IEEE Press, 243–254.

[36] 	V. Sze, “Efficient Computing for Deep Learning, AI and Robotics,” Dept EECS, MIT, Available online at https://lexfridman.com/files/slides/2020_01_15_vivienne_sze_efficient_computing.pdf

[37] 	V. Sze, Y. Chen, “Efficient Processing of Deep Neural Networks: A Tutorial and Survey” Proc. of IEEE, 2017, Available online at: https://www.semanticscholar.org/paper/Efficient-Processing-of-Deep-Neural-Networks%3A-A-and-Sze-Chen/3f116042f50a499ab794bcc1255915bee507413c

[38] 	Stanford University, CS231n – Lecture 5-7: CNN, Training NNs, Available at YouTube.com

[39] 	S. Han, J. Pool, J. Tran, and W, J. Dally, "Learning both weights and connections for efficient neural networks", NIPS, May 2015

[40]	P. A. Merolla, et al., “A million spikingneuron integrated circuit with a scalable communication network and interface”,Science, vol. 345, no. 6197, pp. 668–673, Aug. 2014.

[41]	R. Collobert, J. Weston, L. Bottou, M. Karlen, K. Kavukcuoglu, and P. Kuksa, “Natural language processing (almost) from scratch,” J. Mach. Learn. Res., vol. 12 pp. 2493–2537, Aug. 2011.

[42]	T. N. Sainath, A.-R. Mohamed, B. Kingsbury, and B. Ramabhadran, “Deep convolutionalneural networks for LVCSR”, in Proc. ICASSP, 2013, pp. 8614–8618.

[43]	L. P. Kaelbling, M. L. Littman, and A. W. Moore, “Reinforcement learning: A survey”, J. Artif. Intell. Res., vol. 4, no. 1, pp. 237–285, Jan. 1996.

[44] 	3 AI Trends for Enterprise Computing. [Online]. Available: https://www.gartner.com/smarterwithgartner/3-ai-trends-for-enterprise-computing/

[45] 	Shiming Ge ; Zhao Luo ; Shengwei Zhao ; Xin Jin ; Xiao-Yu Zhang, “Compressing deep neural networks for efficient visual inference”, In proc. 2017 IEEE International Conference on Multimedia and Expo (ICME)

[46]	3GPP TS 23.303, "Proximity-based services (ProSe); Stage 2"

[47]	Yusuf Aytar ; Carl Vondrick ; Antonio Torralba, “SoundNet: Learning Sound Representations from Unlabeled Video”, 27 Oct 2016

[48]	Shawn Hershey et al., “CNN ARCHITECTURES FOR LARGE-SCALE AUDIO CLASSIFICATION”, 10 Jan 2017

* * * *   Second Change   * * * *

## 5.x	Local AI/ML model split on factory robots

### 5.x.1	Description

In a modern factory, a team on a workstation comprises two human operators, two mobile robots and a fixed robot. Everyone has his own pre-defined task. Robots assist the human operators by accomplishing painful tasks in a fluid and precise manner; they also monitor that the workstation environment remains safe for the human operators. The mobile robots shall not interfere with humans and between them.

The robot control is not executed on a distant server in the cloud because reliability and confidentiality were not ensured at a sufficient level. Furthermore, the overall end-to-end latency is not always guaranteed which can cause production loss. Communications between robots rely on private wireless networks in the factory that enable expected QoS (reliability, throughput, and latency) as well as confidentiality.

The new robots are autonomous robots that can react to human voices or learn in real-time what operators do. They can perceive their environment and transmit information to other robots. They can communicate, learn from each other, assist each other and do self-monitoring.

The robot’s skills rely on several AI/ML models running on the robot itself which has inconvenience for mobile robot to drain their battery quicker. To overcome this issue, when the battery level reaches a certain value, a part of AI/ML model is transferred to another robot by splitting the AI/ML model as defined in [13]. The split model approach of [13] is applicable to a UE-to-UE (or robot-to-robot) architecture. Indeed, once the AI/ML model is split, both split AI/ML sub-models can be located on two different connected devices. The location of the devices doesn’t matter as long as they have sufficient and adapted resources to run the AI/ML sub-model. Thus, the AI/ML model M is split and shared between (e.g.) 2 robots, say an assisted robot and an assistant robot. Intermediate data generated by the assisted robot are transferred to the assisting robot which finalizes the inference and transmits the results back to the assisted robot. This intermediate data transfer shall be extremely efficient in terms of latency and throughput. When many models are at stake, the split model method is an additional challenge for the 5GS in terms of throughput and latency.

Table 5.x.6-1 shows expected data rates for some typical AI/ML models depending on the amount of intermediate data to be transferred and the transfer time. For each model, many split points are envisioned. The amount of intermediate data vary from one split point to the other.

We consider two situations:

- Intermediate data size below 1 MB with a transfer time of 5 ms. This gives a 1.6 Gb/s data rate. This configuration will cover most of the split cases.

- Intermediate data above 1 MB with a less stringent transfer time of 50 ms. This configuration covers the rest of the split cases.

Table 5.x.1-1: Data rate for intermediate AI/ML data

| Model Name | Model type | Intermediate data size (MB) | Transfer time (ms) | Data rate (Mb/s) |
| --- | --- | --- | --- | --- |
| AlexNet [7] | Image recognition | [0.02 – 0.27] | 5 | [32 – 432] |
| VGG-16 [8] | Image recognition | [0.1 – 0.8] | 5 | [16 – 1280] |
|  |  | [1.5 – 3] | 50 | [240 – 480] |
| ResNet50 [18] | Image recognition | [0.1 – 0.8] | 5 | [16 – 1280] |
|  |  | [1.6 – 3.2] | 50 | [256 – 512] |
| SoundNet [47] | Sound recognition | [0,0017 – 0,88] | 5 | [2.72 – 1408] |

As previously said latency is a critical requirement. Figure 5.x.1-1 summarizes the latency cost in three cases:

- The inference of model M is done locally. Latency is denoted LLI.

- The inference process is fully offloaded on a second device (Robot/UE). Latency is denoted LFO.

- The inference process is partially offloaded on a second device (Robot/UE). Latency is denoted LPO.

Figure 5.x.1-1 Latency summary

The current Use Case promotes the (C) scenario where a model M is split in two sub-models Ma and Mb. If both robots (UEs) have a similar computing power, the assumption is that the latency due to the inference of model M is almost equal to the latency of model Ma plus the latency of model Mb.

Hence, once the split model is deployed on the two robots (UEs), the aim is to minimize the E2E latency and to be as close as possible to the non-split case. This is done with a transfer delay of both the intermediate data and the inference results as small as possible. We may note that if the computing power on the assistant robot is more important, then (C) would be the preferred scenario.

In scenario (B), the inference process is fully offloaded on the assistant robot (UE). The major inconvenience is the strong and negative impact on latency of the raw data transfer towards the assistant robot.

### 5.x.2	Pre-conditions

Two human operators are working.

Two mobile AI-driven robots (Arobot and Brobot) and one static AI-driven robot (Crobot) assist them.

The three robots (Arobot , Brobot and Crobot ) belong to the same service area, embed the same two powerful AI/ML models M1 and M2, sensors (e.g. LIDAR, microphone) and cameras (e.g. 8K videos stream).

Arobot and Brobot are powered with a battery and Crobot with fixed ground power.

The three robots (Arobot , Brobot and Crobot) are locally connected.

The workstation is equipped with camera and sensors.

The service area is 30 m x 30 m and the robot speed is at maximum 10 km/h.

The service area is covered by a small cell.

### 5.x.3	Service Flows

Figure 5.x.3-1 Factory service flow

- Brobot battery is rather low but it can still work for a while if a part of its machine learning process is offloaded.

- Brobot broadcasts a request message to get assistance. Crobot responses positively.

- Brobot negotiates with Crobot what parts of M1 and M2 for the inference process it may have the charge of, knowing that the quality of the prediction shall not be under a certain level and that the end-to-end latency shall not be above a certain value.

- Brobot and Crobot agree on a split point and Brobot starts sending the intermediate data to Crobot.

- Crobot infers and transmits in unicast the predictions back to Brobot in a very short delay.

- In the meanwhile, Arobot is carrying a load to the opera tor Aoperator.

- Aoperator bends down to pick up a screw that has fallen on the floor. At the same time Boperator is passing between Aoperator and Arobot. Arobot can’t see Aoperator anymore.

- Brobot is busy with another task but it can see the scene.  It reports the scene as intermediate data to Crobot.

- Crobot infers.  In addition to transmit in unicast the prediction back to Brobot, Crobot broadcasts an alert message telling that the operator Aoperator is endangered.

### 5.x.4	Post-conditions

Intermediate data can be exchanged between two robots (UEs) and robot with a low battery level can continue working for a while.

All the robots in the group receive the alert message and react:

- They all stop working; or

- Arobot changes its trajectory.

Aoperator and Boperator can work safely.

The huge amount of data that is required for inferring is kept locally.

### 5.x.5	Existing features partly or fully covering the use case functionality

The Use Case can rely on the Proximity Service (ProSe) services as defined in 3GPP TS 23.303 [46].

3GPP TS 23.303 [46] defines ProSe Group communication. Arobot, Brobot and Crobot belong to that group. When Brobot battery level is low, Brobot broadcasts a message asking for a support.

### 5.x.6	Potential New Requirements needed to support the use case

[P.R.5.x-001] The 5G system shall support the transfer of AI/ML model intermediate data from UE to UE via the D2D/sidelink communication path or the 5GC path.[P.R.5.x-002] The 5G system shall support a 1.6 Gb/s data rate from UE to UE via the D2D/sidelink communication path or via the 5GC path (UL and DL).

[P.R.5.x-003] The 5G system may support a 10 Gb/s data rate from UE to UE via the D2D/sidelink communication path or the 5GC path (UL and DL).

[P.R.5.x-004] The 5G system shall guarantee a latency of [5ms-50ms] from UE to UE via the D2D/sidelink communication path or the 5GC path.

[P.R.5.x-005] The 5G system shall guarantee a communication service availability higher than 99.9999 % for the communication path (D2D/sidelink or 5GC) within a service area of 30m x 30m when robots are moving with a 10 km/h speed.

[P.R.5.x-006] The 5G system shall guarantee a QoS (latency, reliability, data rate as defined above) of the communication path (D2D/sidelink or 5GC).

[P.R.5.x-007] The 5G system shall be able to adjust the QoS of the communication path (D2D/sidelink or 5GC).

Note 1: The split point selection is dynamic. In consequence, the amount of intermediate data will vary. To maintain the QoS, the bandwidth shall be adjusted.

[P.R.5.x-008] The 5G system shall support a mechanism to broadcast messages via the D2D/sidelink communication path or the 5GC path.

Note 2: The broadcast messages can be:

- “assistance message” to request for offload resources

- “alert message” to warn that a human operator is endangered

* * * *   End-of Changes   * * * *