3GPP TSG-SA WG1 Meeting #92e	S1-204015r2

Electronic Meeting, 11-20 November 2020	(revision of S1-20xxxx)

Title:	FS_AMMT – Updating of new requirements and KPIs for the use-case on real time media editing with on-board AI inference

Agenda Item:	7.4.1

Source:	InterDigital

Contact:	Atle Monrad <Atle.Monrad@InterDigital.com>

Abstract:

The potential new requirements for the use case on real time media editing with on-board AI inference is reviewed and updated.

All Editor’s Notes are considered and potential KPIs are included. For the potential KPIs, two cases are considered, “independent” (single) user and “crowd”.

Proposal

It is proposed to include the below improvements to clause 6.2.6 for the use case on real time media editing with on-board AI inference in 3GPP TR 22.874 version 0.1.0.

* * * *   First Change   * * * *

### 6.2.6	Potential New Requirements needed to support the use case

### 6.2.6.1	Introduction

Potential new requirements needed to support the use case result of the following parameters:

- AI/ML model size.

- latency constraint of the application or service.

- number of concurrent downloads, i.e. number of UEs requesting AI/ML model downloads within same time window.

The number of concurrent downloads further depends on the density of UE in the covered area and the covered area size.

The tables 6.2.6-1, 6.2.6-2 and 6.2.6-3 contain KPI for different aspects of the real-time media editing use case.

Table 6.2.6-1. Typical sizes of AI/ML models for the UC

| AI/ML Model | Number of parameters (Million) | Size of the AI/ML model (MByte) | Comments |
| --- | --- | --- | --- |
| MobileNet | 3.2 | 3.2 | 8-bit parameters |
| MobileNet | 3.2 | 12.8 | 32-bit parameters |
| RCAN | 15.44 | 15.44 | 8-bit parameters |
| DarkNet | 20 | 20 | 8-bit parameters |
| Inception v4 | 41 | 41 | 8-bit parameters |
| RCAN | 15.44 | 61.78 | 32-bit parameters |
| YOLONet | 64 | 64 | 8-bit parameters |
| DarkNet | 20 | 80 | 32-bit parameters |
| VGGNet | 134 | 134 | 8-bit parameters |
| Inception v4 | 41 | 164 | 32-bit parameters |
| YOLONet | 64 | 256 | 32-bit parameters |
| VGGNet | 134 | 536 | 32-bit parameters |

From Table 6.2.6-1, AI/ML models currently available to elaborate the use case have sizes that vary from 3.2 MB to 536 MB.

As indicated in the use case in clause 6.1, it can be noted that the size of AI/ML models can be reduced prior to transmission with dedicated model compression techniques. On the contrary, AI/ML models with more neural network layers and more complex architectures arise to solve more complex tasks and to improve accuracy. This trend is expected to continue in the coming years. Typical model sizes in the range of 3 MB to 500 MB appear to be a reasonable compromise to consider for this use case.

In the following, two categories are considered for AI/ML model sizes:

- AI/ML model sizes below 64 MB, which can be associated to models optimized for fast transmission,

- AI/ML model sizes below 500 MB, which can be associated to models optimized for higher accuracy.

Latency requirements in function of application or service:

- Videocall service requires an end-to-end latency below 200 ms,

- Video recording, video streaming, and object recognition applications require a latency below 1 s.

DL bitrate requirements resulting from above AI/ML models’ sizes and latency requirements are summarized in the following table:

Table 6.2.6-2. UC AI/ML model download – single UE – KPIs

| UC model download | AI/ML model size | Latency | User experienced DL data rate | Supported |
| --- | --- | --- | --- | --- |
| Single Model / Single UE | [3 MB – 64 MB] | <1 s | [24 Mb/s ~ 512 Mb/s] | Yes |
| Single Model / Single UE | [3 MB – 64 MB] | <200 ms | [120 Mb/s ~ 2.56 Gb/s] | Yes |
| Single Model / Single UE | [64 MB – 500 MB] | <1 s | [512 Mb/s ~ 4 Gb/s] | Yes |
| Single Model / Single UE | [64 MB – 500 MB] | <200 ms | [2.56 Gb/s ~ 20 Gb/s] | No |

As indicated above, the number of concurrent downloads is a third parameter to determine potential new requirements. This corresponds to the maximum number of UEs requesting a AI/ML model download in a same time window and same covered area/cell.

The case of a concert hall is an illustration of the scenario, “Broadband access in a crowd” from 3GPP TS 22.261 [4]. This scenario evoques an overall user density of 500 000 UE / km2  (i.e. 0.5 UE / m2) and an activity factor of 30 %.

In the concert hall case, it is also assumed that only a part of the UEs intends to request AI/ML model downloads. Moreover, only a subpart will request AI/ML model download during the same time window in the same cell. The activity factor is finally estimated to 1 % (i.e. % of UE requesting a AI/ML model download within same time window).

Typical number of UEs in a concert hall vary from ~1000 seats to ~ 5000 seats.

Based on these figures and UE activity assumption, the number of concurrent downloads is estimated as follows.

Table 6.2.6-3. Estimated number of concurrent downloads

| Number of UEs | Estimated area (density of 0.5 UE / m2 ) | Activity Factor | Number of concurrent downloads in the same cell |
| --- | --- | --- | --- |
| 1000 | 2000 m2 | 1 % | 10 |
| 5000 | 10000 m2 | 1 % | 50 |

From Table 6.2.6-2 and Table 6.2.6-3, requirements on the covered area are estimated as follows:

Table 6.2.6-4. Estimated covered area DL bitrate requirements

| Number of UEs | Activity Factor | Number of concurrent downloads | AI/ML model size | Latency | User experienced DL data rate | Covered area DL data rate requirement |
| --- | --- | --- | --- | --- | --- | --- |
| 1000 | 1 % | 10 | [3 MB – 64 MB] | <1 s | [24 Mb/s ~ 512 Mb/s] | [240 Mb/s ~ 5.12 Gb/s] |
|  |  |  | [3 MB – 64 MB] | <200 ms | [120 Mb/s ~ 2.56 Gb/s] | [1.2 Gb/s ~ 25.6 Gb/s] |
|  |  |  | [64 MB – 500 MB] | <1 s | [512 Mb/s ~ 4 Gb/s] | [5.12 Gb/s ~ 40 Gb/s] |
| 5000 | 1 % | 50 | [3 MB – 64 MB] | <1 s | [24 Mb/s ~ 512 Mb/s] | [1.2 Gb/s ~ 25.6 Gb/s] |
|  |  |  | [3 MB – 64 MB] | <200 ms | [120 Mb/s ~ 2.56 Gb/s] | [6 Gb/s ~ 128 Gb/s] |
|  |  |  | [64 MB – 500 MB] | <1 s | [512 Mb/s ~ 4 Gb/s] | [25.6 Gb/s ~ 200 Gb/s] |

Another approach to estimate the number of concurrent downloads is to estimate the number of different AI/ML models requested by UEs instead of the number of UEs requesting AI/ML models. The AI/ML models can then be broadcast/multicast to multiple UEs. The number of different AI/ML models depends on the accuracy expectations of the AI/ML models, the execution environments and the hardware characteristics of end devices. When the number of UE requesting AI/ML models is very high, the number of different AI/ML models can remain smaller. This approach is well suited for very large crowd.

The number of concurrent downloads when transmitted in broadcast/multicast to many UEs can be estimated between 1 (i.e. all UEs request the same AI/ML model) and 50 (i.e. all UEs request different AI/ML models).

#### 6.2.6.2	Potential KPI Requirements

Independent user:

[P.R.6.2-I-001] The 5G system shall support the download of AI/ML models with a latency below 200 ms and a data rate of 2.56 Gb/s.

NOTE 1:	This requirement concerns AI/ML models having a size below 64 MB.

[P.R.6.2-I-002] The 5G system shall support the download of AI/ML models with a latency below 1 s and a data rate of 4 Gb/s.

NOTE 2:	This requirement concerns AI/ML models having a size below 500 MB.

Crowd:

[P.R.6.2-C-001] The 5G system shall support the parallel download of up to 50 AI/ML models with a latency below 1s.

NOTE 3:	This requirement concerns AI/ML models having a size below 64 MB.

[P.R.6.2-C-002] The 5G system should support the functionality to broadcast/multicast a same AI/ML model to many UEs with a latency below 1s.

NOTE 4:	This requirement concerns AI/ML models having a size below 64 MB.

* * * *   End of Changes   * * * *