3GPP TSG-SA WG1 Meeting #92e 	S1-204067r04

Electronic Meeting, 11-20 November	 (revision of S1-20xxxx)

Title:	PINs –PIN definition updates

Agenda Item:	7.12.1 - FS_PINs

Source:	vivo Mobile Communications Ltd

Contact:		Adrian Buckley <adrian.buckley@vivo.com>

Abstract: This contribution proposes to update PIN definition

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

direct device connection: See definition in TS 22.261 [2].

direct network connection: See definition in TS 22.261 [2].

IoT device: See definition in TS 22.261 [2].

PIN Element: the basic component making up a PIN-Users Personal IoT Network.  The PIN Element maybe an IoT device, TE, MT, ME, “thing” (See sub clause 26a.1 3GPP TS 22.101 [3]) or even a complete UE.

PIN Element with Gateway Capability: can act as a gateway that provides access to and from the public operators network (fixed/mobile/cable) and a PIN.

NOTE:	A PIN Element can have both PIN management capability and Gateway Capability.

Editor’s Note:	The relationship with FS_RESIDENT Evolved Residential Gateway still needs FFS.

PIN Element with Management Capability: A PIN Element with PIN management Capability has capability to manage the PIN.

Editor’s Note:	Examples could be added to PIN Element with Management Capability based on agreed usecase.

Personal IoT Network: one or more PIN Elements that communicate with each other.

PIN-User: The PIN-User is the person who owns the PIN with respective subscriptions at one service provider.

Editor’s Note:	The above terms PIN Element, PIN Element with Gateway Capability, PIN Element with Management Capability where agreed at SA1#92e, they need to be applied to all usecases in this TR.  This will be via contribution to the next SA1 meeting.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

PIN	Personal IoT Network