3GPP TSG-SA WG1 Meeting #92e 	S1-204162r1

Electronic Meeting, 10-19 November 2020	(revision of S1-20xxxx)

Title:	FS_EASNS: Clarification on PLMN selection in service flow

Agenda Item:	7.6.1

Source:	Apple

Contact:	Mona Mustapha

Abstract: This document proposes to clarify that PLMN selection occurs in the use case service flow, which was indicated by an Editor's Note.

Proposed Text Change:

## 5.6	Simultaneous access to multiple slices on different VPLMNs

### 5.6.1	Description

A UE has access to multiple network slices when on the HPLMN. When the UE is roaming and the VPLMN the UE is currently registered on only provides a subset of the network slices that the UE needs to use, the UE can connect to another VPLMN at the same time to access the other slices.

### 5.6.2	Pre-conditions

-	UE is subscriber of Operator Toffee

-	When on its home network, Operator Toffee, UE has access to three network slices: (A)pricot, (B)anana and (C)herry

-	UE runs different applications that require the UE to simultaneously access Apricot, Banana and Cherry slices

-	Operator Toffee has agreements with:

-	Operator Chocolate to support Apricot and Banana slices

-	Operator Fudge to support Cherry slice

-	Operator Chocolate is the preferred roaming network

-	UE moves out of Operator Toffee’s coverage area, into an area where the coverage is provided by Operator Chocolate and Operator Fudge

### 5.6.3	Service Flows

1. UE connects with Operator Chocolate, and has access to Apricot and Banana slices

2. UE requires simultaneous access to all three slices, Apricot, Banana and Cherry, due to the different applications running on the UE. However, the VPLMN the UE is registered to, Operator Chocolate, does not provide support for Cherry slice

3. UE is authorised by the HPLMN to additionally access the Cherry slice in the Operator Fudge network.

4. UE performs PLMN selection and connects to Operator Fudge, while retaining the connection to Operator Chocolate