3GPP TSG-SA WG1 Meeting #92e 	S1-204203r1

Electronic Meeting, 16 – 25 November 2020	(revision of S1-20xxxx)

Title:	Underground 3GPP Access

Agenda Item:	7.9.1 (FS_5GSEI)

Document for:	Approval

Source:	Samsung, EUTC, ZTE, China Telecom, Huawei

Contact:	Erik Guttman <erik.guttman@samsung.com>

Abstract: This 22.867 P-CR provides an annex to 3GPP access underground, or deep within facilities with no coverage from outdoor deployments of 3GPP access.

Discussion

There are existing means to achieve underground access. An additional promising approach is described in this contribution. As transport networks are out of scope of 3GPP standards, this is presented as an informative annex.

Proposal

It is proposed to agree to add the following change to TR 22.867.

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	                       3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	SunWeiqing etc.  Generalized Energy Storage Control Strategies on User Side in Power Ancillary Service Market.  Automation of Electric Power System Vol.44 No.2 Jan.25, 2020

[3]	China Southern Power Grid, China Mobile, HuaWei: "White Paper: 5G enables smart grid applications".http://www.sgcio.com/technology/anli/2018/0716/86656.html

[4]  	GB/T 1936-2018 Guidelines of cyber security protection for electric power system supervision and control http://www.howtoabc.com/book8/14078-0.html, https://www.secrss.com/articles/24761

[5]	IEEE C37.2-2008 IEEE Standard Electrical Power System Device Function Numbers, Acronyms, and Contact Designations.

[6]	IEC TR 61850-90-1:2010, Communication Networks and Systems for Power Utility automation – Part 90-1: Use of IEC61850 for the communication between substations.

[7]	IEEE 1588-2019 – IEEE Standard for a precision clock synchronization protocol for networked measurement and control systems.

[8]	IEEE Guide for Application of Digital Line Current Differential Relays Using Digital Communication.

[9]	61850-9-3-2016 – IEC/IEEE International Standard - Communication Networks and Systems for Power Utility automation – Part 90-3: Precision time protocol profile for power utility

[10]                     3GPP TR 22.804: "Study on Communication for Automation in Vertical domains (CAV)".

[11]	Sendin, A., et. al., “Telecommunication Networks for the Smart Grid,” Artech House, 2016.

[12]	Goel, S., S. F. Bush, and D. Bakken, (eds.), IEEE Vision for Smart Grid Communications: 2030 and Beyond, New York: IEEE, 2013.

[13]	US Department of Energy, “Communications Requirements of Smart Grid Technologies, 2010”, accessed 14.08.20, http://energy.gov/sites/prod/files/gcprod/documents/Smart_Grid_Communications_Requirements_Report_10-05-2010.pdf

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

[y]	Open PLC European Research Alliance (OPERA). https://cordis.europa.eu/project/id/026920

[z]	“Applications of ITU-T G.9960, ITU-T G.9961 transceivers for Smart Grid applications: Advanced metering infrastructure, energy management in the home and electric vehicles”, 06/2020. https://www.itu.int/dms_pub/itu-t/opb/tut/T-TUT-HOME-2010-PDF-E.pdf

Second change

## Annex A: Underground 3GPP Access (Informative)

### A.1	Description

The topic of this annex is transport networks, normally out of scope of 3GPP standardization. For this reason, the material is provided as an informative annex.

Editor’s Note:	 This Annex may be restated in a different from than a use case in future.

This use case considers how to provide 3GPP access underground. Equivalently, this use case considers providing 3GPP access anywhere in which 3GPP access cannot penetrate.) The use case considers what actions to take from the perspective of a facility – to provide smart energy services to components that are located underground.

This is a relevant use case for smart energy because there are many facilities in the energy system which are either underground or inside structures that have thick walls with little or no ability for transmissions to penetrate.

There are essentially three possible approaches – as shown in Figure A.1-1, below.

Figure A.1-1: Underground Access Approaches

The radio access on the roof can be extended inside by means of a cable as shown in pink (e.g. coaxial cable, with support hardware) to provide radio access to an indoor station (on the lower right, underground.)

Another approach is to provide a gateway and a wireless extension topology, as shown in green. This could provide backhaul to a 3GPP access station in the lower right. Alternatively, all access within the building and underground could be accomplished by means of non-3GPP access. The (fixed mobile) 3GPP access remains on the roof, as a gateway providing network access.

The third approach is to make use of the existing power line deployment in the building and underground as a means of providing backhaul to the underground access point. There are a number of Power Line Communications (PLC) standards that are already used to provide data access to the edge of the electrical network, especially to physical locations that cannot be accessed easily in other ways.

Two prominent examples are Open PLC European Research Alliance (OPERA) [y] and G.9960 [z], which are used for support of Ethernet functionality over medium and low voltage power lines in a restricted range. In OPERA, information is modulated using OFDM with a set of 1536 sub-carriers. Effectively, this network provides an Ethernet over a power-line. G.9960 achieves PHY data rates up to 20 Mbit/s and highly robust communications with PHY bit rates up to 5 Mbit/s (using 4x repetition encoding.) 3GPP compenents can be integrated above this layer two standard that carries Ethernet frames.

OPERA achieves PHY data rates of up to 200 Mbps with a bandwidth of 30 MHz (this is typically not achieved in practice. Implementation is done in either 5 MHz or 10 MHz channels with much lower maximum PHY bit rate up to 40 Mbps and 80 Mbps respectively), for medium voltage (MV) cables. These throughput levels are not achievable at all in low voltage (LV) scenarios in which medium conditions are different to MV’s. (This is an area of current development at the time this document was written, with researchers currently performing tests to evaluate the LV scenario.) The evolution of Ghn defined in ITU standard G.9960 might be more adequate for LV broadband power line (BPL) and for this use case (use of the LV cabling within a building) because it was specifically defined for domestic cables.

Whereas adding a cable for range extension through a structure or wireless extension topologies entail extra planning, materials and complexity, power lines already exist in all (modern) structures.

### A.2	Pre-conditions

A facility will include components of the smart grid. The facility may either exist, or be planned and built from scratch.

The builders provide an efficient and effective means to extend wireless communication for smart grid communications underground (or in a portion of the facility which cannot be penetrated by emissions for wireless communications.)

Figure 5.x.2-1 depicts an example deployment architecture.

Figure A.2-1: Example Underground 3GPP Access Deployment using PLC

A PLC connection between the UE and the underground gNB has been configured. The UE has a preconfigured tunnel to the MNO’s network. The UE’s user plane tunnel transports both the user plane and control plane traffic of the underground gNB. The UEs that camp on the underground gNB have full 5G service, limited only by the (limited) bandwidth available (and added latency) due to the PLC and UE to gNB links.

### A.3	Service Flows

The power lines in the facility are checked for their suitability for transmission of communications data. Power line communication termination is placed in the radio access station in the roof and the 3GPP access indoor station underground. Communications through the facility enable communication to be performed by the indoor station, then using the backhaul provided by the PLC, to communication via the access station on the roof.

### A.4	Post-conditions

The facility has underground 3GPP communication without need for additional wires (for a coverage repeater) or an indoor wireless extension topology. While the performance of the underground access will be limited to the capabilities of the PLC performance, this suffices for the smart energy services operating in the underground location.

### A.5	Existing features partly or fully covering the use case functionality

There are multiple ways that the configuration described in 5.x.3 could be realized with existing 3GPP standards. For example, the indoor station could be a gNB whose backhaul is supported by means of PLC.

Another possibility would be for the indoor station to provide non-3GPP wireless access underground. The PLC could provide backhaul to the rooftop UE. The rooftop UE would then provide network access through a PLMN.

### A.6	Potential New Requirements needed to support the use case

This use case is included for completeness but no new requirements have been identified.

Third change

Annex B:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/comment | New version |  |
| 2020-08 | SA1#91e | S1-203164 |  |  |  | TR Skeleton agreed | 0.1.0 |  |
| 2020-08 | SA1#91e | S1-203165 |  |  |  | FS_5GSEI overview | 0.1.0 |  |
| 2020-08 | SA1#91e | S1-203166 |  |  |  | FS_5GSEI scope | 0.1.0 |  |
| 2020-08 | SA1#91e | S1-203337 |  |  |  | Use Case of Distributed Power Storage | 0.1.0 |  |
| 2020-08 | SA1#91e | S1-203338 |  |  |  | Use Case of Advanced Metering for Smart Grid | 0.1.0 |  |
| 2020-08 | SA1#91e | S1-203339 |  |  |  | Use case of Smart Metering | 0.1.0 |  |
| 2020-08 | SA1#91e | S1-203340 |  |  |  | Use Case of Distributed Automation | 0.1.0 |  |
| 2020-08 | SA1#91e | S1-203342 |  |  |  | Use case of line current differential protection in power distribution grid | 0.1.0 |  |
| 2020-08 | SA1#91e | S1-203343 |  |  |  | Smart Energy Differentiated QoS for Transported Encrypted Data | 0.1.0 |  |
| 2020-08 | SA1#91e | S1-203344 |  |  |  | Service Lifetime for Utility Communication Services Deployments | 0.1.0 |  |
| 2020-08 | SA1#91e | S1-203345 |  |  |  | Remote DSO management of connectivity for Smart Energy | 0.1.0 |  |

End of changes