3GPP TSG-SA WG1 Meeting #92e 	S1-204247r1

Electronic Meeting, 10 - 19 November 2020	(revision of S1-20xxxx)

Title:	New Use case: Intelligent automotive system

Agenda Item:	7.4.1

Source:	Qualcomm Incorporated

Contact:	Francesco Pica, fpica@qti.qualcomm.com

Abstract: This document proposes a new use case on intelligent vehicles utilizing AI/ML, for TR 22.874.

Use Case

# x.y AI/ML Networked Systems Emergency Response to Disabled Vehicles

## x.y.1 Use-Case Description

This use-case is illustrative for a specific example but is readily extensible for more advanced scenarios involving disaster scenarios, rescue operations or even autonomous drone operations.  The key point of this scenario is the overlapping and independent emergency response of machine-to-machine AI/ML transfer learning and inferencing systems that are networked together among multiple services and 5G networks.

The scenario assumes that multiple independent distributed and/or federated AI/ML systems are available in a high-reliability, low-latency, high-bandwidth system optimized for machine-to-machine interaction.  The proposed AI/ML systems may exchange and share model layers in a distributed and or federeated network in real-time as determined by the system in response to an emergency.  These systems may optimize the AI/ML inference latency by executing different layers on various AI/ML networked systems.

AI/ML Systems Emergency Response to Disabled Vehicle

Figure 1 - Dangerous vehicle break-down hidden by a blind curve

On-board vehicle sensors detect the following: mechanical failure, speed reduction, GPS coordinates and a variety of technical parameters used by on-board AI systems to immediately diagnose the failure and notify surrounding networks.  Networked AI/ML systems contact the appropriate road-side assistant services and activate warning failure beacons.  AI systems use the gathered failure training data to refine prediction and prevention models that is anonymously shared with the vehicle manufacturer to refine further improvements.

Traffic Surveillance Camera and MEC Metro Traffic Safety System

Surveillance camera ML algorithms use image sensor to detect and categorize the type of accident.  This processed accident information is uploaded to the local near-edge MEC for AI/ML system model prediction and algorithm processing to suggest the immediate next course of action.  These AI/ML systems suggest an automated warning broadcast.  The automated AI/ML systems rely on trained ML models to predict the position of the vehicle is especially dangerous due to the hidden blind curve and informs local Emergency Response Unit dispatch to send help to set up a traffic bypass earlier on the road.

Approaching Intelligent Vehicle Reaction

Figure 3 - Approaching Autonomous Vehicle

On an approaching autonomous vehicle, sensors receive multiple warning notifications and AI/ML systems use trained models to suggest appropriate automatic manoeuvres to move the vehicle to the far lane to avoid any danger.

Dynamic Model Changes due to Weather Conditions

Extra data from temperature, rain/snow and other weather measurements provide information to AI/ML system models to improve accuracy of emergency response predictions.

## x.y.2 Pre-conditions

The scenario described above assumes the following pre-conditions:

| Broken Autonomous Vehicle | Approaching Autonomous Vehicle | Road Surveillance Camera | Smart City Metro Traffic Control | Smart City Emergency Services Network | Weather Network |
| --- | --- | --- | --- | --- | --- |
| AI/ML Vehicle model has detected the fault. All comm. systems are available & active. | AI/ML Vehicle models are all functioning properly. All comms systems ready &active. | Camera is powered, active and connected to the network. | Software and comms are running properly. | Software, Comms and emergency assets are running properly. | Constant weather data is presented on the network. |

## x.y.3 Service Flows

Appropriate AI/ML systems shall provide security and data protection as dictated by law and other appropriate policies.

The service flows described below are all machine-to-machine AI/ML intelligent system interactions designed for the lowest-latency response for maximum safety and security.  Each level of service may be owned and operated by a different organization.

#### Vehicle to Vehicle

The stalled vehicle’s AI/ML system communicates distributed model data to the AI/ML systems of other approaching vehicles (e.g. using V2X).  Approaching vehicle receives model data and adjusts local AI/ML situational model accordingly.

#### Traffic Surveillance Camera to Metro Traffic Control

Camera senses accident and sends notification with model data to traffic control.  Traffic control may use multiple cameras to cover the same incident and re-train and redistribute reference AI model accordingly.

#### Metro Traffic Control to Metro Emergency Services Network

AI/ML models use automation and prediction to send appropriate EMS vehicles to the accident site.

#### Stalled Vehicle to Manufacturer

Stalled vehicle sends AI/ML model data to automobile manufacturer to help diagnose the problem.  Manufacturer uses the information to improve product quality, reliability and performance.  Federated and/or Distributed Learning techniques are used to improve on-board vehicle AI/ML model.

#### Stalled Vehicle to Metro Traffic Control

Stalled vehicle sends AI/ML model data to MTC to help warn others and improve MTC response models.  The specific blind curve location along with other regional data is used to improve all MTC responses.

#### Stalled Vehicle to Repair Service

Stalled vehicle sends AI/ML model data to repair service to ensure appropriate response vehicles provide proper tools and equipment.  Repair service response use Federated and/or Distributed Learning techniques for constant model improvement.

#### Approaching Vehicle communication with Metro Traffic Control

MTC warns approaching vehicle of upcoming danger and depending on the SAE level of driving automation, the vehicle responds appropriately.

#### Approaching Vehicle communication with Weather Service

Weather Service warns approaching vehicle of weather conditions and depending on the SAE level of driving automation, the vehicle responds appropriately.

## x.y.4 Post-conditions

All systems identified in this scenario use independent, networked AI/ML Distributed or Federated Learning algorithms to aggregate information from multiple sources to improve each system AI/ML model.  Independent AI/ML training systems systems ensure that improved models are distributed back to each end-system to improve the overall safety and robustness of the next response.

## x.y.5 Existing features partly or fully covering the use case functionality

[FFS]

## x.y.6 Potential New Requirements needed to support the use case

NOTE: Requirements below focus on data model download for image recognition.

Applicable use-case models used by ML-Commons for AI/ML benchmarking are:

| DNN model for image recognition | 32 bits per parameter |  | 8 bits per parameter |  |
| --- | --- | --- | --- | --- |
|  | Size of the model (MByte) | Required DL data rate (Mbps) | Size of the model (MByte) | Required DL data rate (Mbps) |
| 1.0 MobileNet-224 [10] | 16.8 | 134.4 | 4.2 | 33.6 |
| SSD-ResNet34 | 81 | 648 | 20.25 | 162 |
| SSD-MobileNet-v1 | 27.3 | 218.4 | 6.9 | 55.2 |
| MASK R-CNN | 245 | 980 | 61.25 | 245 |
| Deep Learning Recommendation Model (DLRM) | 400 | 100 | 100 | 25 |

One assumption is that all model training and inferencing is continuous and require upload/downloads to occur in real-time to ensure the most accurate predictions.

| User application | Latency requirements |  |
| --- | --- | --- |
|  | Image recognition latency | Model downloading latency |
| Vehicle Detects Fault and Pulls Over to Avoid Accident | [8ms @120FPS] | [~8ms] |
| Roadside Camera Detects Road Hazard and Warns Smart City Network | [8ms @120FPS] | [~8ms] |
| On-Coming Traffic Detects Hazard and Avoids Accident | [8ms @120FPS] | [~8ms] |
| Video recognition | [8ms @120FPS] | [~8ms] |
| Smart City Detect Accident and Issues Local Warning | [10 seconds] | [Slow] |
| Car Manufacturer/Insurance warned about fault | [10 minutes] | [Slow] |

Requirements:

[PR-001] The 5G system shall support the functionality to enable AI/ML model downloading for image recognition with latency not higher than [8 ms].

[PR-002] The 5G system shall support the functionality to enable AI/ML model downloading for image recognition with DL data rate not lower than [1 Gbps ~ 3 Gbps].

[PR-003] The 5G system shall support the functionality to enable AI/ML model downloading for image recognition with communication service availability not lower than [99.999 %].