3GPP TSG-SA WG1 Meeting #92e	S1-204264r1

Electronic Meeting, 11-20 November 2020	(revision of S1-20xxxx)

Title:	FS_Resident: Use case on seamless switching from a UE-to-UE direct communication to an indirect communication via a residential gateway

Agenda Item:	7.11.1 FS_Resident

Source:	InterDigital

Contact:	Atle Monrad <atle dot monrad at interdigital dot com>

Abstract

This document proposes a use case on “seamless switching from a UE-to-UE direct communication to an indirect communication via a residential gateway”.

Proposal

It is proposed to include the below new use case on seamless switching from a UE-to-UE direct communication to an indirect communication via a residential gateway in 3GPP TR 22.858 version 0.1.0.

* * * *   First Change   * * * *

## 5.x	Use case: seamless path switch from a UE-to-UE direct communication to an indirect communication via a residential gateway

### 5.x.1	Description

This use case is depicted in Figure 5.x.1-1. It shows the scenario where two UEs have been in direct communication using 3GPP technology, but have to switch seamlessly to an indirect communication also using 3GPP technology via a 5G-RG as the two UEs move apart from each other.

Figure 5.x.1-1. Seamless switching from residential direct UE-to-UE to a residential network path via a residential gateway

### 5.x.2	Pre-conditions

Multiple 5G indoor base stations are deployed in the home (building) for serving different floors or rooms and interconnected via the 5G-RG. The 5G-RG is connected to the 5G-CN and the base stations are connected to the 5G-CN via the 5G-RG.

### 5.x.3	Service Flows

We consider a scenario where two UEs, UE1 and UE2, are connected via a 3GPP direct communication path. Both UEs are assumed to be in proximity of one another (e.g. on the same floor) while enjoying their service via the direct link. An event occurs that triggers the move of UE1 away from UE2. While the UE1 moves away from UE2, it is the expectation of both UEs to continue their service seamlessly with the same QoS albeit through an indirect communication path via the network. Below we present the service flows:

1.	UE1 (e.g. smartphone or tablet) and UE2 (e.g. laptop) are both on the same floor (Floor 2 in this example) in the home and connected to each other via 3GPP direct communication. UE1 is receiving a direct service from UE2. This service has a specific QoS (e.g. latency sensitive).

2.	UE1 moves to another floor (Floor 1 in this example) whilst maintaining its service with UE2 with the same specific QoS.

3.	As the direct communication path between UE1 and UE2 degrades because the two UEs move apart, UE1 connects automatically to indoor BS1 while UE2 connects automatically to indoor BS2.

4.	The UE1-UE2 service is maintained with the same QoS through an indirect communication path via the 5G-RG connecting BS1 and BS2.

### 5.x.4	Post-conditions

The UE1-UE2 service is continued through the residential gateway with the same QoS as through the direct communication.

### 5.x.5	Existing features partly or fully covering the use case functionality

[3GPP TS 22.261 [2] clause 6.2.3 Service continuity: Requirements]

The 5G system shall support service continuity for a remote UE, when the remote UE changes from a direct network connection to an indirect network connection and vice-versa.

[3GPP TS 22.261 [2] clause 6.7.2: Priority, QoS, and policy control: Requirements]

The 5G system shall be able to support E2E (e.g. UE to UE) QoS for a service.

NOTE:	E2E QoS needs to consider QoS in the access networks, backhaul, core network, and network to network interconnect.

The 5G system shall be able to support QoS for applications in a Service Hosting Environment.

[3GPP TS 22.261 [2] clause 7.6.1 AR/VR: Gaming and Training Data Exchanging]

This use case is characterized by the exchange of the gaming or training service data between two 5G AR/VR devices.

### 5.x.6	Potential New Requirements needed to support the use case

[PR 5.x.6-1] The 5G system shall be able to support seamless switch of the communication path between UEs via the residential gateway.

[PR 5.x.6-1] The 5G system shall be able to provide E2E QoS control for the communication path between the UE and the 5G residential gateway via indoor 5G base station.

* * * *   End of Changes   * * *