3GPP TSG-SA WG1 Meeting #93e 	S1-210173

Electronic Meeting, 22 February – 4 March 2021	(revision of S1-21xxxx)

Title:	22.858 terminology clean-up

Agenda Item:	7.11

Source:	KPN

Contact:	Toon Norp (toon .norp@ tno. nl)

Abstract: This contribution proposes clean up of terminology to follow the agreed terms of PRAS and eRG.

## 5.1	Use case QoS for small indoor base station connectivity

### 5.1.1	Description

In many cases, residential indoor coverage will require the deployment of indoor base stations or access points. Especially for multimedia services like UHD TV, or AR/VR gaming high bitrates are needed. This in turn requires the use of higher frequency bands (e.g. 3.5 GHz or 26/28 GHz for NR or 5 GHz, 6 GHz or 60 GHz for WiFi based radio access). At these frequencies, outdoor to indoor coverage from an outdoor PLMN is challenging. Also indoor penetration throughout a house is problematic, requiring multiple Premises Radio Access Stations (PRASs).

For 5G services that require specific QoS (e.g. guaranteed bitrate, latency) or e.g. that rely on edge applications, it is important that the 5G network can differentiate the related service data flows in order to treat them accordingly. This also applies in case a PRAS is connected via an evolved Residential Gateway (eRG) and an indoor infrastructure. In the Wireline Wireless Convergence work between 3GPP and BBF, the eRG is seen as an UE. That way the eRG can request QoS for the fixed access network. Also on the radio interface between the UE and the PRAS, QoS control can work as normal. An issue however is how QoS is provided on the backhaul between the PRAS and the eRG and in the 5G-RG. It is not guaranteed that this backhaul has sufficient QoS. Also prioritization of specific service data flows may be needed to provide the expected QoS. Finally, the eRG may have to request specific QoS for the fixed access network for specific service data flows.

Note: in the use case, we generally refer to PRAS, but the similar requirements apply for e.g. a WiFi6 access point that is connected to the 5G core (non-3GPP access) via an eRG.

### 5.1.2	Pre-conditions

The following pre-conditions and assumptions apply to this use case:

-	A Premises Radio Access Station (PRAS) is deployed inside a residential home.

-	The Premises Radio Access Station (PRAS) provides access to the 5G system (e.g not local IP access)

-	UEs using the Premises Radio Access Station (PRAS) have individual subscriptions to access the 5G system.

-	The Premises Radio Access Station (PRAS) is connected to an evolved Residential Gateway (eRG)

-	5G RG is connected to the same 5G system (and has a subscription to the same 5G system) as the indoor small base station belongs to.

### 5.1.3	Service Flows

Teenagers Oliver and Scott are avid gamers. They get together in the games room in the basement of Scott's house and discuss the new game that has become available that day. Scott decides to download the new game on his laptop. The size of that game is over 100 Gbyte so even on 5G it will take a while. Good thing that Scott's parents had 5G PRASs installed in the house. 5G coverage would otherwise have been dismal in the basement and downloading would have taken forever.

While the game is downloading, Oliver and Scott decide to play their favorite multi-user AR/VR game. Each put on a VR headset, get a controller and start gaming. The AR/VR game they play is cloud based with rendering in an edge node. This way they can play the game wherever they want and still have optimal quality of experience. The low latency ensures they have no 'lag' when they move their head or when they try to shoot each other in the game. Good thing also that the 5G system makes sure that the AR/VR game is prioritized over downloading the game. That the download takes a bit more time is not a big issue, but without the usual settings for frames per second and resolution or with additional lag because of the download the game would be 'unplayable'.

### 5.1.4	Post-conditions

The game consoles have set up a 5G connection to the Premises Radio Access Station (PRAS). They have requested the same session with the same QoS, DNN, Slice, et cetera as they would have done with an outdoor basestation.

The eRG ensures that prioritization within the eRG and residential network takes requested QoS into account. This e.g. implies that the AR/VR game gets priority over the downloading of the game.

The eRG also ensures that the 5G core network can differentiate the service data flows for the AR/VR games from general Internet access, to ensure that specific handling of these service data flows (e.g. routing to an edge node) is possible.

### 5.1.5	Existing features partly or fully covering the use case functionality

3GPP TS 22.220 "Service requirements for Home Node B (HNB) and Home eNode B (HeNB)" specifies requirements for indoor basestations in a 3G (HNB) or 4G (HeNB) context. It is not very clearly specified whether these requirements also apply for 5G. Requirements are included to request resources from the IP backhaul in the fixed broadband access. Note that TS 22.220 does not assume WWC convergence. There are no requirements for the provision of QoS within the residential network itself.

3GPP TS 22.261, clause 6.26.2.1 mentions indoor small basestations connected in the context of 5GLAN: "The 5G system shall enable the network operator to provide the same 5G LAN-type service to any 5G UE, regardless of whether it is connected via public base stations, indoor small base stations connected via fixed access, or via relay UEs connected to either of these two types of base stations."

3GPP TS 22.261, clause 6.3.2.4 discusses fixed broadband access. It does not really use WWC terminology (e.g. 5G-RG), but either assumes a residential gateway that functions as a relay UE, or a residential gateway that integrates an indoor base station. There is no mentioning of a basestation that is connected to the residential gateway. 3GPP TS 23.316 is the architecture specification for Wireline Wireless Convergence.

### 5.1.6	Potential New Requirements needed to support the use case

[PR. 5.1.6-001] The 5G system shall enable the network operator to provide any 5G services to any 5G UE via a Premises Radio Access Station (PRAS) connected via an evolved Residential Gateway (eRG).

NOTE1:	The eRG may be connected via fixed access, via 5G Fixed Wireless Access, or hybrid access.

NOTE2:	The PRAS may also be co-located with the eRG

[PR. 5.1.6-002] The 5G system shall enable the evolved Residential Gateway (eRG) to provide backhaul with the required QoS for the services provided via a Premises Radio Access Station connected via the eRG.

NOTE3:	Backhaul for the PRAS includes the residential network between PRAS and eRG and the (fixed) access network between the eRG and the 5G core network.

Editor's Note: residential network to be defined as network infrastructure inside the house

## 5.2	Use case visitor access to small indoor base station

### 5.2.1	Description

In many cases, residential indoor coverage will require the deployment of Premises Radio Access Stations (PRASs). The assumption is that these PRASs are installed by the home owner. Nevertheless, it is quite possible that these PRASs can also be used by visitors to the home.

Visitor access is only possible with a few security provisions. Communication for the visitor has to be protected against eavesdropping or manipulation in the residential in home network and/or evolved residential gateway. These elements are not under control of the operator and cannot be sufficiently trusted.

From the other hand, the home owner needs to be shielded from the visitor. One aspect is that the residential home owner may not want to be charged for traffic, services, and applications initiated by the visitor. Also for lawful intercept, traffic originating from a visitor should be identifiable as such.

### 5.2.2	Pre-conditions

The following pre-conditions and assumptions apply to this use case:

-	An Premises Radio Access Station (PRAS) is deployed inside a residential home.

-	The PRAS provides access to the 5G system (e.g. not local IP access)

-	UEs using the PRAS have individual subscriptions to access the 5G system, these subscriptions may be with different operators.

-	The PRAS is connected to an evolved Residential Gateway (eRG)

-	eRG is connected to the same 5G system (and has a subscription to the same 5G system) as the PRAS belongs to.

### 5.2.3	Service Flows

Doctor Joe is a general practitioner (GP) that works from a practice next to his home in a small village. Every day patients visit his practice for all kinds of consultations.

Because of the local radio conditions (small village) and the construction of his home (concrete walls), there is very poor outdoor-to-indoor coverage in his practice. Doctor Joe has therefore installed Premises Radio Access Stations (PRASs) in his practice, both for himself and for waiting patients.

Doctor Joe does not mind that patients are using the PRAS that he has paid for. However, Doctor Joe does not want to pay for traffic, services, or applications that his patients may initiate via his PRAS and fixed access. Furthermore, he does not want to get in trouble when patients e.g. download illegal content via his fixed access. Therefore, Doctor Joe prefers that communication from/for his patients is identified as such.

Patient Mary is waiting in the waiting room. She wants to communicate in private with her mother. Even though she is using a PRAS installed by Doctor Joe, she can trust that her communication remains private.

### 5.2.4	Post-conditions

Communication for each patients is identified and billed separately from the traffic from Doctor Joe and other patients.

Communication for patients cannot be eavesdropped or manipulated.

### 5.2.5	Existing features partly or fully covering the use case functionality

3GPP TS 22.220 "Service requirements for Home Node B (HNB) and Home eNode B (HeNB)" specifies requirements for indoor basestations in a 3G (HNB) or 4G (HeNB) context. It is not very clearly specified whether these requirements also apply for 5G. TS 22.220 also includes requirements on security and privacy.

3GPP TS 22.261, clause 6.26.2.1 mentions indoor small basestations in the context of 5GLAN: "The 5G system shall enable the network operator to provide the same 5G LAN-type service to any 5G UE, regardless of whether it is connected via public base stations, indoor small base stations connected via fixed access, or via relay UEs connected to either of these two types of base stations."

3GPP TS 22.261, clause 6.3.2.4 discusses fixed broadband access and the use of a home base station. It mentions: "The 5G system shall support use of a home base station that supports multiple access types (e.g. 5G RAT, WLAN access, fixed broadband access)."

### 5.2.6	Potential New Requirements needed to support the use case

[PR. 5.2.6-001] The 5G system shall enable the network operator to provide any 5G services via a Premises Radio Access Station (PRAS) to any 5G UE with a valid subscription to the HPLMN associated with the PRAS.

[PR. 5.2.6-002] The 5G system shall enable the network operator to provide any 5G services via a Premises Radio Access Station (PRAS) to any 5G UE with a valid subscription to any VPLMN that has a roaming agreement with the HPLMN.

NOTE:	Whether 5G UEs from VPLMNs in the same country as the HPLMN can use the PRAS is subject to regulatory policy on national roaming.

[PR. 5.2.6-003] The 5G system shall ensure the use of a Premises Radio Access Station (PRAS) does not compromise the security of any PLMN or broadband access network.

[PR. 5.2.6-004] The 5G system shall ensure the use of a Premises Radio Access Station (PRAS) does not compromise the security of the UE. The PRAS (and its associated backhaul connectivity) shall provide a level of security equivalent to regular 5G base stations.

[PR. 5.2.6-005] The 5G system shall enable the network operator associated with the Premises Radio Access Station (PRAS) to control the security policy of the PRAS.

[PR. 5.2.6-006] The 5G system shall ensure the Premises Radio Access Station (PRAS) does not compromise user privacy for UEs that are using the PRAS, including communication confidentiality, location privacy and identity protection.

[PR. 5.2.6-007] The 5G system shall be able to generate charging information that can differentiate between UEs connected to the Premises Radio Access Station (PRAS) and between backhaul for the PRAS and other data traffic over the same access.

Editor's Note: it is FFS whether some functionality is needed for the owner of the PRAS to restrict usage of the PRAS by others

## 5.3	Use case of QoS maintenance from outdoor to indoor

### 5.3.1	Description

UEs can get all way QoS support from the 5G system when accessing the network with 3GPP RATs outdoors. However, due to the issue of 5G outside-to-inside coverage, UEs will probably access to 5GC through an evolved residential gateway (eRG) when moving from outdoor to indoor. If the all way QoS control can still be guaranteed at this moment, especially when high bandwidth or low latency consuming service is ongoing (e.g. video gaming), users might have a better service experience.

### 5.3.2	Pre-conditions

Tom has a mobile phone which is registered to MNO1. Tom loves to play video game at his leisure time. He even made a specific QoS setting with MNO1 to guarantee the high quality of the gaming service.

There is an eRG at Tom’s house. The eRG also subscribes to the same MNO as Tom’s mobile phone. Devices in Tom’s house can automatically be connected to the eRG via non-3GPP (R)ATs (e.g. WIFI).

### 5.3.3	Service Flows

(1)	Tom is on his way home by bus. It is so boring, so he starts playing video game on his mobile phone.

(2)	An hour later Tom arrives home. The video game is so exciting and fierce, so Tom is still obsessed with it. He keeps playing game when he enters the house.

(3)	Once Tom enters the house, his mobile phone is automatically connected to the eRG. So that his mobile phone can still access to the network.

(4)	The eRG identifies Tom’s mobile phone.

(5)	The eRG is informed of the relevant QoS characteristics of Tom’s mobile phone at this moment.

(6)	The 5G network and the eRG provide corresponding QoS control for Tom’s mobile phone.

### 5.3.4	Post-conditions

Even if Tom moves from outdoor to indoor while playing UHD video game, the quality of the video game he’s playing can be well maintained. Tom can still have a wonderful service experience.

### 5.3.5	Existing features partly or fully covering the use case functionality

3GPP TS 22.261 [2], clause 6.7.2 mentions the support of harmonised QoS framework to multiple accesses :"The 5G system shall be able to support a harmonised QoS and policy framework applicable to multiple accesses. "

3GPP TS 22.261 [2], clause 6.26.2.8 mentions QoS requirement between remote UE and relay UE: "The 5G network shall be able to provide a remote UE using 5G LAN-type service with same level of service as if the remote UE would be using a direct network connection (i.e. provide required QoS for the Ethernet packets transferred between remote UE and relay UE if they are using 3GPP access)."

3GPP TS 22.261 [2], clause 8.5 mentions different identifier association: "The HPLMN shall be able to associate a temporary identifier to a UE’s subscriber identity."But it doesn’t specify whether other entity (e.g., evolved residential gateway) can also perform the identity association.

### 5.3.6	Potential New Requirements needed to support the use case

[PR. 5.3.6-001] The 5G system shall support a mechanism for an evolved residential gateway to identify a UE connected to it via non-3GPP (R)At and associate the UE to its 3GPP identifier.

Editor’s Note: this requirement may need to be further clarified.

[PR. 5.3.6-002] The 5G system shall be able to indicate to the evolved residential gateway that a specific UE connected to it either via 3GPP RAT or non-3GPP (R)AT needs specific QoS and what the required QoS characteristics are.

## 5.4	Use case on efficient routing between UE and non-3GPP device

### 5.4.1	Description

This use case assumes that Premises Radio Access Stations were already deployed in individual rooms behind the Evolved Residential Gateway, to provide better cellular coverage at home. This use case is to enable efficient routing for the communications between UE and non-3GPP device via the Evolved Residential Gateway.

Figure 5.4.1-1: efficient routing between UE and non-3GPP device

Editor’s Note: The figure needs to be updated

### 5.4.2	Pre-Conditions

The following pre-conditions and assumptions apply to this use case:

-	Multiple Premises Radio Access Stations were deployed in individual rooms insuide a residential home

-	The Premises Radio Access Station provides cellular access to UEs

-	The Premises Radio Access Station is connected to an Evolved Residential Gateway via wireline or wireless link

In particular, a Premises Radio Access Station is deployed in the living room.

There is a smart TV in the living room, which is a non-3GPP device, connecting to the Evolved Residential Gateway via wireline.

### 5.4.3	Service Flows

Alice is sitting in the living room. Her smartphone is connecting to the Premises Radio Access Station.

Alice finds an interesting video in her smartphone and want to project to the smart TV in the living room.

The request sent from the smartphone reaches the Evolved Residential Gateway via the Premises Radio Access Station and then routed by the residential gateway to the smart TV via the wireline.

### 5.4.4	Post-Conditions

Alice could enjoy the video on her smart TV.

### 5.4.5	Existing features partly or fully covering the use case functionality

3GPP TS 22.220 "Service requirements for Home Node B (HNB) and Home eNode B (HeNB)" specifies the requirements for local IP Access for 3G and 4G.

3GPP TS 22.261 specifies the requirements for routing efficiency, e.g., private communication for 5G LAN-type service, however, the efficient routing on Evolved Residential Gateway is not covered yet.

### 5.4.6	Potential New Requirements needed to support the use case

[PR. 5.4.6-001]	The 5G system shall support routing efficiency for data traffic between a UE and a non-3GPP device through an Evolved Residential Gateway.

Editor’s Note: This requirement can be merged during potential requirement consolidation.

## 5.5	Use Case on efficient routing for UE-to-UE communications via residential gateway

### 5.5.1	Description

This use case assumes that multiple Premises Radio Access Stations were already deployed in individual rooms behind an Evolved Residential Gateway, in order to provide better coverage at home. This use case is to enable efficient routing for the communications between two UEs via the Evolved Residential Gateway.

Figure 5.5.1-1: efficient routing for UE-to-UE communications via residential gateway

Editor’s Note: The figure needs to be updated.

### 5.5.2	Pre-Conditions

The following pre-conditions and assumptions apply to this use case:

-	Multiple Premises Radio Access Stations were deployed in individual rooms inside a residential home.

-	The Premises Radio Access Station provides cellular access to UEs

-	The Premises Radio Access Station is connected to an Evolved Residential Gateway via wireline or wireless link

-	The Evolved Residential Gateway is connected to the same 5G system (and has a subscription to the same 5G system) as the Premises Radio Access Station belongs to.

In particular, one Premises Radio Access Station is deployed at the attic and another one is deployed at the basement. The connections between the Premises Radio Access Stations and the Evolved Residential Gateway could have multiple options, e.g., fiber, WiFi, or 3GPP licensed spectrum.

A security sensor is deployed at the basement which is capable of detecting smoke or fire. The sensor having a SIM is a 3GPP-capable UE which is connecting to the Premises Radio Access Station in the basement.

### 5.5.3	Service Flows

Alice is working alone at home, staying in her office in the attic. Her smartphone is connecting to the Premises Radio Access Station deployed in the attic.

Suddenly there is a fire in the basement.

The security sensor detects the fire and smoke, and then immediately sends an alarm to Alice’s smartphone.

The alarm message first reaches the Evolved Residential Gateway via the Premises Radio Access Station in the basement, and then routed by the Evolved Residential Gateway to Alice smartphone via the Premises Radio Access Station in the attic.

### 5.5.4	Post-Conditions

Alice reads the alarm message from her smartphone. After double-check the situations in the basement, Alice makes the call to the fire policy.

### 5.5.5	Existing features partly or fully covering the use case functionality

The 5G system shall support on-demand establishment of UE to UE, multicast, and broadcast private communication between members UEs of the same 5G LAN-VN. Multiple types of data communication shall be supported, at least IP and Ethernet.

3GPP TS 22.261 [2] specifies the requirement to support E2E QoS for a service, however, the existing requirement only consider QoS in the access networks, backhaul, core network and network to network interconnect. The QoS between UE, PRAS and eRG is not covered yet.

3GPP TS 22.261 [2] specifies the requirements for routing efficiency within the 5GC, e.g., private communication for 5G LAN-type service. However, the efficient routing on eRG is not covered yet.

### 5.5.6	Potential New Requirements needed to support the use case

[PR. 5.5.6-001] The 5G system shall be able to provide QoS control for the communication path between a UE and an Evolved Residential Gateway via a Premises Radio Access Station.

[PR. 5.5.6-002] The 5G system shall support routing efficiency for data traffic between two UEs through an Evolved Residential Gateway.

Editor’s Note: This requirement can be merged during potential requirement consolidation.

[PR. 5.5.6-003] The 5G system shall support a mechanism to minimize the security impact on any PLMN or broadband access network when using an Evolved Residential Gateway.

[PR. 5.5.6-004] The 5G system shall support a mechanism to minimize the security impact on the UE when using an Evolved Residential Gateway.

[PR. 5.5.6-005] The 5G system shall enable the network operator associated with an Evolved Residential Gateway to control the security policy of the an Evolved Residential Gateway.

[PR 5.5.6-006] The 5G system shall ensure an Evolved Residential Gateway does not compromise user privacy for UEs that are using the Evolved Residential Gateway, including communication confidentiality, location privacy and identity protection.

## 5.6	Use case on E2E QoS monitoring

### 5.6.1	Description

This use case assumes that multiple Premises Radio Access Stations were already deployed in individual rooms behind Evolved Residential Gateway, in order to provide better cellular coverage at home. This use case is to enable E2E QoS monitoring for the whole communication path, i.e., from/to a UE to/from the 5GC via a Premises Radio Access Station and a Premises Radio Access Station.

Figure 5.6.1-1. E2E QoS monitoring

Editor’s Note: The figure needs to be updated by using correct terminologies.

### 5.6.2	Pre-Conditions

The following pre-conditions and assumptions apply to this use case:

- Multiple Premises Radio Access Stations were deployed in individual rooms inside a residential home.

- The Premises Radio Access Station provides cellular access to UEs

-	The Premises Radio Access Station is connected to a Evolved Residential Gateway via wireline or wireless link

-	The Premises Radio Access Station is connected to the same 5G system (and has a subscription to the same 5G system) as the Premises Radio Access Station belongs to.

In particular, an Premises Radio Access Station is deployed in the living room.

### 5.6.3	Service Flows

Alice sits in the living room and her smartphone connects to the Premises Radio Access Station.

Alice is watching an interesting video on her smartphone. The smartphone downloads the video content in real time from the video application server in the cloud.

After a while, the download speed becomes slow and the video is stuck.

Alice makes a call to the network operator and reports the downgrade of the network performance.

The network operator checks the E2E QoS status and identifies there is heavy interference on the Premises Radio Access Station. The network operator reconfigures the Premises Radio Access Station and recovers the E2E QoS.

### 5.6.4	Post-Conditions

The download speed recovers normal and Alice could continue enjoy the video.

### 5.6.5	Existing features partly or fully covering the use case functionality

3GPP TS 22.261 [2] Section 6.23.2 "QoS monitoring”.

The 5G system shall be able to provide information that identifies the type and the location of a communication error for the Premises Radio Access Station (e.g. the ID of this Premises Radio Access Station).

### 5.7.6	Potential New Requirements needed to support the use case

[PR. 5.7.6-001] The 5G system shall provide a mechanism for supporting real time E2E QoS monitoring for the data traffic path (i.e., from/to a UE to/from the 5GC via a Premises Radio Access Station and a Evolved Residential Gateway).

Editor’s Note: Whether this requirement is already covered by existing requirements needs further studied.

## 5.10	Use case: seamless path switch from a UE-to-UE direct communication to an indirect communication via an evolved residential gateway

### 5.10.1	Description

This use case is depicted in Figure 5.10.1-1. It shows the scenario where two UEs have been in direct communication via a first path (Path #1) using 3GPP technology, but as the two UEs move apart from each other, they have to switch seamlessly to an indirect communication via any of the paths (Path #2, Path #3, Path #4) going through an eRG also using 3GPP technology.

Figure 5.10.1-1. Seamless switching from a direct UE-to-UE path to an indirect path going through an evolved residential gateway.

### 5.10.2	Pre-conditions

Multiple Premises Radio Access Stations (PRASs) are deployed in the home (building) for serving different floors or rooms and interconnected via the eRG. The eRG is connected to the 5G-CN and the PRASs are connected to the 5G-CN via the eRG.

### 5.10.3	Service Flows

We consider a scenario where two UEs, UE1 and UE2, are connected via a 3GPP direct communication path (Path #1 in this example). Both UEs are assumed to be in proximity of one another (e.g. on the same floor) while enjoying their service via the direct link. An event occurs that triggers the move of UE1 away from UE2. While the UE1 moves away from UE2, it is the expectation of both UEs to continue their service seamlessly with the same QoS albeit through an indirect communication path via the network. Below we present the service flows:

1.	UE1 (e.g. smartphone or tablet) and UE2 (e.g. laptop) are both on the same floor (Floor 2 in this example) in the home and connected to each other via 3GPP direct communication path (Path #1 in this example). UE1 is receiving a direct service from UE2. This service has a specific QoS (e.g. latency sensitive).

2.	UE1 moves to another floor (Floor 1 in this example) whilst maintaining its service with UE2 with the same specific QoS.

3.	As the direct communication path (Path #1) between UE1 and UE2 degrades because the two UEs move apart, UE1 connects automatically to PRAS 2 while UE2 connects automatically to PRAS 1.

4.	The UE1-UE2 service is maintained with the same QoS through an indirect communication path (Path #2, Path #3, Path #4) going through the eRG connecting PRAS 1 and PRAS 2 to the 5G-CN.

### 5.10.4	Post-conditions

The UE1-UE2 service is continued through indirect communication path going via the evolved residential gateway with the same QoS as through the original direct communication path.

### 5.10.5	Existing features partly or fully covering the use case functionality

3GPP TS 22.261 [2] clause 6.2.3 Service continuity: Requirements

The 5G system shall support service continuity for a remote UE, when the remote UE changes from a direct network connection to an indirect network connection and vice-versa.

3GPP TS 22.261 [2] clause 6.7.2: Priority, QoS, and policy control: Requirements

The 5G system shall be able to support E2E (e.g. UE to UE) QoS for a service.

NOTE:	E2E QoS needs to consider QoS in the access networks, backhaul, core network, and network to network interconnect.

The 5G system shall be able to support QoS for applications in a Service Hosting Environment.

3GPP TS 22.261 [2] clause 7.6.1 AR/VR: Gaming and Training Data Exchanging

This use case is characterized by the exchange of the gaming or training service data between two 5G AR/VR devices.

### 5.10.6	Potential New Requirements needed to support the use case

[PR 5.10.6-001] The 5G system shall be able to support seamless switching from a direct communication path between the UEs to an indirect communication path going through the eRG.

[PR 5.10.6-002] The 5G system shall be able to provide E2E QoS control for the communication path between the UEs and going through the eRG.

Editor’s note:	The potential new requirement [PR 5.10.6-2] must be evaluated to see if this should be moved to clause  5.10.5.

# 6	Traffic Scenarios

Editor's Note: Use cases are mainly targeted at the identification of Potential Functional Requirements, Traffic Scenarios are mainly targeted at the identification of Potential Key Performance Requirements

## 6.1	Traffic Scenario: inHome

### 6.1.1	Description

Many houses suffer from problems of coverage due to the number of floors and other obstacles (i.e. walls, doors, columns, furniture). Outdoor-to-indoor coverage may be an issue at 3.5 GHz and will certainly be difficult at mmwave frequencies. Providing coverage with indoor solutions may be the answer; Premises Radio Access Stations (PRAS) can be connected via fixed access. 3GPP together with BBF has provided Wireline Wireless Convergence architecture solutions where fixed access is considered part of the 5G System.

Most houses have only one entry network point where the  evolved Residential Gateway (eRG) will be installed. Connectivity to the eRG may be fixed access, 5G Fixed Wireless Acess, or a hybrid of both.

Figure 6.1.1-1 – InHome Scenario

Providing connectivity throughout the house from a single PRAS will not be feasible. Concrete floors and walls, will require multiple PRASs to be deployed in the house. That way it can be guaranteed that even in the attic, cellular and garage, there is sufficient coverage. At mmwave frequencies, even a PRAS per room (living room, kitchen, bed rooms, attic) may be required as higher frequencies will be blocked significantly by walls, et cetera.

### 6.1.2	Assumptions

Let’s consider a fairly large house with dimensions 10m x 10m and 3 floors. The owners of the house are a family of four; mother, father and two teen age children.

The traffic scenario for in the house is mainly determined by multimedia application usages. Most other services (e.g. mobile telephony) will not significantly contribute to the traffic load.

We assume the house has two UHD TVs, two VR headsets for gaming, and two office settings for working from home (for e.g. multimedia conferencing). Not all devices will be used at the same time. On the other hand, visitors may also bring/use devices. Assumption is that one UHD TV, two VR headsets for gaming and two office settings are used simultaneously, each at the maximum user experienced data rates of 1 Gbit/s DL and 500 Mbit/s UL. This results in an overall capacity for the house of 5 Gbit/s DL (UHD TV is DL only) and 2 Gbits/s UL.

The assumption is that different Premises Radio Access Stations (PRASs) are deployed around the house. Not all devices will use the same PRAS. Maximum per PRAS data rates are assumed to be half of that per house.

### 6.1.3	Potential Functional Requirements

None

### 6.1.4	Potential Key Performance Requirements

Values for max data rate and area capacity per home are based on values in TS 22.261 [2] for InHome scenario and indoor hotspot. The area capacity per base station is calculated based on two base stations per home (10m x 10m with 3 floors).

Table 6.1.4-1 – Potential key performance requirements for InHome scenario

| Scenario | Max. data rate (DL) | Max. data rate (UL) | End-to-end latency | Area traffic capacity (DL) (note 1) | Area traffic capacity (UL) (note 1) | Area traffic capacity (DL) (note 2) | Area traffic capacity (UL) (note 2) | Area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| InHome Scenario | 1 Gbit/s | 500 Mbit/s | 10 ms | 5 Gbit/s/ home | 2 Gbit/s /home | 2.5 Gbit/s/ PRAS | 1 Gbit/s/ PRAS | 10mx10m – 3 floors |
| NOTE 1: Area traffic capacity is determined by high bandwidth consuming devices (e.g. ultra HD TVs, VR headsets, …) assuming a family of 4 members (one UHD TV with DL only and 4 high bandwidth consuming devices with UL and DL used simultaneously). NOTE 2: Multiple Premises Radio Access Stations (PRASs) are assumed (e.g. one per room), maximum traffic per PRAS is assumed to be half of the overall traffic per house. |  |  |  |  |  |  |  |  |