3GPP TSG-SA WG1 Meeting #93-e	S1-210241

Electronic Meeting, 22 February – 4 March 2021	(revision of S1-21aaaa)

Title:	FS_PIN: Use case update:	 “inhome” – add coverage / connectivity requirements

Agenda Item:	7.12.1 FS_PIN

Source:	vivo Mobile Communications Co. LTD.

Contact:	Adrian Buckley <adrian dot buckley at vivo dot com>

Background

When you look at home automation networks today individuals can purchase new light switches, sockets, locks etc and

a)	Add them into the home network without the need for internet connectivity from the home to the internet provider;

b)	Control the elements in the home automation network when there is no internet connectivity from the home to the internet provider.

These aspects have not currently fully been captured in the “inHome” usecase.

There might be situations where specific devices within the home (e.g. a door lock) need a more reliable, stable connection e.g. its not much good if you cant get into your home because of a bad connection.  In such circumstances operator managed spectrum can be used direct device connectivity.   However such devices, because they use operators spectrum, need to be identified by the 5G network and authorised to use the operator managed spectrum.  This set of requirements has not been completely brought out in the usecase either.

In calculating availability, the following calculator was used (see below).  Given that alarm based PIN Elements can exist in a home environment and incase of illegal entry life is at hand a downtime can only be in the order of a few seconds per week. 99.999 equates to 1 second or less per day.

http://www.slatools.com/sla-uptime-calculator#:~:text=Uptime%20is%20the%20amount%20of,and%2050%20seconds%20of%20downtime.

****Changes****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 22.101: "Service principles".

[4]	Christiane Attig, Nadine Rauh, Thomas Franke, & Josef F. Krems (May 2017) "System Latency Guidelines Then and Now – is Zero Latency Really Considered Necessary?
https://www.researchgate.net/publication/317801643_System_Latency_Guidelines_Then_and_Now_-_Is_Zero_Latency_Really_Considered_Necessary/link/5ae18fc4458515c60f662370/download

[5]	3GPP TS 22.278: "Service requirements for the Evolved Packet System (EPS); Stage 1".

[6]	3GPP TR 22.858: " Study of Enhancements for Residential 5G; Stage 1".

[7]	3GPP TS 22.115: “Service aspects; Charging and billing”.

[8]	3GPP TS 22.228: "Service requirements for the IP multimedia core network subsystem"

[x]	2020 US Census
https://www.census.gov/construction/chars/highlights.html#:~:text=The%20median%20size%20of%20a,house%20was%202%2C301%20square%20feet.

[x1]	Minimum provision of electrical Socket-outlets in the home
https://www.electricalsafetyfirst.org.uk/media/1204/guidance-on-minimum-provision-socketsv2.pdf

[x2]	David Wilson Homes
https://www.dwh.co.uk/advice-and-inspiration/average-house-sizes-uk/#:~:text=Average%20UK%20house%20size%3A%20656%20sq.

[x3]	National Electrical Code 70®
https://www.nfpa.org/codes-and-standards/all-codes-and-standards/list-of-codes-and-standards/detail?code=70

****Changes****

#### 5.1.3.2.	Onboarding

When Florence first setup her PIN she had one device that was designated as a gateway UE.  She is aware of the following capabilities of the gateway (it is in the instruction manual and available by the UI). Florence also configures one PIN device to be able to adds subsequent PIN Device to the PIN by scanning in a QR or bar code from the device.

NOTE 1:	Other methods could be possible whereby Florence needs to type in information including encryption keys.  This could be by the authorised PIN device (e.g. smartphone) or a UI etc. on the gateway.

She knows that the PIN device that needs to be added has to be within communications range of the gateway device (i.e. no relay devices can be used yet) so that it can be provisioned into the PIN. Florence is aware that she can provision devices without the gateway UE being connected to the 5G network, but if the gateway UE is connected to the 5G network it will give her the option to:

a)	allow her PIN to be more secure.  In this situation the PIN Element provider has the credentials that need to be downloaded into the PIN Element with Gateway capability; and

b)	if a PIN device, if it supports the capability, to have a better user experience e.g. security equipment, door locks have guaranteed real time operation (e.g. using operator managed PIN direct connection).

NOTE 2:	b) requires credentials to be managed by Florence’s service provider.

In both a) and b) the PIN can function (continue to operate) when the PIN Element with Gateway capability has connectivity to Florence’s service provider or does not have connectivity. In the case of b) and operator managed PIN direct connection is used the PIN element has to be in coverage of Florence’s service provider. As an option if Florence’s was a business she could also have a plan that allows the door lock to still work when there is no coverage from her service provider when the PIN Element with Gateway capability has connectivity or not.

This later feature, b), requires Florence to call her service provider to activate the capability. The service provider also provides a service that allows Florence to easily move a PIN device from using one gateway to another gateway.  Service provider services are flat rate or volume charged (e.g. data size, number times operation occurs).

Florence also has the option to allow which PIN devices can interact with other PIN devices via the UI of the PIN device that acts as a gateway.

### 5.1.4	Existing features partly or fully covering the use case functionality

See sub-clause 5.5.5.

3GPP TS 22.101 clause 26A contains requirements for an operator to manage credentials that are independent of existing ones.  These requirements require the participation of the 5G system.  They are applicable to PIN Elements that use operator managed direct device connection spectrum.  However a 5G system should not necessary be involved in management of all credentials used in a PIN when a PIN Element uses non operator managed spectrum for direct device connections.

### 5.1.5	Potential Functional Requirements

[PR 5.1.5-1]	The 5G system shall support the ability for a network operator or authorised 3rd party to create a Personal IoT Network.

[PR 5.1.5-2]	A PIN shall support both delay and non-delay tolerant services. Maximum delay for non-delay tolerant services shall be 200ms [4] from the sending PIN Device to the receiving PIN Device (e.g. ask the voice assistant [sending PIN Device] to turn a light on [receiving PIN Device]).

| Use case # | Characteristic parameter |  |  |  |  |  | Influence quantity |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Communication service availability: target value [%] |  | Communication service reliability: mean time between failures |  | End-to-end latency: maximum |  | Message size [byte] |  | Transfer interval |  | Survival time |  | PIN Element speed |  | # of PIN Elements |  | Service area (note 1) |  |
| US Home | 99.999 |  | ~ 1 years |  | 200ms[4] |  | 1 k |  | TBD |  | TBD |  | stationary |  | [150] |  | 214 sq m [x] |  |
| UK Home | 99.999 | ~ 1 years | 200ms[4] | 1 k | TBD | TBD | stationary | 31 [x1] + [12] NOTE 1 | 90 sq m [x2] |
| NOTE 1 This assumes a UK house has medium rooms of: 3 bedrooms (2 double, 1 single), living room, kitchen, garage, 2 bathrooms, hallway and dining room. 31 sockets plus 12 lightbubs |  |  |  |  |  |  |  |  |  |

[PR.5.1.5-3] The PIN shall support fault tolerant operations.

[PR.5.1.5-4] A PIN shall still be able to operate when it has no connectivity to the 5G system.

[PR.5.1.5-5] It shall be possible to add a PIN Element in a PIN when the PIN has no connectivity to the 5G system.

[PR.5.1.5-6] When the PIN has temporary no connectivity to the 5G system, it shall be possible to authorise a PIN Element if it can use operator managed spectrum for PIN direct device connection:

-	when the PIN Element is in coverage; or

-	when the PIN Element is not in coverage.

[PR.5.1.5-7]	The PIN shall be able to collect stastical information regarding PIN Elements that use operator managed spectrum for PIN direct device connections and report it to the 5G system.

[PR.5.1.5-8]	A PIN Element shall be able to support various different levels of security (e.g. shall be possible to on-board a device when the PIN has no connectivity to the 5G system, when the PIN has connectivity to the 5G system and credentials are from a 3rd party or when the PIN has connectivity to the 5G system and those credentials are managed by the service provider (see 3GPP TS 22.101 [x] clause 26A).

Window, door sensors should be secure as they provide information related to the integratory of a building and the security mechanisms used on those communications links should be more secure than those probably on a light bulb,  In the case of the sensor credentials are stored by e.g. the sensor vendor whereas the light bulb ones could be on packaging or provisioned in a more an open fashion.

This is based on calculation done at this website (below).  However accurate references need to be provided.  Figure has been increased to account for lights, appliances, door bells etc

https://www.quora.com/How-many-electrical-outlets-exist-in-the-United-States-Or-how-should-I-calculate-this