3GPP TSG-SA WG1 Meeting #93e	S1-210457

Online, Feb. 23 – Mar.4 2021	revision of S1-210073

Title:	 clean up definition

Agenda Item:	7.10.1

Source:	ZTE，Xiaomi

Contact:	 xu.ling@zte.com.cn

1. Discussion

This proposal is to clean up the following definitions in the clause 3.1 of TR22.855. The reason and proposed revision are show in the follow table:

| Terminology | Change Reason | Proposed revision |
| --- | --- | --- |
| Ranging accuracy | using “distance and/or direction” instead of “relative positioning” to align the terms used in the TR. Using “absolute value of the deviation” to replace “closeness” for a quantitative definition. | It describes the absolute value of the deviation of the measured distance and/or direction between two UEs to the true distance and/or direction value. |
| Confidence level | it is suggested to describe it more clearly referring the traditional definition from statistics dictionary, e.g. https://stattrek.com/statistics/dictionary.aspx?definition=confidence_level | It describes the percentage of all the possible measured distance and/or direction that can be expected to include the true distance and/or direction considering the Ranging accuracy. |
| Availability | it is suggested to align with the definition in TS22.261 | It is a percentage value of the amount of time when a ranging system is able to provide the required Ranging-related data within the performance targets or requirements divided by the amount of time the system is expected to provide the Ranging service in a targeted service area. |
| Ranging interval | The current definition is conflict with it in the KPI table. It is suggested to refer the related definition of TS22.104 | It is a time difference between two consecutive Ranging operations. |

2. Proposal

------------------------------- Begin changes--------------------------------------------------------------------------------

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Key performance indicators and key attributes for Ranging use cases are defined as follows:

-	Ranging accuracy: describes the absolute value of the deviation of the measured distance and/or direction between two UEs to the true distance and/or direction value.

-	Confidence level: describes the percentage of all the possible measured distance and/or direction that can be expected to include the true distance and/or direction considering the Ranging accuracy.

-	Effective Ranging distance: the largest distance between the UE who initiates the Ranging and target UEs in the Ranging operation.

-	Environment of use: the physical environment between the UE who initiate the Ranging and target UEs, such as LOS environment and NLOS environment. Also the physical environments of the UE who initiate ranging, such as in coverage and out of coverage.

-	Relative UE velocity: the target UE can be either static or mobile relative to the UE who initiates the Ranging. In the latter, the attribute shall also provide some elements about its motion, e.g. maximum speed, trajectory.

-	Availability: percentage value of the amount of time when a ranging system is able to provide the required Ranging-related data within the performance targets or requirements divided by the amount of time the system is expected to provide the Ranging service in a targeted service area.

-	Latency: time elapsed between the event that triggers the determination of the Ranging-related data and the availability of the Ranging-related data at the Ranging system interface.

-	Power consumption: electrical power used by Ranging during Ranging operation.

-	Ranging interval: time difference between two consecutive Ranging operations.

------------------------------- End of changes-------------------------------------------------------------------------------