3GPP TSG-SA WG1 Meeting #92e	S1-210046

Electronic Meeting, 2 February – 4 March 2021

Title:	FS_5TRS Consolidation of potential requirements and KPIs

Agenda Item:	7.8.1

Source:	Nokia, Nokia Shanghai Bell

Contact:	Betsy Covell	betsy.covell@nokia.com

Abstract: This contribution provides the consolidated potential requirements and consolidated potential KPIs for the study.  The following changes from the content of the use cases have been made in the process of consolidation:

Consolidated potential requirements are updated based on S1-210044r1, S1-210045r1 and S1-210216r1.

Proposed New Text:

# 7	Considerations

## 7.1	Potential security considerations

[CPR 7.1-1] The 5G system shall support a mechanism for a 3rd party application to verify authorization to use a 5G timing resiliency service.

[CPR 7.1-2] The 5G system shall support a mechanism to monitor and verify the integrity of the received clock signals, where supported by the time source.

[CPR 7.1-4] The 5G system shall support a mechanism to monitor and verify the integrity of the received clock signals.

## 7.2	Potential charging considerations

[CPR 7.2-1] The 5G system shall be able to collect charging information based on the timing source (e.g., the source in use, the viability level of the source at the time of a change, start and stop of source usage).

[CPR 7.2-2] The 5G system shall be able to collect charging information per UE for use of a timing resiliency service (e.g., start/stop time and source used by a UE, timing service used by UE, holdover capability of the service).

[CPR 7.2-3] The 5G system shall be able to collect charging information on 5G system timing resiliency service (e.g., service KPIs, holdover capability, number of UEs using the service).

[CPR 7.2-4] The 5G system shall be able to collect charging information per application, including 3rd party application (e.g., timing resiliency service KPIs, holdover capability, number of UEs using the service, timing source).

# 8	Consolidated potential requirements and KPIs

## 8.1	Consolidated potential requirements

### 8.1.1	General

[CPR 8.1.1-1] The 5G system shall be able to receive accurate timing signals from one or more independent timing source(s), which can offer a timing alternative to GNSS, e.g. TBS/MBS [7] [8]], Sync over Fiber [7].

[CPR 8.1.1-2] The 5G system shall be able to maintain accurate time synchronization as appropriate for the supported applications in the event of degradation or loss of GNSS timing signals.

[CPR 8.1.1-3] The 5G system shall support a mechanism to provide timing service to UEs and applications with specific KPIs (e.g., accuracy, interval, coverage area).

[CPR 8.1.1-4] The 5G system shall support a mechanism to manage and correlate (e.g., compare, prioritize, correct) clock information from multiple time sources in the network.

[CPR 8.1.1-5] The 5G system shall support a mechanism to configure granularity of the timestamp for cases where UE/DS-TT performs the timestamping of the financial transaction.

### 8.1.2	Monitoring and Reporting

[CPR 8.1.2-1] The 5G system shall monitor for timing source failure.

[CPR 8.1.2-2] The 5G system shall be able to detect when reference timing signals (e.g., from GNSS or other timing source) are no longer viable for network time synchronization.

[CPR 8.1.2-3] The 5G system shall support a mechanism to determine the time uncertainty of the 5G time synchronization service.

[CPR 8.1.2-4] The 5G system shall be able to indicate to devices (e.g., UEs, applications) using a timing service that they need to use an alternate time source (e.g., to 5G system internal holdover capability, atomic clock, Sync over Fiber, TBS, GNSS), taking into account the holdover capability of the devices.

[CPR 8.1.2-5] The 5G system shall be able to detect when a timing source fails or is restored for network time synchronization.

[CPR 8.1.2-6] The 5G system shall support mechanisms to monitor different time sources and adopt the most appropriate.

[CPR 8.1.2.7The 5G system shall support a mechanism to report timing resiliency service information (e.g., divergence from UTC, time uncertainty) to 3rd party applications.

### 8.1.3	Service Exposure

[CPR 8.1.3-1] The 5G system shall support a mechanism for a 3rd party application to request a timing service with specific KPIs (e.g., accuracy, interval, coverage area).

### 8.1.4	Holdover

[CPR 8.1.4-1] The 5G system shall support a holdover capability (maintaining UTC required accuracy) of up to 24h.

## 8.2	Consolidated potential KPIs

Table 8.2-1: Timing resiliency service performance requirements for 5G System

| Use-case | Holdover Time | Sync Target | Sync accuracy | Service area | Mobility | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| Power grid (5G network) | Up to 24 hour | UTC (note 1) | <250ns-1000ns [3], [9] (note2) | < 20 km2 | low | When 5G System provides direct PTP Grandmaster capability to sub-stations |
| Power grid (time synchronization device) | >5 s | UTC (note 1) | <250ns-1000ns [3], [9] (note2) | < 20 km2 | Low | When 5G sync modem is integrated into PTP grandmaster solution (with 24h holdover capability) at sub-stations) |
| Note 1: A different synchronization target is acceptable as long as the offset is preconfigured when an alternatively sourced time differs from GNSS. In this case, a 5G end device shall provide PPS output which can be used for measuring the difference. Note 2: Use case [New] in [9] illustrates the different accuracy measurements based on different configurations needed to support the underlying requirements from IEC [3]. The range is between 250 ns and 1000 ns. The actual requirement depends on the specific deployment. |  |  |  |  |  |  |

Table 8.2-2: Level of accuracy for members or participants of a trading venue [13, 14]

| Type of trading activity | Maximum divergence from UTC | Granularity of the timestamp (note 1) |
| --- | --- | --- |
| Activity using high frequency algorithmic trading technique | 100µs | ≤1µs |
| Activity on voice trading systems | 1s | ≤1s |
| Activity on request for quote systems where the response requires human intervention or where the system does not allow algorithmic trading | 1s | ≤1s |
| Activity of concluding negotiated transactions | 1s | ≤1s |
| Any other trading activity | 1ms | ≤1ms |
| Note 1: Only relevant for the case where the time synchronization service assists in configuring the required granularity for the timestamp (for direct use), otherwise it will be configured separately as part of the financial transaction timestamp process. |  |  |