3GPP TSG-SA WG1 Meeting #93-e	S1-210239r0`

Electronic Meeting, 23 February – 4 March 2021	(revision of S1-21aaaaa)

Title:	FS_PIN: Definition update – PIN Element

Agenda Item:	7.12.1 FS_PIN

Source:	vivo Mobile Communications Ltd, InterDigital

Contact:	Adrian Buckley <adrian dot buckley at vivo dot com>

Abstract

This document addresses the editor note for PIN Element in definitions section.

Section 5.5 and 5.6 will be edited in different contributions.

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

direct device connection: See definition in TS 22.261 [2].

Editor’s Note: since this term only applies between two UEs, this term needs to be adapted to apply in the context of this TR, e.g. to describe a connection between two PIN elements that are not UEs.

direct network connection: See definition in TS 22.261 [2].

Editor’s Note: since this term only applies between a UE and 5G Network, this term may need to be adapted to apply in the context of this TR, e.g. in case that PIN element with Gateway capability is not a UE.

IoT device: See definition in TS 22.261 [2].

PIN Element: the basic component making up a PIN-Users Personal IoT Network.  The PIN Element maybe an IoT device, TE, MT, ME, “thing” (See sub clause 26a.1 3GPP TS 22.101 [3]) or even a complete UE.

PIN Element with Gateway Capability: can act as a gateway that provides access to and from the public operator’s network (fixed/mobile/cable) and a PIN.

NOTE:	A PIN Element can have both PIN management capability and Gateway Capability.

Editor’s Note:	The relationship with FS_RESIDENT Evolved Residential Gateway still needs FFS.

PIN Element with Management Capability: A PIN Element with PIN management Capability has capability to manage the PIN.

Personal IoT Network: one or more PIN Elements that communicate with each other.

PIN-User: The PIN-User is the person who owns the PIN with respective subscriptions at one service provider.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

PIN	Personal IoT Network

UPnP	Universal Plug and Play

DNLA	Digital Living Network Alliance

****NEXT CHANGE****

# 5	Use cases

## 5.1	Traffic Scenario: inHome

### 5.1.1	Description

Houses have many opportunities to be automated, the traditional light bulbs, power sockets, thermostats, sprinkler systems, leak detection and the new smart appliances such as ovens, washing machines, faucets etc. can communicate with each other via a Personal IoT Network (PIN). These devices, to be known as PIN Elements, can have the following characteristics:

i)	some have continuous power and others don’t;

ii)	some have strict latency requirements e.g. items that are part of an alarm system, while others provide delay tolerant data;

iii)	some need to perform an action so an individual sees an action as instantaneous (e.g. ask the voice assistance to turn the light bulb on and its appears instantaneous); and

iv)	need to perform an action but they don’t have strict latency requirements e.g. sprinklers need to come on but it doesn’t matter if its few seconds late.

The PIN Elements listed above and many others that are not listed above can communicate with hubs / gateways within the PIN where some control can take place at the gateway (e.g. thermostats and sprinkler controllers are usually self-contained within the PIN Element, whereas power sockets and light bulbs might communicate with a PIN Element that manages the PIN that control their actions). In addition, thermostats etc. can also communicate with these PIN Elements that manage the network, known as PIN Element with Management Capability. Power sockets, light bulbs and switches can also act as relay type devices extending the coverage of the PIN.

NOTE 1:	A thermostat controller may have remote sensors that communicate with the thermostat. Effectively there can be a PIN within a PIN.

In case of the PIN Elementthat provides access to the 5GS, known as PIN Element with Gateway Capability (shown as a GW in Figure 5.1.1-1) these could be placed in one corner of the house.

NOTE:	The PIN Element with Gateway Capability in this diagram also manages the PIN and also is a PIN Element with Management Capability.

When planning a network in a house many houses suffer from problems of coverage due to the number of floors and other obstacles (i.e. walls, doors, columns, furniture). When the PIN Element with Management Capability connects to a broadband network (also acting as a PIN Element with Gateway Capability) most houses have only one entry network point where the PIN Element with Gateway Capability can be installed.

GW

5G

Figure 5.1.1-1: inHome scenario

Furthermore, with the use of millimetre waves there is the danger that the signal is blocked even by people while moving and alternative connection paths are needed. Therefore, enhanced "fully meshed relays" will play a key role in achieving extensive fault tolerant coverage in inHome scenarios. These fully meshed relays allow the resident of the home to position relays (PIN Elements) to create a network that is fault tolerant e.g. should a relay fail, Pin Elements such as those that detect motion or a door opening can still reach the PIN Element with Management Capabilities. Data is encrypted from the end device to the PIN Elementwith Management capabilities.

NOTE 2:	It is up to the home resident to plan their network accordingly so a single failure in the PIN does not stop a PIN Element communicating with any other PIN Element. The home resident is aware that certain types of devices can act as relays.

### 5.1.2	Pre-conditions

Let’s consider an average house can be around 100m2-120m2 with 3 floors. Regarding devices and applications within the home we can categorize them in the following groups:

a)	Lighting, appliances, sockets and climate control: This is the traditional home automation and control network where continuous power is available to each PIN Element. These PIN Elements can act as fully mesh relays due to being continually powered. Their time to change state (e.g. light bulb from off to on) needs to occur within 200ms [4].

b)	Security Systems: This includes traditional security components such as motion detector, door / window sensors, automatic lock (PIN Elements). These items are battery powered, in case of door / window sensors it can be expected that the battery should last for 2+ years. Due to nature of these PIN Elements the delay to inform the PIN Element with Management capabilities that an event has occurred needs to be 200ms [4]. Some devices will need to rely on mesh relays for routing their data.

PIN-user is aware that they need to plan their network (PIN) and walls, doors etc can present challenges to planning a network.

PIN-user is aware which PIN Elements can act as a relay and which ones cannot.

Some example dimensions of products PIN Elements are:

I	Light switch: 1.76 x 2.2 x4.1 inches (USA), 86 x 86 x 41 mm (EU);

II	Power outlet: 1.75 x 2.1 x 4.2 inches (USA), 86 x 86 x 50 mm (EU);

III	Motion sensor: 18 x 71 x 19 mm, Battery ER14250;

IV	Light bulb: 3 x 4.9 x 3 inches (USA), 6.5 × 6.5 × 14 cm (EU).

A PIN has at least one PIN Element with Management capabilities. This PIN Element contains a list of PIN Elements that are in the PIN, what each PIN Element is allowed to do (act as a relay in the PIN, end device), if the PIN Element can be communicated with and what credentials they use to access the PIN. A PIN Element may be used to provision each PIN Element in the PIN Elementthat manages the PIN (PIN Element with Management Capability).

PIN Element use direct device connections to communicate with each other.

### 5.1.3	Service Flows

#### 5.1.3.1	General

Florence (PIN-User) has decided to build a home automation network (PIN) and has purchased a number of PIN devices i.e. some light bulbs, power sockets, a gateway that acts as a voice assistant and door / window sensors (these are known as PIN Elements). The instructions on the PIN Element indicate that the PIN Element  should be no more than 100m direct line of sight from the PIN device managing the PIN however this might be affected by walls, furniture etc. To improve connectivity some PIN Element s can help extend coverage of the PIN e.g. relays (PIN Relay). The light bulbs and power sockets indicate that these devices can help extend the coverage and that the purchaser should position them around their house / condominium so that devices that cannot communicate directly with the device acting as a gateway can use multiple relays if necessary. PIN relays can also be daisy chained together to greatly improve coverage. Florence provisions her PIN Elements into the PIN Element with Management Capability. Checking her APP she sees one of the PIN Elements, a door sensors, is offline. She moves a PIN Element that acts as a relay, smart light bulb, and then opens and closes the door and sees in real time the door sensor reports the action. A while later her son turns the light switch off as he sees it in the wrong position. Later that day Florence sees the PIN Element, door sensor, offline again and finds out her son turned the light switch off. She decides to install a smart light switch near the door sensor to prevent that issue from occurring again. Florence speaks to the voice assistant that turns the light bulbs on and off in real time. The door sensor reports when the door is opened and closed with notifications on her smartphone in real-time. Later Florence install a smart door lock on the door with the door sensor. The door lock created a secure connection with the PIN.  When she activates the door to open using her app she notices it takes a second before the bolt moves.

When Florence installs a new PIN Element into the PIN the instructions indicate that the PIN Element with Management Capability has to be within 2m so that the 2 PIN Element scan successfully communicate. After that a PIN Element may use a PIN Element that extends the PIN network coverage to successfully communicate with the PIN Element with Management Capability.

#### 5.1.3.2.	Onboarding

When Florence first setup her PIN she had one device that was designated as a gateway UE.  She is aware of the following capabilities of the gateway (it is in the instruction manual and available by the UI). Florence also configures one PIN Element to be able to adds subsequent PIN Elementsto the PIN by scanning in a QR or bar code from the PIN Element.

NOTE:	Other methods could be possible whereby Florence needs to type in information including encryption keys.  This could be by the authorised PIN Element (e.g. smartphone) or a UI etc. on the gateway.

She knows that the PIN Element that needs to be added has to be within communications range of the gateway device (i.e. no relay devices can be used yet) so that it can be provisioned into the PIN. Florence is aware that she can provision devices without the gateway UE being connected to the 5G network, but if the gateway UE is connected to the 5G network it  will give her the option to:

a)	allow her PIN to be more secure; and

b)	if a PIN Element, if it supports the capability, to have a better user experience e.g. security equipment, door locks have guaranteed real time operation.

This later feature, b), requires Florence to call her service provider to activate the capability. The service provider also provides a service that allows Florence to easily move a PIN Element from using one gateway to another gateway.  Service provider services are flat rate or volume charged (e.g. data size, number times operation occurs).

Florence also has the option to allow which PIN Element can interact with other PIN Element via the UI of the PIN device that acts as a gateway.

****NEXT CHANGE****

### 5.1.5	Potential Functional Requirements

[PR 5.1.5-1]	The 5G system shall support the ability for a network operator or authorised 3rd party to create a Personal IoT Network.

[PR 5.1.5-2]	A PIN shall support both delay and non-delay tolerant services. Maximum delay for non-delay tolerant services shall be 200ms [4] from the sending PIN Elementto the receiving PIN Element (e.g. ask the voice assistant [sending PIN Element] to turn a light on [receiving PIN Element]).

Editor’s Note:	KPI table needs to be added.

[PR.5.1.5-3] The PIN shall support fault tolerant operations.

Editor’s Note:	Additional potential requirements are pending.

****NEXT CHANGE****

### 5.2.2	Pre-conditions

The PIN Element (termed for glasses, smartphone, etc. wearable devices, power point, light bulb etc.) can send out signals that can enable other PIN Elementsto measure and conduct positioning based on the measurements from the signals. The PIN user is aware that they need to position a number of PIN Elements in their room / house so that their PIN Elements that participate in AR/VR games can provide precise position into AR/VR games.

The PIN Elements use direct device connections to communicate with each other.

There is an immersive game called NEXGalaxy. In the game, the BOAT(s) are chasing and competing for limited resources on different planets in the universe. Each player has control of the speed and direction of a BOAT with a smartphone (PIN Element). Each player is also viewing the planet with the glasses (PIN Element) they are wearing. The BOAT can also be moved laterally if the controller/player walks or jumps left and right. Depending where the player stands in the room dictates where the player starts in the game e.g. which lane etc.  When the game console was setup that hosts the NEXGalaxy game the user (PIN User) had to enter the game consoles position in the room, including room rough dimensions and a rough map of major obstacles e.g. sofa, dining table etc.  This allows the game console to allow a person to move around in the room without hitting obstacles there as the game requires a lot of movement, the movement being reflected as actions in the game. E.g. in NEXGalaxy there is volcano that you can look around, the game console can setup the AR such as a dining table could be the volcano and as person walks around the dining able they walk around the volcano.  Players also jump to perform actions in the game, however as a person jumps they change their position in the room and the game display has to adapt to ensure the person does not hit a table, sofa, chair etc.

Friends Yuan and Xun each other their own smartphone (PIN Element) and a smartring (PIN Element). Yuan owns a pair of VR glasses (PIN Element). Each has configured a personal PIN:

-	Yuan’s PIN consists of the following PIN Elements: her smartphone, smartring, 2 VR glasses, a number of smart home automation devices e.g. power socket, light bulbs;

-	Xun’s PIN consists of the following PIN Elements: her smartphone and smartring.

Each PIN Element has at least an accelerometer in it.

Yuan has subscribed to a service from her service provider to provide an operator managed games service that among other things provides the ability to provide accurate absolute positioning. This operator managed game services uses the operator direct device connections capability using the operators managed spectrum.

### 5.2.3	Service Flows

Yuan and her friend Xun want to play the immersive game NEXGalaxy in Yuan’s home. They want to enjoy virtual scenarios and real interaction in Yuan home through some PIN Elementss (e.g. glasses, smartphone, and wearable devices).  Yuan scans a QR code on Xun’s smartphone (PIN Element) screen so that Yuan can allow Xun to play the game via Yuan smartphone (PIN Element).  Yuan also lets Xun borrow one of her VR glasses (PIN Element).  Xun configures her smartphone (PIN Element) to communicate with the borrowed set of VR glasses (PIN Element).

-	Yuan and Xun put on some VR glasses (PIN Element) and hold their smartphone (PIN Element) and began the games. They each control a BOAT. They start the BOAT using the smartphone (PIN Element) at the same time from the Base in MARs and compete to see who get to the Volcano first.

-	Yuan starts more quickly by controlling the BOAT using the smartphone (PIN Element).

-	Xun launches a missile targeting Yuan by using the smartphone (PIN Element) to aim at Yuan.

-	Yuan jumps right and left to avoid the missile.

-	During the time when Yuan tries to avoid the missile, Xun accelerates her BOAT through the smartphone (PIN Element) and begins to lead in the race.

-	Xun wins the game. She walks off the plane and turns her head around to check the views around the volcano. It’s really beautiful.

Yuan and Xun feel excited about the games. They continue to play the game about an hour.

### 5.2.4	Post-conditions

Yuan and Xun take off their glasses (PIN Element) and put their smartphone (PIN Element) down. They are discussing the details in the game and check the details through replay on their smartphones.

****NEXT CHANGE****

## 5.3	Media share within PINs Use case

### 5.3.1	Description

Nowadays a person has many electronic devices, such as smart phone, TV, earbuds, speaker, watch, and AR glasses (known as PIN Elements). Ideally the user should be able to choose the device he/she wants to watch the video/listen to the audio among all the devices the user has, with simple operation, without interruption the media being watched.

The media transmitted by a PIN Element could voice, video, Game audio/video and other type of data traffic. The media could be generated locally in the PIN network or from application server. Media share within PINs mean:

-	the owner of PIN could choose any device to play the media; or

-	the owner of PIN could switch from one PIN Element to anther PIN Element and the media continue to play during the switch between the PIN Elements.

### 5.3.2	Pre-conditions

The User has a smartphone (PIN Element) and at least one of smart earbuds, AR glasses, watch and TV, baby monitor speaker (PIN Elements).  The first 3 (smart earbuds, AR glasses, watch) are collectively known as wearables. The collection of all 6 is known as a Personal IoT Network (PIN). All the PIN Elements communicate wirelessly using direct device connections.

The user uses the PIN Elements for entertainment, for example, listening to music, watching videos and having WeChat video and phone calls. The earbuds will play notifications, sound and the AR glasses will display video images and notifications. The speaker will play sound.

The user uses the PIN Element to check the real-time monitoring recording (baby camera) of the baby or the house when in or outside of house.

Media (video, audio, voice and etc.) originates from within the PIN e.g. music from smartphone, video and audio from the baby monitor.

### 5.3.3	Service Flows

Whenever the user listens to music, watches movie and has WeChat video and phone calls, the media(audio/video) can be shared among all the devices belonging to the same PIN, and therefore the media can be easily switched to other PIN Elements in the same PIN without interruption to the user entertainment.

Service flow 1_watch a movie:

1.	The user starts to watch a movie on the smart phone.  The movie is coming from a streaming platform.

2.	The user finds this movie very interesting and decide to watch the movie using AR glasses for 3D audio-visual enjoyment which uses direct device connections to transfer the media from the smartphone to AR glasses.

3.	The user wears the AR glasses and choose the movie that was being watched on the smart phone.

4.	The AR glass continue to play the movie from the part that was paused on the smart phone.

5.	The user starts enjoying the movie on the AR glass with better 3D audio-visual enjoyment.

Service flow 2_listening to an audio book or music

1.	The User is doing housework with the speaker playing some audio book or music that are stored on the user’s smartphone.  The connection from the smartphone to the speaker is using direct device connection.

2.	The user decides to go running after finishing the house work.

3.	The User wears the earbuds and the audio seamlessly switched from the speaker to the earbuds that communication with the smartphone using direct device connection.

4.	The user goes out for running wearing the earbuds and watch, without carrying the smart phone.

5.	The audio continues to play while the user runs outside using the cellular network to communicate with the watch which subsequently uses direct device connection to communicate with the earbuds.

Service flow 3_ baby monitor

1.	The user has bought a baby monitor (PIN Element) in their house, the instructions indicate that other PIN Elements can be used to extend the range of the baby monitor and as well if the purchaser wants they can use multiple PIN devices to make the baby monitor solution more resilient.

2.	The baby is put for a daytime nap and the user needs to do some housework. The video from the camera that is in the house is sent to his smartwatch via a direct device connection so he can keep an eye on the baby.

3.	In a few rooms of the house he has a large TV and as he goes into a room with a TV the video then appears on the TV from the baby monitor using direct device connections to the TV so the user can get on with what he needs to do but can keep an eye on the baby, this is more convenient than looking at a small image on the smartwatch.

4.	The TV/smartwatch detects when the user cannot see the TV and stops displaying the video stream.

5.	The user enters another room with a large TV and the same thing happens, again direct device connection is used from the video camera to the TV.

6.	He then moves to the kitchen. As he is cleaning in the kitchen the baby wakes up and he hears the baby crying on the voice assistant that is in the kitchen. The voice is from the video camera and uses direct device connections to communicate with the voice assistant.

7.	The user uses the PIN Element to check the real-time monitoring recording of the baby when outside of house.

Service flow 4_Over the top application call

1.	when the user is running outside wearing his watch, that uses direct network connection, he receives an over the top application call from his friend.

2,	The user starts to talking to his friend on the over the top application using his watch and his earbud while walking back home.

3.	When the user arrives home, he got a notification that the battery of his watch is low, so he picks up his smartphone that uses a direct network connection and continues to talk to his friend on the over the top application using his smartphone and earbuds.

4.	During his over the top application call with his friend, his friend experiences no interruption at all.

NOTE:	In service flow 4 there is only one PIN however it has 2 entry points from the operator’s network, the watch and the smartphone.

### 5.3.4	Post-conditions

The users can freely switch the video and/or audio between PIN Elements used and enjoy the same media(audio/video/voice calls) without interruption.

****NEXT CHANGE****

### 5.3.6	Potential New Requirements needed to support the use case

[PR 5.3.6-1] A PIN Elementshall be able to use direct device connection or direct network connection for PIN communications.

[PR 5.3.6-2] For intra-PIN communications, a PIN Elementshall be able to transmit media to one or more PIN Elementat the same time.

[PR 5.3.6-3] A PIN Elementshall support service continuity when a PIN Elementchanges the communication path from one PIN Elementto another PIN Element.

[PR 5.3.6-4] For a PIN it shall be possible to have more than one PIN element with Gateway capabality.

Editors note:	Gateway UE term will need aligning with terminology discussions in FS_PINs and FS_RESIDENT.

Editors Note:	SA3 need to be consulted on the security aspects of having more than one gateway UE in the PIN.

****NEXT CHANGE****

### 5.4.2	Pre-conditions

Lihua has one smartphone and one smart glass. The Smartphone (PIN Element ) can connect with the Smart glasses (PIN Element) using non 3GPP RAT direct device connections capability. This is Lihua’s Personal IoT Network (PIN).

Lihua has also subscribed to her service provider for an operator managed video service, therefore, the Smartphone (PIN Element) can also connect with the Smart glassed (PIN Element) using the operator direct device connections capability using operators managed spectrum. The service provider bills based on data consumption,  time, or the operator managed resources used for the service data transmission, e.g. operators managed spectrum. The Smart glasses (PIN Element) can determine if non 3GPP RAT and or the operator managed direct device connections capabilities are congested or not. If operator managed direct device connection service is not available e.g. out of 3GPP coverage the service will not be used by a PIN Element and non 3GPP RAT direct device connection can be used.

The Smart glass (PIN Element) can transmit the video service via both the WLAN and the operator managed direct device connections capability simultaneously. The smartphone is allowed to select to transmit over non 3GPP RAT direct device connection or the non-operator managed spectrum based on some criteria, which are provided by lihua or the operator. The smartphone (PIN Element) allows Lihua to set up a set of parameters to control the video quality when her operator managed video service will be used so that she can control the usage of her operator managed direct device connections provided service.

If requested by the operator, the Smartphone (PIN Element) collects and reports to the operator network the statistics information of the non 3GPP RAT direct device connection, such as the data volume transmitted over non 3GPP RAT direct device connection or the non-operator managed spectrum, if 3GPP authentication was used, service discovery was used.

### 5.4.3	Service Flows

Lihua has one smartphone (PIN Element) and one smart glasses (PIN Element). The Smartphone (PIN Element) connects with the Smart glasses (PIN Element) using non 3GPP RAT direct device connection for transmitting video data from the smartphone (PIN Element) to the smart glasses (PIN Element). After work Lihua take the subway back home, she lives in Vienna that has great continuous coverage in the subway system. Lihua watches the video on her smart glasses (PIN Element). The video is being played from her smartphone (PIN Element). When Lihua enters the subway station where there are lots of people, the smart glasses (PIN Element) detects that the quality of non 3GPP RAT direct device connections is bad and the smart glasses (PIN devElementice) requests the smart phone (PIN Element) to switch all or some of the video packets to operator managed direct device connection service. Lihua may get a notification that this happens however it’s the start of the month and she has turned off notifications. Based on some criteria, which are provided by lihua or the operator, the smart phone (PIN Element) requests from the network for some operator managed direct device connections resource to stream the video from the smartphone (PIN Element) to smart glasses (PIN Element). While the video service is transmitted from the smart phone (PIN Element) to smart glasses (PIN Element) using the operator managed direct device connection resource and when requested by the operator, the smart phone gathers the information needed for charging and reported to the operator network. The information needed for charging includes the data transmitted for the video service over the operator managed direct device connection between the PIN Element, time, the operator managed resources used for the service data transmission, e.g. operators managed spectrum and etc.

As Lihua rides the subway, more and more passengers get on. These passengers have also subscribed to the same video service. After some time, the network has limited operator managed direct device connections service resources to give Lihua. However, the smart glasses (PIN Element) finds that some non 3GPP RAT direct device connections resources can be used. The smart phone then aggregates non 3GPP RAT with operator managed direct device connection services Lihua can receive uninterrupted viewing experience.

If requested by the operator, the Smartphone (PIN Element) collects and reports to the operator network the statistics information of the non 3GPP RAT direct device connection, such as the data volume transmitted over non 3GPP RAT direct device connection or the non-operator managed spectrum if 3GPP authentication was used, service discovery was used.

****NEXT CHANGE****

### 5.4.6	Potential New Requirements needed to support the use case

[PR 5.4.6-1] The PIN Elementcan act upon user and operator preferences to aggregate, switch or split the service between non-3GPP RAT and operator managed direct device connection services.

[PR 5.4.6-2] When operator managed direct connections are used for inter PIN UE Elementcommunications the 5G System shall be able to collect charging data, including data transmitted over the operator managed direct device connection between the PIN Element, time, the operator managed resources used for the data transmission, e.g. operators managed spectrum and etc.

[PR 5.4.6-3] When PIN Elementuses unlicensed spectrum for direct device connections for intra PIN Elementdevice communications the 5G System may, subject to local/regional regulations and user consent, collect statistics data, including if 3GPP authentication was used.

****NEXT CHANGE****

### 5.7.6	Potential New Requirements needed to support the use case

Editor’s Note:	These requirements will need adjusting based on section 5.7.5.

[PR 5.7.6-1] The 5G system shall support that a PIN Elementmay be a member of more than one Personal IoT Network.

[PR 5.7.6-2] The 5G system shall support a PIN Element being added or removed from a PIN by an authorised 3rd party.

[PR 5.7.6-3] The 5G system shall enable direct device communications between PIN Elementsin a PIN to use licensed spectrum (under the control of a MNO) or between UEs or devices to use unlicensed spectrum (may be under the control of the MNO, or not).

****NEXT CHANGE****

### 5.8.4	Post-conditions

The person can control the wireless Hi-Fi system indoor and outdoor, using both in the in-home network and via the public mobile network (PLMN).

****NEXT CHANGE****

### 5.9.5	Existing features partly or fully covering the use case functionality

From TS 22.261 [2] "Service requirements for the 5G system":

The connection between a remote UE and a relay UE shall be able to use 3GPP RAT or non-3GPP RAT and use licensed or unlicensed band.

The 5G system shall support a secure mechanism for a home operator to remotely provision the 3GPP credentials of a uniquely identifiable and verifiably secure IoT device.

- Note that the above requirement only partially covers the above mentioned uses case. 3GPP currently relies on the external GSMA remote provisioning framework to perform this function. However, the remote provisioning framework requires a (e)UICC to be supported on the UE, which is not required in this use case. This requirement may need to be further clarified or additional requirements may need to be added to cover non-UICC, non-3GPP RAT devices and the use of a gateway UE to be involved in the provisioning or communication.

From TS 22.101 [3] "Service aspects; Service principles":

The 3GPP network shall be able to provide a User Identifier for a non-3GPP device that is connected to the network via a UE that acts as a gateway.

The 3GPP network shall support to perform authentication of a User Identity used by devices that are connected via a UE that acts as a gateway.

A subscriber shall be able to link and unlink one or more user Identities with his 3GPP subscription.

The User Identifier may be provided by some entity within the operator’s network or by a 3rd party.

The 3GPP system shall support to interwork with a 3rd party network entity for authentication of the User Identity.

The 3GPP system shall support to perform authentication of a User Identity regardless of the user's access, the user's UE and its HPLMN as well as the provider of the User Identifier.

The 3GPP system shall support user authentication with User Identifiers from devices that connect via the internet; the 3GPP system shall support secure provisioning of credentials to those devices to enable them to access the network and its services according to the 3GPP subscription that has been linked with the User Identity.

NOTE: 	it is not clear whether this requirement only covers provisioning of devices that are already properly configured to have a working internet connection or not. For this use case the PIN Element may initially only be able to set up a direct device connection with the gateway UE for onboarding, but it may not have a fully working internet connection. Therefore, this requirement may require further clarification. It may also need to be clarified that the PIN Elementmay be a headless device.

### 5.9.6	Potential New Requirements needed to support the use case

The 5G system shall support access to the 5G network and its services for an authorized PIN Element (linked to a 3GPP subscription and provisioned with credentials) via a gateway UE or directly via non-3GPP access.

## 5.10	Personal health monitoring PIN Elements

### 5.10.1	Description

Personal health monitoring devices are becoming more readily available to consumers and may be used by doctors to provide remote monitoring of patients. Doctors may provide the devices to their patients and ask the patients to regularly upload measurements to the doctor’s office.

### 5.10.2	Pre-conditions

The personal health monitoring devices are PIN Elementsthat are designed to work within a patient’s home network and communicate with the doctor’s servers via the 5G network through a gateway UE. The devices belong to doctors and/or hospitals, who are the owners of the devices. The devices are configured by the doctor’s office or hospital with credentials belonging to the doctor or hospital to enable communications over the cellular network. The devices can also use direct device connections to communicate with other PIN Elementsin the network.

John has recently suffered from a heart attack and is recovering from treatments in a hospital. Before John is discharged, his cardiologist provides a heart rate monitoring device that he can use so the doctor can monitor his heart rate while doing light exercises at home. John needs to also participate in a cardiac rehabilitation program at a local clinic where nurses can monitor his heart rate when he participates in a medically supervised exercise program. He attends the clinic 3 days a week and for other days of the week, John is encouraged to exercise lightly at home. He uses the heart rate monitoring device provided by his doctor to record his heart rate while exercising at home and also when he is resting.

John also suffers from diabetes and his endocrinologist has provided him with a glucose monitoring device. The glucose monitoring device is a PIN Elementthat is configured with credentials belonging to his endocrinologist so that measurements from the device can be sent to the endocrinologist’s servers via the 5G network. John needs to measure his glucose levels before and after each meal and sometimes on-demand if requested by his endocrinologist.

When John visits his daughter’s house, he brings both the heart rate monitoring device and the glucose monitoring device so he can continue to provide measurements to both his doctors. The PIN Elementmay either communicate to the gateway UE at his daughter’s house (if one is available) or the PIN Elementmay use John’s smartphone as the gateway UE. Either way, the data traffic sent from each device is charged to the respective doctors.

### 5.10.3	Service Flows

John brings the PIN Elementsthat were provided by his doctors to his home and connects them to his home network.

-	Each PIN Elementconnects to the home network

-	A gateway UE allows the PINs within the home to connect to the 5G system

-	Each PIN Elementsends data through the gateway UE and the 5G system to the corresponding doctor’s servers

-	The 5G system is able to identify what PIN Elementssent each piece of data and generates charging records accordingly

### 5.10.4	Post-conditions

John’s doctors are able to monitor his health remotely through the measurements provided by the PIN Elements.

John doesn’t need to see his doctors as often as he recovers and if his vitals show he is relatively in good health.

The 5G system can correctly identify traffic sent by each PIN Elementand generate the appropriate charging records.

### 5.10.5	Existing features partly or fully covering the use case functionality

From 3GPP TS 22.101 [3],

The 3GPP system shall be able to provide User Identities with related User Identifiers for a user.

The User Identifier may be provided by some entity within the operator’s network or by a 3rd party.

The 3GPP network shall be able to provide a User Identifier for a non-3GPP device that is connected to the network via a UE that acts as a gateway.

From 3GPP TS 22.115 [7],

The 3GPP system shall be able to create charging data containing the User Identifier and the subscription to which it is linked for access and use of network services by a non-3GPP device that was authorized with its User Identifier linked to a subscription.

****END****

An example product https://manuals.fibaro.com/door-window-sensor-2/