3GPP TSG-SA1 Meeting #93e	S1-210249r2

Electronic Meeting, 22 February – 4 March 2021	(revision of S1-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.989 | CR | 0002 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Changes to Critical Support Applications “Inviting-a-FRMCS User to a voice communication” use case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UIC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_eFRMCS |  |  |  |  |  | Date: |  |  | 2021-02-01 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Changes to Critical Support Applications “Inviting-a-FRMCS User to a voice communication” use case to align 3GPP specifications to UIC FRMCS User Requirement Specifications v5.0.0. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Restrict the “Inviting-a-FRMCS User to a voice communication” use case to ongoing voice communications only. - Allow multiple users to be invited - Allow a private call to be translated into a group call - Notify users involved in the call that a user has joined the call - Service interworking with GSM-R is FFS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 3GPP standards not aligned with latest UIC FRMCS specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.5.1, 9.5.2, 9.5.4, 9.5.x. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************************************Change**************************************************

### 9.5.1	Introduction

In this chapter the use cases related to the function of inviting a FRMCS User to a communication are defined.

-	Invite-a-FRMCS User to a FRMCS communication

-	FRMCS User receiving an invitation to a FRMCS communication

-	FRMCS User accepting an invitation to a FRMCS communication

-	FRMCS User rejecting an invitation to a FRMCS communication

-	FRMCS User ignoring an invitation to a FRMCS communication

-	Service interworking with GSM-R

*************************************End of Change***********************************************

****************************************Change**************************************************

### 9.5.2	Use Case: Inviting-FRMCS User(s) to a voice communication

#### 9.5.2.1	Description

An entiled FRMCS User can invite other(s) FRMCS User(s) to join an ongoing voice communication.

#### 9.5.2.2	Pre-conditions

The inviting FRMCS User is part of an already established FRMCS voice communication of two or more FRMCS Users.

#### 9.5.2.3	Service flows

Inviting user

From a list of FRMCS Users provided by the FRMCS System or by entering an unlisted identity, the inviting FRMCS User selects another FRMCS User, or a pre-defined group of users, to be invited.

#### 9.5.2.4	Post-conditions

The FRMCS User (s) have been invited.

The inviting FRMCS User remains connected to the FRMCS voice communication involving two or more FRMCS Users.

#### 9.5.2.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-9.5.2-001] | If entitled to, a FRMCS User shall be able to invite other(s) FRMCS User(s) to join an ongoing voice communication. | A | 22.280 | Partially covered by 6.4.6.2 Negotiated change" provides the basic functionality (limited to ongoing group communications). |
| [R-9.5.2-002] | The FRMCS System shall provide a list of identities for the inviting FRMCS User to select from. | A | N/A | Implementation option: 22.280 6.4.6.2 Negotiated change does not make any assumption on the way and means an authorised user gets the identities to be included - i.e. this happens on application layer |
| [R-9.5.2-003] | The invitation request shall either be based on the functional identity, FRMCS User Identity, FRMCS Equipment Identity. | A | N/A | Implementation option: Keep it on application layer as otherwise, considering the sheer number of users, we would have to give an indication on how to filter that list. |

*************************************End of Change***********************************************

****************************************Change**************************************************

### 9.5.4	Use case: FRMCS User accepting an invitation to a voice communication

#### 9.5.4.1	Description

A FRMCS User can accept the invitation sent by the inviting FRMCS User.

#### 9.5.4.2	Pre-conditions

The FRMCS User has received an invitation to a voice communication which is not yet expired.

#### 9.5.4.3	Service flows

Invited FRMCS User not involved in a voice communication

The FRMCS User accepts the invitation of the voice communication.

The FRMCS User joins the ongoing voice communication. If the ongoing voice communication involves two users (user-to-user), the ongoing voice communication is translated into a multi-users voice communication.

Other FRMCS Users involved in the voice communication receive a notification that the FRMCS User has joined the communication by which the joining FRMCS User is identified to the other FRMCS Users.

Invited FRMCS User is involved in a voice communication

The FRMCS User accepts the invitation of the voice communication.

The FRMCS System prompts the invited FRMCS User about the treatment of the existing voice communication.

The FRMCS User may have the choice between some or all of these options:

- Terminate: The FRMCS User terminates the current voice communication.

- Leave the existing voice communication. If the communication consisted only of two participants, a participant leaving the communication will cause the communication to be terminated.

- Merge: The FRMCS System merges the two voice communications into one single voice communication.

The FRMCS User joins the ongoing voice communication. If the ongoing voice communication involves two users (user-to-user), the ongoing voice communication is translated into a multi-users voice communication.

Other FRMCS Users involved in the voice communication(s) are notified that the invited user has joined the communication, or the communication has been merged.

#### 9.5.4.4	Post-conditions

The invited FRMCS User is participating in the voice communication he was invited to.

The invited FRMCS User is indicated as connected to the voice communication to all FRMCS Users in the communication.

The presence status of the invited user is updated with the appropriate attributes.

#### 9.5.4.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-9.5.4-001] | The FRMCS User shall be able to accept an invitation to join a voice communication. | A | 22.280 | R-6.4.6.2-005 |
| [R-9.5.4-002] | When accepting a communication invitation, in case the FRMCS User has a voice communication, the FRMCS User shall be able to leave, terminate or merge the voice communication. If the FRMCS User decides to merge the communications both voice communications shall be merged. | A | 22.280 | 5.4.1, 5.4.2, 5.1.5 R-6.4.4-003, R-6.4.4-004 The affiliation mechanism is sufficient to mimic the desired behavior of leave and terminate Merging can be done by 22.280 6.6.1 Group regrouping" |
| [R-9.5.4-003] | When accepting a communication invitation (i.e. the FRMCS User joins the ongoing voice communication), If the ongoing voice communication involves two users (user-to-user), the ongoing voice communication is translated into a multi-users voice communication. | A | Not covered | TS 22.280 6.4.6.2 Negotiated change is limited to group communications. |
| [R-9.5.4-004] | When accepting a communication invitation, other FRMCS Users involved in the voice communication(s) are notified that the invited user has joined the communication, or the communication has been merged. | A | 22.280 | Covered by the affiliation mechanism. |

*************************************End of Change***********************************************

****************************************Change**************************************************

### 9.5.x	Service interworking with GSM-R

Editor’s note: Service interworking with GSM-R is FFS.

*************************************End of Change***********************************************