3GPP TSG-SA WG1 Meeting # 94bis-e	S1-212016r6

# Electronic Meeting, No Country,  5 July 2021 - 12 July 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.844 | CR | 0001 | rev | - | Current version: | 18.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FS_PALS Update: Chapter 5.6.6 Requirement for User Awareness |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Charter Communications, CableLabs, Futurewei Technologies, Lenovo, Motorola Mobility, Philips, Qualcomm, vivo Mobile Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_PALS |  |  |  |  |  | Date: |  |  | 2021-06-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | User attending a sport event needs to receive online user awareness to the available hosting network and 3rd party services |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new requirement for use case sport event at a venue to provide information about the available hosting network and 3rd party service with no prior subscription needed |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No user awareness of 3rd party services of hosting networks. No requirements to chapter 6 for normative consolidated requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.3, 5.6.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.6.3	Service Flows

- A user is made aware of the 3rd party service being offered via Network A’s access. This may be done either offline (e.g., in ticket, flyer, posters) or online (e.g., Internet advertisement.).

NOTE: The online user awareness should contain information that a user can read to make a decision to determine if access should be made.

- During the event service negotiation between 3rd party provider and Network A, the Network A provides configuration information to 3rd party provider.

- The 3rd party provider provides the UE with Network A’s information, configuration (e.g., slicing configuration) for the event service and the event service information, and time validity.  This is sent over any access.

- During the time validity the UE searches for Network A.

- If the UE finds a suitable cell from Network A, the UE notifies the user who can manually initiate access to Network A. The UE indicates request to access to 3rd party event service(s) access.

- Network A validates with 3rd party provider that the UE is authorized to access content for event.

- If necessary, the UE is provisioned temporary access credentials for Network A.  The UE then accesses Network A for 3rd party event service(s) with the temporary credentials.

- The UE may if capable maintain two separate registrations, one with configured credentials for access to Network A, and an independent registration using a USIM. In that case, if the UE was already registered to a PLMN when identifying the Network A, the UE may initiate access and registration to Network A with the configured credentials while maintaining the access to the registered PLMN independently.

### 5.6.6	Potential New Requirements needed to support the use case

[PR.5.6.6-1] The 5G system shall support means for a hosting network to provide a 3rd party with configuration information for automatic discovery of the network and available access to specific 3rd party services.

[PR.5.6.6-2] The 5G system shall enable a UE to be able to use the 3rd party provided configuration to discover, select and access to a hosting network, and configuration and/or credentials to access for specific services with a certain time (starting time and duration) and location validity.

[PR.5.6.6-3] The 5G system shall be able to inform a UE about available 3rd party services and hosting network the UE can use at a given point in time (starting time and duration) and location.

[PR.5.6.6-4] The 5G system shall support a secure means for a UE which may or may not have prior subscription to the hosting network to display human readable information on how to gain access to the hosting network, available 3rd party services name, and on how to gain access to services.