3GPP TSG-SA WG1 Meeting # 94bis-e	S1-212038r09

# Electronic Meeting,  5 July 2021 - 12 July 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.844 | CR | 0003 | rev | - | Current version: | 18.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating PALS Consolidated Requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_PALS |  |  |  |  |  | Date: |  |  | 2021-07-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Finalizing consolidated requirements, among those left TBD/FFS from S1-94e (S1-211483, Annex), |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new consolidated requirements to sec. 6, including new/updated grouping suggestions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete consolidated requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

========= First Change ==========

# 6	Potential Consolidated Requirements

NOTE 1: Both the home and the hosting network can be a PLMN or NPN.

NOTE 2: Only subscribers of a public network can roam into a PLMN.

Configuration of localized services in hosting network:

[CPR 001] The 5G network shall support suitable mechanisms to allow automatically establishing localized service agreements for a specific occasion (time and location) and building temporary relationship among hosting network operator and other service operators including network operators or 3rd party service providers.

[CPR 002] The 5G system shall support means for the service operator to request the hosting network via standard mechanisms to provide access to 3rd party services at a specific period of time and location. This period of time shall be flexible, so that a change in service provision can be decided at any time (e.g., to cancel or prolong local services in the locality of service delivery) based on localized services agreements.

[CPR 003] Based on localized services agreements, the 5G system shall provide suitable means to allow the service operator to request and provision various localized service requirements, including QoS, expected/maximum number of users, event information for discovery, network slicing, required IP connectivity etc, and routing policies for the application of the localized services via the hosting network.

[CPR 004] The 5G system shall support means for a hosting network to create policies and configure resources for the requested time and location for the 3rd party services based on the received request.

[CPR 005] The 5G system shall support means for a hosting network to notify the service operator of the accepted service parameters and routing policies.

[CPR 006] Subject to regulatory requirements and localized service agreements, the 5G network shall allow a home network operator to automatically negotiate policies with the hosting network for allowing the home network’s subscribers to connect at a specific occasion, e.g., time and location, for their home network services.

[CPR 007] Subject to the automatic localized services agreements between the hosting network operator and home network operator, for UE with only home network subscription and with authorization to access hosting networks the 5G system shall support:

- access to the hosting network and use home network services or selected localized services via the hosting network.

- seamless service continuity for home network services or selected localized services when moving between two hosting networks or a host network and the home network.

[CPR 008] The 5G System shall support a mechanism to enable configuration of a network that provides access to localized services such that the services can be limited in terms of their spatial extent (in terms of a particular topology, for example a single cell), as specified by a 3rd party.

[CPR 009] The 5G System shall support a mechanism to enable configuration of a network that provides access to localized services such that the services can be limited in terms of the resources or capacity available, to correspond to requirements that apply only to the locality of service delivery, as specified by a 3rd party.

[CPR 010] The 5G system shall support means for a hosting network to provide a 3rd party service provider with information for automatic discovery of the hosting network by the UEs to allow access to specific 3rd party services.

[CPR 011] The 5G system shall support secure mechanism to allow a home network to coordinate with a hosting network for a subscriber to temporarily access the hosting network (e.g. based on temporary credentitals) at a given time (start time and duration) and location.

User manual selection of localized services via hosting network:

[CPR 012] The hosting network shall allow a UE to manually select temporary localized services which are provided via local breakout at the hosting network.

NOTE 3: localized services are provided via local breakout at the hosting network based on interworking scenarios for hosting network owned/collaborative services as indicated in Annex A.

UE Configuration, Provisioning, Authentication and Authorization:

[CPR 013] Subject to localized services agreements, the 5G network shall enable a home network operator to authorize a UE for using its home network services via a hosting network for a certain period of time and/or location.

[CPR 014] The 5G network shall allow a trusted 3rd party to provide UEs with localized service policy (e.g. QoS, network slice in the hosting or home network, service restriction such as time and location) via the hosting network or the UE’s home network.

[CPR 015] The 5G system shall enable a UE to use credentials provided by the hosting network with or without coordination with the home network of the UE,  to make use of localized services via the hosting network with a certain time (including starting time and the duration) and location validity.

[CPR 016] The 5G network shall be able to allow the home network to steer its UE(s) to a hosting network with the consideration of the location, times, coverage of the hosting network and services offered by the home network and hosting network.

[[CPR TBD 1] The 5G system shall support a secure means to authenticate a user of a UE to a hosting network (e.g. through biometrics), including cases in which a UE has no subscription to the hosting network yet.

NOTE TBD: It can be assumed that a service provider deploying a hosting network has access to respective identification information about the user, e.g. through a separate registration process outside the scope of 3GPP.]

[[CPR TBD 2] The 5G system shall be able to authorize the UE of a user authenticated to a hosting network to access the hosting network and its localized services on request of the service provider.]

UE Discovery, Selection and Access

[CPR 017] The 5G system shall enable a UE to receive and use configuration provided by a 3rd party service provider  to discover and access a hosting network and localized services, including the considerations of prior service agreement with a 3rd party service provider and no prior subscription to hosting network. If the UE is able to obtain services from two networks simultaneously, it may additionally select the hosting network. If the UE cannot maintain the connection to the home network while selecting the hosting network, the selection shall only be done on request by the user, i.e., using manual selection.

[CPR 018]  The 5G system shall support secure means for a UE to select and access localized services which may be provided by a 3rd party service provider via a hosting network, independent of prior subscription to the hosting network or 3rd party service provider.

[CPR 019] The 5G system shall enable the home network to instruct a UE to automatically select a hosting network for accessing localized services when specified conditions (e.g. predefined time, location) are fulfilled.

[CPR 020] The 5G system shall be able to prevent a UE to re-access the hosting network after the UE terminated the localized services if the authorization for the localized services is no longer valid (e.g., can be based on certain conditions such as time or location of the user).

[[CPR TBD 3] The 5G system may support a secure means for a UE with no prior subscription to the hosting network to display human readable information on how to gain access to the hosting network.]

[[CPR TBD 4] The 5G system shall be able to provide access to a hosting network only for UEs in a configurable targeted area within the coverage area in a privacy sensitive manner.

NOTE TBD: the targeted area may be smaller than the coverage area of a single cell.

NOTE TBD: determining location information related to a UE is subject to regional and national regulatory requirements, and is subject to authorization (e.g. of home network operator of a UE) and user consent.]

[[CPR TBD 5]The 5G system shall enable the home network to instruct a UE to select a hosting network with certain conditions (e.g. predefined time, location) based on the request from a service provider.]

[[CPR TBD 6] The UE shall be able to select a hosting network or change to another hosting network, without any additional user consent as long as the delivered services, including localized services, are unchanged.]

Hosting network localized services and home operator services:

[CPR 021] The 5G network shall enable the home network operator to indicate to the UE what services are preferred to be used from the home network when the UE connects to a hosting network and the requested service is both available from the hosting and the home network.

[CPR 022] Based on localized service agreements, the hosting network shall be able to provide required connectivity and QoS for a UE simultaneously connected to the hosting network for localized services and its home network for home network services.

[CPR 023]A UE shall be able to connect to its home network via the hosting network, if supported by the hosting network and the home network based on localized service agreements.

Returning to Home Network
[CPR 024] The 5G system shall provide mechanisms to mitigate user plane and control plane overload caused by a high number of UEs returning from a temporary local access of a hosting network to their home network in a very short period.

[CPR 025] The 5G system shall provide mechanisms to minimize the impact on the UEs communication e.g.  to prevent user plane and control plane outages when returning to a home network with other high number of UEs in a very short period, after terminating their temporary local access to a hosting network.

Charging:

[CPR 026]The 5G system shall be able to collect charging information for the use of localized services at the hosting network and provide the charging records to UEs’ home operators based on localized service agreements and charging policies provided by the localized service providers.

Regulatory Services:

[CPR 027] A hosting network using the 5G system shall be able to support regulatory services (e.g. PWS, LI, and emergency calls), based on regional/national regulatory requirements.

Multicast/Broadcast:

[CPR 028] The operator of a hosting network shall support a mechanism allowing different local service providers and content providers to disseminate their services and content over broadcast/multicast transport. This mechanism should also provide a means to include diverse content in the same transmission, e.g., to include advertisements with other content, or to include multiple content in the same media delivered to the user.

[CPR 029] A hosting network shall provide multicast and broadcast services in an energy efficient manner to UEs receiving this service.

[CPR 030] A hosting network shall support resource efficient content delivery through multicast/broadcast.

[CPR 031] A hosting network shall support a mechanism to provide low latency signalling for efficient delivery of content to the UE.

[CPR 032] Subject to home operator policy, a hosting network shall be able to prioritize specific services for local access over home routed access, even if the same service is available in both networks.

Consolidation Summary (informative)

| CPR# | PR# |
| --- | --- |
| 001 | PR.5.3.6-1 |
| 002 | PR.5.5.6-1 |
| 003 | PR.5.5.6-2 |
| 004 | PR.5.5.6-3 |
| 005 | PR.5.5.6-4 |
| 006 | PR.5.3.6-2 |
| 007 | PR.5.3.6-5 |
| 008 | PR.5.10.6-1 |
| 009 | PR.5.10.6-2 |
| 010 | PR.5.6.6-1 |
| 011 | PR.5.8.6-1 |
| 012 | PR.5.4.6-2 |
| 013 | PR.5.3.6-3 |
| 014 | PR.5.4.6-x |
| 015 | PR.5.4.6-7 |
| 016 | PR.5.11.6-1 |
| 017 | PR.5.6.6-2 |
| 018 | PR.5.7.6-1, PR.5.4.6-8 |
| 019 | PR.5.12.6-2 |
| 020 | PR.5.15.6-4 |
| 021 | PR.5.3.6-4 |
| 022 | PR.5.4.6-3, PR.5.4.6-5 |
| 023 | PR.5.4.6-4 |
| 024 | PR.5.14.6-1 |
| 025 | PR.5.14.6-2 |
| 026 | PR.5.4.6-6 |
| 027 | PR.5.9.6-1 |
| 028 | PR.5.13.6-1 |
| 029 | PR.5.13.6-2 |
| 030 | PR.5.13.6-3 |
| 031 | PR.5.13.6-4 |
| 032 | PR.5.13.6-5 |
| TBD 1 | PR.5.15.6-2 |
| TBD 2 | PR.5.15.6-3 |
| TBD 3 | PR.5.15.6-1 |
| TBD 4 | PR.5.16.6-1 |
| TBD 5 | PR.5.17.6-1 |
| TBD 6 | PR.5.17.6-2 |

========= End of Changes ==========