3GPP TSG-SA WG1 Meeting # 94bis-e	S1-212072

# Electronic Meeting, No Country,  5 July 2021 - 12 July 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.859 | CR | 0009 | rev | - | Current version: | 18.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of potential requirements on PIN and PIN Element identity and discovery of PIN Element identity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Huawei Devices |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_PIN |  |  |  |  |  | Date: |  |  | 2021-07-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Adds/modifies requirement around PIN and PIN Element Identity and the discovery of PIN Element identity |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modifies [PR.5.1.5-10] to reflect that the PIN discovery mechanism will need to dicover the identity of the target PIN Element as well as the other conditions. - Add new PR [PR.5.1.5-11] to reflect that the 5G System shall support mechanism(s) to identify a PIN and a PIN Element. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PIN and PIN Element Identity is not incldued in the Potential Requirements and discovery does not include PIN Element identity. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--- FIRST CHANGE ---

### 5.1.5	Potential Requirements

[PR 5.1.5-1]	The 5G system shall support the ability for a network operator or authorised 3rd party to create a Personal IoT Network.

[PR 5.1.5-2]	A PIN shall support both delay and non-delay tolerant services. Maximum delay for non-delay tolerant services can be up to 200ms [4] from the sending PIN Element to the receiving PIN Element (e.g. ask the voice assistant [sending PIN Element] to turn a light on [receiving PIN Element]). Other communication KPIs are shown in Table 5.1.5-1.

Table 5.1.5-1 – KPIs for communications within a home automation PIN

| Use case # | Characteristic parameter |  |  | Influence quantity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Communication service availability: target value [%] | Communication service reliability: mean time between failures | End-to-end latency: maximum | Message size [byte] | Transfer interval | Survival time | PIN Element speed | # of PIN Elements in the service area | Service area (note 1) |
| US Home | 99.999 | TBD | 200ms[4] | TBD | TBD | TBD | stationary | [150] | 214 sq m [11] |
| UK Home | 99.999 | TBD | 200ms[4] | TBD | TBD | TBD | stationary | 31 [12] + [12] NOTE 1 | 90 sq m [13] |
| NOTE 1 This assumes a UK house has medium rooms of: 3 bedrooms (2 double, 1 single), living room, kitchen, garage, 2 bathrooms, hallway and dining room. 31 sockets plus 12 lightbubs |  |  |  |  |  |  |  |  |  |

NOTE:	The definitions for the titles of each column can be found in 3GPP TS 22.104 [15].

[PR.5.1.5-3] The PIN shall support fault tolerant operations.

[PR.5.1.5-4] The 5G system shall support mechanisms to provision a PIN to use PIN direct connection in non-operator managed spectrum when it has no connectivity to the 5G system.

[PR.5.1.5-5]	The 5G system shall support mechanisms for the PIN to collect charging information (e.g. timestamp for start and stop of communications, amount of data sent/received) regarding PIN Elements that use operator managed spectrum for PIN direct connections, and to report charging data to the 5G system.

[PR.5.1.5-6]	The 5G system shall support a PIN Element using either non operator managed credentials (e.g. provided by a third party), or credentials that are managed by a service provider (e.g. see 3GPP TS 22.101 [3] clause 26A).

[PR.5.1.5-7]	A PIN shall be able to still operate when no connectivity exists from a PIN Gateway to the 5CN and or internet.

NOTE 1:	PIN Elements can only use non operator managed spectrum for communications and authentication mechanisms that do not require 5CN and or internet connectivity i.e. PIN Elements that are managed locally.

[PR.5.1.5-8]	A PIN shall have a least one PIN Element with Management Capabilities.

[PR.5.1.5-9]	The 5G network shall be able to provide backup of management data for PIN elements with management capability based on operator’s policy and local regulations.

[PR.5.1.5-10]	A PIN Element shall efficiently support a PIN discovery mechanism where PIN Elements can discover, subject to access rights:

-	identity of other PIN elements;

-	status of other PIN Element (e.g. on/off);

-	if the topology of the PIN has changed;

-	capabilities of other PIN Elements (e.g. relay, PIN Element with management capabilities, PIN Element with gateway capabilities);

-	PIN Element with gateway capability has external IP connectivity.

NOTE 2:	external IP connectivity could be local break out or via the 5G core network.

-	connection types support by other PIN Elements (e.g. operator managed, non operator managed); and/or;

-	battery operated.

[PR.5.1.5-11] The 5G System shall support mechanism(s) to identify a PIN and a PIN Element.

--- END FIRST CHANGE ---

This is based on calculation done at this website (below). However accurate references need to be provided. Figure has been increased to account for lights, appliances, door bells etc

https://www.quora.com/How-many-electrical-outlets-exist-in-the-United-States-Or-how-should-I-calculate-this