3GPP TSG-SA WG1 Meeting #94e 	S1-211040r1

Electronic Meeting, 10 – 20 May 2021	(revision of S1-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.104 | CR | 0072 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding LPHAP requirements for Industrial IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon, Sony |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LPHAP |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Low power high accuracy positioning (LPHAP) is essential and critical for industrial IoT device. On one hand, previous works had identified the KPIs for high accuracy positioning, which means industrial IoT devices have to consume more power to achieve the required high accuracy. On the other hand, various utility sectors (e.g., power plant, chemical plant, mining, nuclear, etc.) have their operation environments labelled as Danger Zones, in which it is necessary for the IoT devices to perform very frequent positioning fix, in order to obtain the real-time location information. Since both high accuracy and frequent positioning fix would consume battery power and recharging these massive IoT devices would be a great burden for the industry operator, low power high accuracy positioning is required for industrial IoT devices in order to sustain a long lifetime without changing the battery. Hence, LPHAP requirements for industrial IoT shall be specified accordingly in SA1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Editorial changes on Clause 5.7.1 - Add the LPHAP requirements for industrial IoT device. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The energy efficiency apects for industrial IoT positioning scenarios are not addressed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7, 5.7.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the Change---

5.7	Positioning performance requirements

5.7.1	General requirements

High accuracy positioning is becoming essential for Factories of the Future. The reason for this is that tracking of mobile devices as well as mobile assets is becoming increasingly important in improving processes and increasing flexibility in industrial environments.

The 5G system shall provide positioning information for a UE that is out of coverage of the network, with accuracy of < [1 m] relative to other UEs that are in proximity and in coverage of the network.

Table 5.7.1-1 below lists typical scenarios and the corresponding high positioning requirements for horizontal and vertical accuracy, availability, heading, latency, and UE speed.

NOTE:	The column on "Corresponding Positioning Service Level in TS 22.261" maps the scenarios listed in Table 5.7.1-1 to the service levels defined in TS 22.261 [2].

Table 5.7.1-1: Positioning performance requirements

| Scenario | Horizontal accuracy | Vertical accuracy | Availability | Heading | Latency for position estimation of UE | UE speed | Corresponding Positioning Service Level in TS 22.261 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mobile control panels with safety functions (non-danger zones) | < 5 m | < 3 m | 90 % | n/a | < 5 s | n/a | Service Level 2 |
| Process automation – plant asset management | < 1 m | < 3 m | 90 % | n/a | < 2 s | < 30 km/h | Service Level 3 |
| Flexible, modular assembly area in smart factories (for tracking of tools at the work-place location) | < 1 m (relative positioning) | n/a | 99 % | n/a | 1 s | < 30 km/h | Service Level 3 |
| Augmented reality in smart factories | < 1 m | < 3 m | 99 % | < 0.17 rad | < 15 ms | < 10 km/h | Service Level 4 |
| Mobile control panels with safety functions in smart factories (within factory danger zones) | < 1 m | < 3 m | 99.9 % | < 0.54 rad | < 1 s | n/a | Service Level 4 |
| Flexible, modular assembly area in smart factories (for autonomous vehicles, only for monitoring purposes) | < 50 cm | < 3 m | 99 % | n/a | 1 s | < 30 km/h | Service Level 5 |
| Inbound logistics for manufacturing (for driving trajectories (if supported by further sensors like camera, GNSS, IMU) of indoor autonomous driving systems)) | < 30 cm (if supported by further sensors like camera, GNSS, IMU) | < 3 m | 99.9 % | n/a | 10 ms | < 30 km/h | Service Level 6 |
| Inbound logistics for manufacturing (for storage of goods) | < 20 cm | < 20 cm | 99 % | n/a | < 1 s | < 30 km/h | Service Level 7 |

5.7.2 Energy efficiency requirements for positioning

The 5G system shall support low power high accuracy positioning mechanisms that allow a battery-constrained UE to sustain a long lifetime without changing battery. Some corresponding use cases and example scenarios are captured in Annex A.7.2.

## ---End of the Change---