3GPP TSG-SA1 Meeting #94e	S1-211108r2

Electronic Meeting, 10 May - 20 May 2021	(revision of S1-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.261 | CR | 0520 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification to Services provided by 5G system with satellite access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2021-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | UE to serving satellite propagation delay and possible satellite to ground station delay are introduced to end-to-end latency which is much larger than that of terrestrial 5G system. It would be difficult or even impossible for 5G system with satellite access to provide voice service by 5G system with GEO based satellite access. In order to support voice service for 5G system with GEO based satellite access, we propose to relax E2E latency for voice services provided by 5G system with GEO based satellite access. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To relax E2E latency for voice services provided by 5G system with GEO based satellite access. And to do some editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Voice service cannot be provided by 5G system with GEO based satellite access. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Begin of Changes * * * *

7.4.2	Requirements

A 5G system providing service with satellite access shall be able to support GEO based satellite access with up to 285 ms end-to-end latency.

A 5G system with satellite access shall be able to support MEO based satellite access with up to 95 ms end-to-end latency.

A 5G system providing service with LEO based satellite access shall be able to support LEO based satellite access with up to 35 ms end-to-end latency.

NOTE 1:	 5 ms network latency is assumed and added to satellite one-way delay.

NOTE 2: E2E latency for voice services provided by 5G system with GEO based satellite access could be relaxed to maximum of [400] ms.

A 5G system shall support negotiation on quality of service taking into account latency penalty to optimise the QoE for UE.

The 5G system with satellite access shall support high uplink data rates for 5G satellite UEs.

The 5G system with satellite access shall support high downlink data rates for 5G satellite UEs.

The 5G system with satellite access shall support communication service availabilities of at least 99,99%.

Table 7.4.2-1: Performance requirements for satellite access

| Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity (DL) (note 1) | Area traffic capacity (UL) (note 1) | Overall user density | Activity factor | UE speed | UE type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pedestrian (note 2) | [1] Mbit/s | [100] kbit/s | 1,5 Mbit/s/km2 | 150 kbit/s/km2 | [100]/km2 | [1,5] % | Pedestrian | Handheld |
| Public safety | [3,5] Mbit/ss | [3,5] Mbit/s | TBD | TBD | TBD | N/A | 100 km/h | Handheld |
| Vehicular connectivity (note 3) | 50 Mbit/s | 25 Mbit/s | TBD | TBD | TBD | 50 % | Up to 250 km/h | Vehicle mounted |
| Airplanes connectivity (note 4) | 360 Mbit/s/ plane | 180 Mbit/s/ plane | TBD | TBD | TBD | N/A | Up to 1000 km/h | Airplane mounted |
| Stationary | 50 Mbit/s | 25 Mbit/s | TBD | TBD | TBD | N/A | Stationary | Building mounted |
| Narrowband IoT connectivity | [2] kbit/s | [10] kbit/s | 8 kbit/s/km2 | 40 kbit/s/km2 | [400]/km2 | [1] % | [Up to 100 km/h] | IoT |
| Note 1: Area capacity is averaged over a satellite beam. Note 2: Data rates based on Extreme long-range coverage target values in clause 6.17.2. User density based on rural area in Table 7.1-1. Note 3: Based on Table 7.1-1 Note 4: Based on an assumption of 120 users per plane 15/7.5 Mbit/s data rate and 20 % activity factor per user Note 5: All the values in this table are targeted values and not strict requirements. Note 6: Performance requirements for all the values in this table should be analyzed independently for each scenario. |  |  |  |  |  |  |  |  |

* * * * End of Changes * * * *