3GPP TSG-SA1 Meeting #94-e	S1-211169r3

# Online, 10th May 2021 - 20th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.835 | CR | 0003 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for simultaneous access to multiple slices on different PLMNs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_EASNS |  |  |  |  |  | Date: |  |  | 2021-04-30 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Proposal to resolve Editor's Note on number of PLMNs, and whether the different PLMNs are VPLMNs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed that the different PLMNs are VPLMNs, and that multiple simultaneous PLMNs are to be considered. Additional references to existing SA2 and SA3 related work are introduced. The use case service flow is updated reflecting these changes. The requirements are updated to reflect the new changes, as well as to re-structure the requirements as several bullet points. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 5.6	Use case on simultaneous access to multiple slices on different VPLMNs

### 5.6.1	Description

A UE has access to multiple network slices when on the HPLMN. When the UE is roaming and the VPLMN where the UE is currently registered only provides a subset of the network slices that the UE needs to use, the UE can connect to other VPLMNs at the same time to access the other subscribed network slices.

### 5.6.2	Pre-conditions

-	UE is subscriber of Operator Toffee

-	When on its home network, Operator Toffee, UE has access to three network slices: (A)pricot, (B)anana and (C)herry

-	UE runs different applications that require the UE to simultaneously access Apricot, Banana and Cherry slices

-	Operator Toffee has agreements with:

-	Operator Chocolate to support Apricot and Banana slices

-	Operator Fudge to support Cherry slice

-	Operator Chocolate is the preferred roaming network

-	UE moves out of Operator Toffee’s coverage area, into an area where the coverage is provided by Operator Chocolate and Operator Fudge

### 5.6.3	Service Flows

1. UE connects with Operator Chocolate, and has access to Apricot and Banana slices

2. UE requires simultaneous access to all three slices, Apricot, Banana and Cherry, due to the different applications running on the UE. However, the VPLMN the UE is registered to, Operator Chocolate, does not provide support for Cherry slice

3. UE is authorised by the HPLMN to additionally access the Cherry slice in the Operator Fudge network.

4. UE connects to Operator Fudge's network, while retaining the connection to Operator Chocolate

### 5.6.4	Post-conditions

Roaming UE is able to access all three network slices simultaneously from Operator Chocolate and Operator Fudge.

### 5.6.5	Existing features partly or fully covering the use case functionality

There are related requirements in TS 22.261 [2], clause 6.18 on "Multi-network connectivity and service delivery across operators ":

The 5G system shall enable users to obtain services from more than one network simultaneously on an on-demand basis.

For a user with a single operator subscription, the use of multiple serving networks operated by different operators shall be under the control of the home operator.

TS 23.501 clause 5.3.2.4 on "Support of a UE registered over both 3GPP and Non-3GPP access" indicates:

A UE supporting registration over both 3GPP and Non-3GPP access to two PLMNs shall be able to handle two separate registrations, including two 5G-GUTIs, one per PLMN, and two associated equivalent PLMN lists.

Also, TS 33.501 has a clause 6.3.2 on "Multiple registrations in same or different serving networks"

### 5.6.6	Potential New Requirements needed to support the use case

[PR.5.6.6-1] For UEs that have the ability to obtain service from more than one VPLMN simultaneously, the following requirements apply:

-	When a roaming UE with a single PLMN subscription requires simultaneous access to multiple network slices and the network slices are not available in a single VPLMN, the 5G system shall enable the UE to:

-	be registered to more than one VPLMN simultaneously; and

-	use network slices from more than one VPLMN simultaneously

-	The HPLMN shall be able to authorise a roaming UE with a single PLMN subscription to be registered to more than one VPLMN simultaneously in order to access network slices of those VPLMNs.

-	The HPLMN shall be able to provide a UE with permission and prioritisation information of the VPLMNs the UE is authorised to register to in order to use specific network slices.

NOTE: 	The above requirements would depend on certain UE capabilities assumptions, e.g. the ability to connect to more than one PLMN simultaneously.

* * * No more changes * * * *