3GPP TSG-SA WG1 Meeting #94e 	S1-211226r1

Electronic Meeting, 10 – 20 May 2021	(revision of S1-21xxxx)

Title:	Resident architecture pictures

Agenda Item:	7.11

Source:	KPN

Contact:	Toon .Norp @tno. nl

Abstract: This contribution proposes an annex to 22.858 with architecture descriptions. Normally, it is SA2 that is responsible for architecture definition. However, the nature of Resident (where new services are tied to new pieces of hardware) makes it relevant to describe architecture in the SA1 report for Resident.

++++++++++++++++++++++++Proposed changes (all new text)++++++++++++++++++++++++

# Annex X: Resident networks overview

## A.1	Introduction

The following provides an overview of the entities defined in this Technical Report and their relations.

## A.2	Customer premises network overview

Figure A.2-1: Customer premises network connected to 5GC

The Resident customer premises network consists of the following network elements.

-	evolved Residential Gateway (eRG).

-	Premises Radio Access System (PRAS).

-	Customer Premises Network (CPN)

-	User Equipment (UE).

-	Non-3GPP Device.

## A.3	Customer Premises Network elements

### 4.3.1	evolved Residential Gateway (eRG)

The evolved Residential Gateway is defined in clause 3.1.

### 4.3.2	Premises Radio Access System (PRAS)

The Premises Radio Access System is defined in clause 3.1.

The PRAS can use licensed spectrum, but also may use unlicensed spectrum.

The PRAS does not provide radio access to non-3GPP devices. The PRAS only provides non-3GPP radio access to UEs with 3GPP credentials.

The PRAS is part of the Customer Premises Network and therefore connected to the eRG.

The PRAS may also provide E-UTRA radio access. However, this may require clarification of the relation between PRAS and HeNB.

### 4.3.3	Customer Premises Network (CPN)

The Customer Premises Network is defined in clause 3.1.

### 4.3.4	User Equipment (UE)

The User Equipment is defined in [1].

### 4.3.3	Non-3GPP device

In the context of Resident, a non-3GPP device is a device that uses non-3GPP access technology and does not have 3GPP credentials.