3GPP TSG-SA WG1 Meeting #94e 	S1-211225

Electronic Meeting, 10 – 20 May 2021	(revision of S1-21xxxx)

Title:	RESIDENT – Consolidated requirements

Agenda Item:	7.11.1

Source:	KPN, Vivo Mobile Communications Ltd

Contact:	Toon(dot)Norp(at)tno(dot)nl

Abstract: This document proposes a list of consolidated requirements.

Discussion

Document contains consolidated requirements from the usecases.  For ease of referencing a table has been created, the table labels each consolidated requirement and provides a reference to with PR it was.  There is also a column that identifies is an equivalent CPR exists in TR 22.859.

****Changes****

# 8	Consolidated requirements

This section provides Consolidated Potential Requirements for consideration to include in the normative specifications.  The CPR’s have been grouped into different functional categories, each category contains a table that lists the original PR and any relationship to TR 22.859.

## 8.0	General

Table 8.0-1 – RESIDENT General Consolidated Requirements

|  | Potential Requirement |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential requirement No. | Potential requirement text |  |
| [CPR.8.0-1] |  | Subject to local regulations, the 5G system shall support regulatory requirements for emergency calls, PWS and eCall for UEs connected to a CPN. |  |

Editor’s Note: the consolidated requirements above are only an indication of grouping of potential requirements in the different sections and as a basis for further work on consolidation. Further consolidation of these requirements is needed. This can imply that requirements are changed, merged or not accepted as consolidated requirement.

## 8.1	Gateway

Table 8.1-1 – RESIDENT Gateway Consolidated Requirements

|  | Potential Requirement |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential requirement No. | Potential requirement text |  |
| [CPR.8.1-1] | PR 5.1.6-002 | The 5G system shall enable the evolved Residential Gateway (eRG) to provide backhaul with the required QoS for the services provided via a Premises Radio Access Station (PRAS)connected via the eRG. |  |
| [CPR.8.1-2] | PR 5.3.6-002 | The 5G system shall be able to indicate to the evolved residential gateway that a specific UE connected to it either via 3GPP RAT or non-3GPP (R)AT needs specific QoS and what the required QoS characteristics are. |  |
| [CPR.8.1-3] | PR 5.5.6-003 | The 5G system shall support a mechanism to minimize the security impact on any PLMN or broadband access network when using an Evolved Residential Gateway. |  |
| [CPR.8.1-4] | PR 5.5.6-004 | The 5G system shall support a mechanism to minimize the security impact on the UE when using an Evolved Residential Gateway. |  |
| [CPR.8.1-5] | PR 5.5.6-005 | The 5G system shall enable the network operator associated with an Evolved Residential Gateway to control the security policy of the an Evolved Residential Gateway. |  |
| [CPR.8.1-6] | PR 5.5.6-006 | The 5G system shall ensure an Evolved Residential Gateway does not compromise user privacy for UEs that are using the Evolved Residential Gateway, including communication confidentiality, location privacy and identity protection. |  |
| [CPR.8.1-7] | PR 5.13.6-001 | The 5G system shall be able to support IP traffic offload in eRG. |  |
| [CPR.8.1-8] | PR 5.19.6-001 | The 5G system shall support use of an eRG that is connected to the 5G Core Network over wireless access, fixed broadband access or hybrid access. |  |
| [CPR.8.1-9] | PR 5.17.6-001 | The PRAS and eRG shall be able to detect a loss of connection with the 5GC. |  |

Editor’s Note: the consolidated requirements above are only an indication of grouping of potential requirements in the different sections and as a basis for further work on consolidation. Further consolidation of these requirements is needed. This can imply that requirements are changed, merged or not accepted as consolidated requirement.

## 8.2	Service Discovery

Table 8.2-1– RESIDENT Service Discovery Consolidated Requirements

|  | Potential Requirement |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential requirement No. | Potential requirement text |  |
| [CPR.8.2-1] | PR 5.2.6-008 | The 5G system shall provide a mechanism to prevent or allow a (guest) UE to discover and/or use the services provided by the devices on the CPN. |  |

Editor’s Note: the consolidated requirements above are only an indication of grouping of potential requirements in the different sections and as a basis for further work on consolidation. Further consolidation of these requirements is needed. This can imply that requirements are changed, merged or not accepted as consolidated requirement.

## 8.3	Service Hosting

Table 8.3-1 – RESIDENT Service Hosting Consolidated Requirements

| CPR No. | Potential Requirement |  |  |
| --- | --- | --- | --- |
|  | Original Potential requirement No. | Potential requirement text |  |
| [CPR.8.3-1] | PR 5.11.6-001 | The 5G system shall support configurations of a Service Hosting Environment connected to the eRG. |  |
| [CPR.8.3-2] | PR 5.11.6-002 | The 5G system shall be able to maintain QoS for a UE moving from one service hosting environment to another via an eRG. |  |

Editor’s Note: the consolidated requirements above are only an indication of grouping of potential requirements in the different sections and as a basis for further work on consolidation. Further consolidation of these requirements is needed. This can imply that requirements are changed, merged or not accepted as consolidated requirement.

## 8.4	Privacy

Table 8.4-1 – RESIDENT Privacy Consolidated Requirements

| CPR No. | Potential Requirement |  |  |
| --- | --- | --- | --- |
|  | Original Potential requirement No. | Potential requirement text |  |
| [CPR.8.4-1] | PR 5.2.6-009 | The 5G system shall ensure that communications associated with individual UEs in a CPN be identifiable (e.g., subscriber identifier) in the 5G network. |  |
| [CPR.8.4-2] | PR 5.3.6-001 | The 5G system shall support a mechanism for an evolved residential gateway to identify a UE connected to it via non-3GPP (R)At and associate the UE to its 3GPP identifier. |  |

Editor’s Note: the consolidated requirements above are only an indication of grouping of potential requirements in the different sections and as a basis for further work on consolidation. Further consolidation of these requirements is needed. This can imply that requirements are changed, merged or not accepted as consolidated requirement.

## 8.5	Direct Communications

Table 8.5-1 – RESIDENT Direct Communications Consolidated Requirements

| CPR No. | Potential Requirement |  |  |
| --- | --- | --- | --- |
|  | Original Potential requirement No. | Potential requirement text |  |
| [CPR.8.5-1] | PR 5.10.6-001 | The 5G system shall be able to minimize disruption to the user when a UE switches from a direct communication path between the UEs to an indirect communication path going through the eRG and there is connectivity to the 5G system. |  |

Editor’s Note: the consolidated requirements above are only an indication of grouping of potential requirements in the different sections and as a basis for further work on consolidation. Further consolidation of these requirements is needed. This can imply that requirements are changed, merged or not accepted as consolidated requirement.

## 8.6	Connectivity - QoS - charging

Table 8.6-1 – RESIDENT Connectivity, QoS and Charging Consolidated Requirements

| CPR No. | Potential Requirement |  |  |
| --- | --- | --- | --- |
|  | Original Potential requirement No. | Potential requirement text |  |
| [CPR.8.6-1] | PR 5.5.6-002 | The 5G system shall support routing efficiency for data traffic between two UEs through an Evolved Residential Gateway. |  |
| [CPR.8.6-2] | PR 5.4.6-001 | The 5G system shall support routing efficiency for data traffic between a UE and a non-3GPP device through an Evolved Residential Gateway. |  |
| [CPR.8.6-3] | PR 5.6.6-001 | The 5G system shall provide a mechanism for supporting real time E2E QoS monitoring for the data traffic path (i.e., from/to a UE to/from the 5GC via a Premises Radio Access Station and a Evolved Residential Gateway). |  |
| [CPR.8.6-4] | PR 5.10.6-002 | The 5G system shall support real time E2E QoS monitoring and control for any data traffic path (i.e. from/to a UE to/from the 5GC and to/from another UE) via a PRAS and an eRG when there is connectivity to the 5G system. |  |

Editor’s Note: the consolidated requirements above are only an indication of grouping of potential requirements in the different sections and as a basis for further work on consolidation. Further consolidation of these requirements is needed. This can imply that requirements are changed, merged or not accepted as consolidated requirement.

## 8.7	Provisioning

Table 8.7-1 – RESIDENT Provisioning Consolidated Requirements

| CPR No. | Potential Requirement |  |  |
| --- | --- | --- | --- |
|  | Original Potential requirement No. | Potential requirement text |  |
| [CPR.8.7-1] | PR 5.18.6-001 | The 5G system shall enable configuration and management of an eRG by both the operator of the public (mobile) network the eRG is connected to and, within the boundaries defined by the operator, by an Authorised Administrator. |  |
| [CPR.8.7-2] | PR 5.18.6-003 | The 5G system shall enable configuration and management of a PRAS by a PLMN and, within the boundaries defined by the operator, by an Authorised Administrator. Specifically the PLMN shall be able to configure: radio settings pertaining to licensed spectrum shall be configured by the PLMN that owns the spectrum. Specifically the Authorised Administrator shall be able to configure:Whether visitor access network via the PRAS is allowed (allowing all or no visitors, or allowing specific visitors only) |  |
| [CPR.8.7-3] | PR 5.7.6-001 | The 5G system shall provide mechanisms for the network operator or an Authorised Administrator (e.g. a homeowner) to trigger remote provisioning of evolved residential gateways and for the network operator to perform remote provisioning of eRGs, which includes verification and configuration of evolved residential gateway identity and initial OA&M provisioning. |  |
| [CPR.8.7-4] | PR 5.7.6-002 | The 5G system shall provide mechanisms for the network operator or an Authorised Administrator (e.g. a homeowner) to trigger remote provisioning of Premises Radio Access Stations and for the network operator to perform remote provisioning of PRASs, which includes verification and configuration of Premises Radio Access Station identity and initial OA&M provisioning. |  |
| [CPR.8.7-5] | PR 5.12.6-002 | The 5G system shall provide means for an authorized user to prioritize access for a certain UE in a PRAS and CPN, within the limits given by the operator policy. |  |

Editor’s Note: the consolidated requirements above are only an indication of grouping of potential requirements in the different sections and as a basis for further work on consolidation. Further consolidation of these requirements is needed. This can imply that requirements are changed, merged or not accepted as consolidated requirement.

## 8.8	Premises Radio Access Station

Table 8.8-1 – RESIDENT Premises Radio Access Station Consolidated Requirements

| CPR No. | Potential Requirement |  |  |
| --- | --- | --- | --- |
|  | Original Potential requirement No. | Potential requirement text |  |
| [CPR.8.8-1] | PR 5.1.6-001 | The 5G system shall enable the network operator to provide any 5G services to any 5G UE via a Premises Radio Access Station (PRAS) connected via an evolved Residential Gateway (eRG). |  |
| [CPR.8.8-2] | PR 5.2.6-001 | The 5G system shall enable the network operator to provide any 5G services via a Premises Radio Access Station (PRAS) to any 5G UE with a valid subscription to the HPLMN associated with the PRAS. |  |
| [CPR.8.8-3] | PR 5.2.6-002 | The 5G system shall enable the network operator to provide any 5G services via a Premises Radio Access Station (PRAS) to any 5G UE with a valid subscription to any VPLMN that has a roaming agreement with the HPLMN. |  |
| [CPR.8.8-4] | PR 5.2.6-003 | The 5G system shall ensure the use of a Premises Radio Access Station (PRAS) does not compromise the security of any PLMN or broadband access network. |  |
| [CPR.8.8-5] | PR 5.2.6-004 | The 5G system shall ensure the use of a Premises Radio Access Station (PRAS) does not compromise the security of the UE. The PRAS (and its associated backhaul connectivity) shall provide a level of security equivalent to regular 5G base stations. |  |
| [CPR.8.8-6] | PR 5.2.6-005 | The 5G system shall enable the network operator associated with the Premises Radio Access Station (PRAS) to control the security policy of the PRAS. |  |
| [CPR.8.8-7] | PR 5.2.6-006 | The 5G system shall ensure the Premises Radio Access Station (PRAS) does not compromise user privacy for UEs that are using the PRAS, including communication confidentiality, location privacy and identity protection. |  |
| [CPR.8.8-8] | PR 5.2.6-007 | The 5G system shall be able to generate charging information that can differentiate between UEs connected to the Premises Radio Access Station (PRAS) and between backhaul for the PRAS and other data traffic over the same access. |  |
| [CPR.8.8-9] | PR 5.5.6-001 | The 5G system shall be able to provide QoS control for the communication path between a UE and an Evolved Residential Gateway via a Premises Radio Access Station. |  |
| [CPR.8.8-10] | PR 5.12.6-001 | The 5G system shall provide means to enable/disable a UE to connect to a Customer Premises Network device via a particular Premises Radio Access Station. |  |
| [CPR.8.8-11] | PR 5.12.6-003 | The 5G system shall minimize service disruption when a CPN communication path changes between two PRASes. |  |
| [CPR.8.8-12] | PR 5.14.6-001 | The 5G system shall be able to support PRAS sharing between multiple PLMNs. |  |

Editor’s Note: the consolidated requirements above are only an indication of grouping of potential requirements in the different sections and as a basis for further work on consolidation. Further consolidation of these requirements is needed. This can imply that requirements are changed, merged or not accepted as consolidated requirement.

## 8.9		5G-LAN

Table 8.9-1 – RESIDENT 5G-LAN Consolidated Requirements

| CPR No. | Potential Requirement |  |  |
| --- | --- | --- | --- |
|  | Original Potential requirement No. | Potential requirement text |  |
| [CPR.8.9-1] | PR 5.8.6-001 | The 5G system shall be able to support large amounts of small 5G LAN-VNs targeting residential deployments. |  |
| [CPR.8.9-2] | PR 5.8.6-002 | The 5G system shall support authorized 3rd parties to authorize/deauthorize UEs to be able to access a 5G LAN-VN. |  |
| [CPR.8.9-3] | PR 5.9.6-001 | The 5G system shall support the use of an evolved Residential Gateway to connect 5G devices from the 5G LAN VN it belongs to with non-3GPP devices on an in-home LAN. |  |
| [CPR.8.9-4] | PR 5.16.6-001 | The 5G system shall support interconnection of a 5G LAN-Virtual Network (5G LAN-VN) with a fixed IP VPN. |  |

Editor’s Note: the consolidated requirements above are only an indication of grouping of potential requirements in the different sections and as a basis for further work on consolidation. Further consolidation of these requirements is needed. This can imply that requirements are changed, merged or not accepted as consolidated requirement.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: " Service requirements for the 5G system; Stage 1".

[3]	3GPP TS 22.220: "Service requirements for Home Node B (HNB) and Home eNode B (HeNB)".

[4]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[5]	3GPP TS 22.278: "Service requirements for the Evolved Packet System (EPS)".

[6]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[7]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS) "

[x]	3GPP TR 22.859: "Study on Personal Internet of Things (PIoT) networks"