3GPP TSG-SA1 Meeting #95e	S1-213026r1

Electronic Meeting, 23 August –2 September 2021	(revision of S1-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.280 | CR | 0146 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Sharing administrative configuration between Mission Critical Organizations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BDBOS, Home Office |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG1 |  |  |  |  |  |  |  |  |  |
| Work item code: | SACI_MCS |  |  |  |  |  | Date: |  |  | 2021-08-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | [R-5.16.2-005] outlines general requirement for sharing information between Mission Critical Organizations. This is to enable interworking between Mission Critical Organizations located at different, but connected systems. In addition and to accomplish the exchange of administrative information a variety of standardized tasks are required. These are currently unspecified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New requirements extracted from 3GPP TR 22.881 for the exchange of administrative configuration between connected MCX Service systems. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Essential requirements for sharing administrative configuration between connected MCX Service systems are missing and specification is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.16.2, 5.16.3, 5.16.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.16.2	General requirements

[R-5.16.2-001] The MCX Service shall be able to support multiple Mission Critical Organizations, each with their own MCX Users and MCX Service Groups, on the same MCX Service system.

[R-5.16.2-002] The MCX Service shall provide a mechanism by which Mission Critical Organizations designate and manage (i.e., add, delete, change authorizations, etc.) MCX Service Administrators with authority to manage users, groups, other MCX Service Administrators, security controls, and other mission affecting parameters (e.g., authorizations and priorities) of the MCX Service.

[R-5.16.2-003] The MCX Service shall protect the operational privacy of Mission Critical Organizations by providing effective separation between the administrative and security management (e.g., key) parameters of those organizations except as authorized by the Mission Critical Organizations involved.

[R-5.16.2-004] The MCX Service shall protect the administrative and security management parameters of Mission Critical Organizations from viewing and manipulation by individuals (including those within and outside of the mission critical organization) not explicitly authorized by the Mission Critical Organization.

[R-5.16.2-005] The MCX Service shall provide a mechanism by which Mission Critical Organizations may share subsets of their administrative and security parameters with other Mission Critical Organizations.

NOTE:	The purposes of these requirements protect the operational security of organizations while still allowing for interworking of MCX UE and Users under the control of the Mission Critical Organizations.

### 5.16.3	Operational visibility for Mission Critical Organizations

[R-5.16.3-001] The MCX Service shall provide a mechanism by which an MCX Service Administrator associated with a Mission Critical Organization has visibility into the operational status of the service (e.g., during a natural disaster).

### 5.16.4	Sharing of administrative configuration between Mission Critical Organizations

Mission Critical Organizations can deploy their own 3GPP standardized mobile, local, countrywide or national broadband mission critical communications systems. In order to grant visiting MCX Service Users access to a Partner MCX Service System and its services, a mechanism is required, which allows the exchange of administrative configuration between connected MCX Service Systems.

NOTE:	Administrative configuration is used for adding MCX Service Users into a Partner MCX Service System’s user and group databases, modifying security controls and other mission affecting parameters, and for enabling or disabling certain services for specific users. Relevant information can be exchanged, for example, prior to the visiting MCX Service Users arriving at a Partner MCX Service System or dynamically during events.

[R-5.16.4-001] An MCX Service shall provide a mechanism to allow an authorised MCX User to request MCX User configuration changes in one or more Partner MCX Service Systems (e.g., priorities, authorizations).

[R-5.16.4-002] An MCX Service shall provide a mechanism to allow an authorised MCX User to evaluate and respond to requests for configuration changes from Partner MCX Service Systems.

[R-5.16.4-003] An MCX Service shall provide a mechanism to allow an authorised MCX User to configure automatic responses to categories of requests for configuration changes from Partner MCX Service Systems (e.g., particular users, or groups).

[R-5.16.4-004] An MCX Service shall provide a mechanism to allow an authorised MCX User to request configuration information (e.g., users, groups, security level) from Partner MCX Service Systems.

[R-5.16.4-005] An MCX Service shall provide a mechanism to allow an authorised MCX User to send configuration information to Partner MCX Service Systems.

[R-5.16.4-006] An MCX Service shall provide a mechanism to allow an authorised MCX User to exchange operationally relevant information for specific MCX Users, e.g., MCX User capability information, with Partner MCX Service Systems.

NOTE:	Capability information could include, but not limited to, information such as chemical, biohazard, medical or equipment handling capabilities for a specific MCX User.

[R-5.16.4-007] An MCX Service System shall provide a mechanism to allow secure exchange of administrative and security related information with interconnected MCX Service systems without compromising the integrity and security of either system.

[R-5.16.4-008] An MCX Service System shall provide a mechanism to allow exchange of administrative and security related information with interconnected MCX Service systems without exposing the internal network topology of either system.