3GPP TSG-SA1 Meeting #95e	S1-213030r1

Electronic Meeting, 23 August – 2 September 2021	(revision of S1-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.261 | CR | 0542 | rev | ‒ | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EXPOSE: addition to QoS monitoring requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Siemens, Harting, Mercedes-Benz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | WG1 |  |  |  |  |  |  |  |  |  |
| Work item code: | EXPOSE |  |  |  |  |  | Date: |  |  | 2021-08-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 5G-ACIA compiled a list of service exposure requirements and shared it with SA WG1 (see S1-211274). An analysis of the 5G-ACIA requirements [R-4.2.4-02], [R-4.2.4-04], [R-4.2.4-06] and [R-4.2.4-08] indicated that three requirements in TS 22.261, clause 6.23.2 need to be extended and one requirement needs to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A note is added to the requirement that addresses the notification of events. This note extends the meaning of the term “event”. It is explained that the response time to monitoring request is subject to negotiation between the user and the 5G system. It is also explained that the time span for collection and evaluation of statistical values can be specified by the user. A requirement concerning QoS monitoring of groups of UEs is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Capabilities that are important to industrial automation are not reflected in Stage-1 specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.23.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | See S1-213028 for more information on this CR. @MCC: a note numbering conflict will occur if CR 0543 is also approved. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

___________________________________Start of  Proposed Changes_______________________________________

_______________________________________Start of Change 1___________________________________________

### 6.23.2	Requirements

The 5G system shall provide a mechanism for supporting real-time E2E QoS monitoring within a system.

NOTE 1:	The end points in E2E are the termination points of the communication service within the boundary of the 5G system.

The 5G system shall support E2E QoS monitoring of individual UEs and groups of UEs.

NOTE 1A: The groups of UEs can but must not be the same as a LAN-type group (see clause 6.26).

The 5G network shall provide an interface to an application for QoS monitoring (e.g. to initiate QoS monitoring, request QoS parameters, events, logging information).

The 5G system shall be able to provide real time QoS parameters and events information to an authorized application / network entity.

NOTE 2:	The QoS parameters to be monitored and reported can include latency (e.g. UL/DL or round trip), jitter, packet loss rate.

The 5G system shall be able to log the history of the communication events. This includes for examples parts of the SLA that are not met, time-stamp of the events, and event position (e.g. UEs and radio access points associated with the events).

The 5G system shall support different levels of granularity for QoS monitoring (e.g. per flow or set of flows).

The 5G system shall be able to provide event notification upon detecting error that the negotiated QoS level cannot be met/guaranteed.

The 5G system shall be able to provide information that identifies the type and the location of a communication error (e.g. cell ID).

The 5G system shall be able to provide notification of communication events to authorized entities per pre-defined patterns.

NOTE 3: An example for a communication event is that the service bit rate drops below a pre-defined threshold for QoS parameters. When such an event occurs, the authorized entity is notified, and the event is logged.

The 5G system shall be able to provide notification of events to authorized entities per pre-defined patterns.

NOTE 4:	An example for such an event is a position change of the pertinent UE. A poisiton change can, for instance, be inferred from 5G positioning service that tracks the UE.

The 5G system shall be able to respond to a request from an authorized entity to provide real-time QoS monitoring information within a specified time after receiving the request (e.g., within 5 s).

NOTE 5:	This time is subject to negotiation between the user and the 5G system.

The 5G system shall support an update/ refresh rate for real time QoS monitoring within a specified time (e.g. at least 1 per second).

The 5G system shall be able to provide statistical information of service parameters and error types while a communication service is in operation.

NOTE 6:	The time span for collection and evaluation of statistical values can be specified by the user.

The 5G system shall provide information on the current availability of a specific communication service in a particular area (e.g. cell ID) upon request of an authorized entity.

_______________________________________End of Change 1___________________________________________

__________________________________End of Proposed Changes_________________________________________