3GPP TSG-SA WG1 Meeting #95-e 	S1-213107

Electronic Meeting, 23 Aug –2 Sep 2021	(revision of S1-21xxxx)

Source:	China Mobile

pCR Title:	pCR on Adding use case for remote UAV control through HD video in First Person View

Draft Spec:	3GPP TR22.847

Agenda item:	7.12

Document for:	Approval

Contact: 	Xiaonan Shi, shixiaonan@chinamobile.com

Abstract: This paper proposes a use case for remote UAV control through HD video in First Person View. In this case, people who control a Non-line-of-sight UAV is wearing a head-mounted display, and can send flight command i.e. C2 message, through control handle to UAV based on the real-time video. During UAV mobility, especially handover case, the target RAN may do not have enough resource, for C2 message flow, the target RAN is expected to send notification to AF(existing feature in R17 for GBR),but since the video(usually non-GBR) is quite important for UAV control, the video traffic flow is expected to be kept in the target RAN. This is a coordination processing for the two traffic flows.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	ITU-T, “Technology Watch Report: The Tactile Internet”, August 2014.

[3]	O. Holland et al., “The IEEE 1918.1 “Tactile Internet” Standards Working Group and its Standards,” Proceedings of the IEEE, vol. 107, no. 2, Feb. 2019.

[4]	3GPP TS 22.263: "Service requirements for Video, Imaging and Audio for Professional Applications".

[5]	S. K. Sharma, I. Woungang, A. Anpalagan and S. Chatzinotas, "Toward Tactile Internet in Beyond 5G Era: Recent Advances, Current Issues, and Future Directions," in IEEE Access, vol. 8, pp. 56948-56991, 2020

[6] 	3GPP TS 22.261: " Service requirements for the 5G system ".

[7] 	Kwang Soon Kim, et al., " Ultrareliable and Low-Latency Communication Techniques for Tactile Internet Services", PROCEEDINGS OF THE IEEE, Vol. 107, No. 2, February 2019

[8]	"SAE J3186: Application Protocol and Requirements for Maneuver Sharing and Coordinating Service", version 0.28, developing document, Apr. 2021.

[9] 	SAE J3224, "V2X Sensor-Sharing for Cooperative & Automated Driving" version 0.06, developing document, Apr. 2021.

[10] 	M. During and K. Lemmer, “Cooperative maneuver planning for cooperative driving,” IEEE Intell. Transp. Syst. Mag., vol. 8, no. 3, pp. 8–22, Jul. 2016.

[11]	D. Soldani, Y. Guo, B. Barani, P. Mogensen, I. Chih-Lin, S. Das, "5G for ultra-reliable low-latency communications". IEEE Network. 2018 Apr 2;32(2):6-7.

[12]	O. Dubuisson and P. Fouquart, "ASN.1: Communication between Heterogeneous Systems", Morgan Kaufmann Publishers Inc., San Francisco, CA, USA, 2001.

[13]	IEEE SA, P1918.1 - Tactile Internet: Application Scenarios, Definitions and Terminology, Architecture, Functions, and Technical Assumptions, https://standards.ieee.org/project/1918_1.html

[14] 	M. Eid, J. Cha, and A. El Saddik, "Admux: An adaptive multiplexer for haptic-audio-visual data communication", IEEE Tran. Instrument. and Measurement, vol. 60, pp. 21–31, Jan 2011.

[15]	 K. Iwata, Y. Ishibashi, N. Fukushima, and S. Sugawara, "QoE assessment in haptic media, sound, and video transmission: Effect of playout buffering control", Comput. Entertain., vol. 8, pp. 12:1–12:14, Dec 2010.

[16]	N. Suzuki and S. Katsura, "Evaluation of QoS in haptic communication based on bilateral control", in IEEE Int. Conf. on Mechatronics (ICM), Feb 2013, pp. 886–891.

[17] 	E. Isomura, S. Tasaka, and T. Nunome, "A multidimensional QoE monitoring system for audiovisual and haptic interactive IP communications", in IEEE Consumer Communications and Networking Conference (CCNC), Jan 2013, pp. 196–202.

[18]	A. Hamam and A. El Saddik, "Toward a mathematical model for quality of experience evaluation of haptic applications", IEEE Tran. Instrument. and Measurement, vol. 62, pp. 3315–3322, Dec 2013.

[19] 	M. Back et al., "The virtual factory: Exploring 3D worlds as industrial collaboration and control environments," 2010 IEEE Virtual Reality Conference (VR), 2010, pp. 257-258

[20] 	S. Schulte, D. Schuller, R. Steinmetz and S. Abels, "Plug-and-Play Virtual Factories," in IEEE Internet Computing, vol. 16, no. 5, pp. 78-82, Sept.-Oct. 2012

[X] 	DJI Innovation, “DJI FPV unbox: One-stop interpretation of product highlights and performance”, https://store.dji.com/cn/guides/dji-fpv-unboxing/, Feb. 2021

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

Multi-modality Data: Multi-modality Data is defined to describe the input data from different kinds of devices/sensors or the output data to different kinds of destinations required for the same task. Multi-modality Data consists of more than one Single-modality Data, and there is strong dependency among each Single-modality Data. Single-modality Data can be seen as one type of data.

Above ground level (AGL): In the context of a UAV it is the UAV altitude referenced to ground level in the vicinity.

Command and Control (C2) Communication: the user plane link to convey messages with information of command and control for UAV operation between a UAV controller and a UAV.

Unmanned Aerial System (UAS): Composed of Unmanned Aerial Vehicle (UAV) and related functionality, including command and control (C2) links between the UAV and the controller, the UAV and the network, and for remote identification. A UAS is comprised of a UAV and a UAV controller.

Note: The definition of AGL, C2 and UAS are referenced from TS22.125.

Non Line of Sight (NLOS): The two points of communication are blocked and cannot see each other.

## 3.2	Symbols

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

DOF 	Degrees Of Freedom

UAV	Unmanned Aerial Vehicle

C2	Command and Control

UAS	Unmanned Aerial System

NLOS	Non Line Of Sight

## 5.X	Remote UAV control through HD video in First Person View

### 5.X.1	Description

In first person view mode, the flight control is further simplified and can allow more people to experience the fun of immersive flight. There are UAV products which can support the first person view UAV control, e.g. DJI FPV[x]

In this use case, the UAV, control handle, head-mounted display with HD video are used together. People who control the UAV should put on the head-mounted display and based on the HD video to control the UAV flight. The head-mounted has ultra wide angle of view (e.g.150 degrees) and provide clear HD video with 120 fps. In this use case, the UAV is in a Non-line-of-sight scenario, people who control the UAV need to see the video for control assistance to see the surrounding of UAV. So the video in this use case need specially guarantee to support the control decision.

### 5.X.2	Pre-conditions

People who control the UAV should put on the head-mounted display and put the control handle in hands.

### 5.X.3	Service Flows

1. The UAV access to the 5G network.

2. The UAV taking off and people can control the UAV around 10km in NLOS. People can see the UAV flight environment thought the head-mounted display.

3. The UAV flies from one 5G RAN to another 5G RAN (target RAN), but the target RAN radio source is limited.

4. Since the C2 message is guaranteed by 5G system with GBR (one possible solution to guarantee the C2 message), once the target RAN cannot support the GBR flow, the target RAN is still expected to continue the handover procedure in order to keep the UAV control continuity. And after the handover procedure the target RAN should evaluate whether current radio condition can support the GBR, if not, the target RAN should notify this to AF (existing solution in 3GPP TS 23.501).

At the same time, since the HD video in head-mounted display is quite important to the UAV control, even this video is carried on non-GBR flow, the target RAN should also keep this traffic flow to help the UAV control.

The relationship between C2 message and HD video in head-mounted display should be configured by UAV AF, therefore in the handover case, once the radio resource is limited, these two traffic flows can be guaranteed together.

### 5.X.4	Post-conditions

The UAV still under people’s control and can fly in the target RAN coverage. People can continue to experience the immersive flying.

### 5.X.5	Existing features partly or fully covering the use case functionality

KPI requirements for video used to aid UAV control can be found in TS 22.125.

Table 7.2-1 KPIs for command and control of UAV operation

| Control Mode | Function | Typical Message Interval | Max UAV ground speed | Typical message Size (note 1) | End to end Latency | Reliability (note 2) | Positive ACK (note 8) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Steer to waypoints (note 3) | UAV terminated C2 message | >=1 s | 300 km/h | 100 byte | 1 s | 99.9% | Required |
|  | UAV originated C2 message (note 4) | 1 s |  | 84-140 byte | 1 s | 99.9% | Not Required |
| Direct stick steering (note 5) | UAV terminated C2 message | 40 ms (note 6) | 60km/h | 24 byte | 40 ms | 99.9% | Required |
|  | UAV originated C2 message (note 7) | 40 ms |  | 84-140 byte | 40 ms | 99.9% | Not Required |
| Automatic flight on UTM (note 10) | UAV terminated C2 message | 1 s | 300 km/h | <10K byte | 5 s (note 9) | 99.9% | Required |
|  | UAV originated C2 message | 1 s (note 9) |  | 1500 byte | 5 s (note 9) | 99.9% | Required |
| Approaching Autonomous Navigation Infrastructure | UAV terminated C2 message | 500 ms | 50 km/h | 4k byte | 10 ms | 99% | Required |
|  | UAV originated C2 message | 500 ms |  | 4k byte | 140 ms | 99.99% | Required |
| NOTE 1: Message size is at the application layer and excludes any headers and security related load. The numbers shown are typical as message size depends on the commands sent and is implementation specific. NOTE 2: Message reliability is defined as the probability of successful transmission within the required latency at the application layer while under network coverage. NOTE 3: Video is neither required nor expected to be used for steering in this mode. NOTE 4: It may be possible to transmit this message on an event driven basis e.g. approaching a geo fence. A status message may but is not required to be sent as a response to a control message. NOTE 5: A video feedback is required for this mode. The KPIs for video are defined in table 7.2-2. NOTE 6: UAVs on board controllers typically update at either 50Hz (20ms) or 25Hz (40ms). NOTE 7: A status message may but is not required to be sent as a response to a control message A 1Hz slow mode also exists. NOTE 8: Positive ACK is sent to the originator of the message (i.e. UAV controller and / or the UTM). The 5G system makes no assumption whether an appropriate ACK is sent by the application layer. NOTE 9: t the application level, the C2 communication between a UAV and UTM can be allowed to experience much longer traffic interruptions, e.g. timeouts of 30S on the uplink and 300S on the downlink. NOTE 10: This only represents periodic message exchange during a nominal mission in steady state. It does not represent unusual or aperiodic events such as conveying dynamic restrictions or a flight plan to the UAV on the downlink. |  |  |  |  |  |  |  |

Table 7.2-2 KPIs for video used to aid UAV control.

| Scenario (note 2) | Data rate | End to end Latency | Reliability (note 1) | Direction | Positive ACK required |
| --- | --- | --- | --- | --- | --- |
| VLOS (visual line of sight) | 2 Mpbs at 480 p, 30 fps | 1 s | 99.9% | Sent by UAV | Not Required |
| Non-VLOS | 4 Mbps at 720 p, 30 fps | 140 ms | 99.99% | Sent by UAV | Not Required |
| NOTE 1: Message reliability is defined as the probability of successful transmission within the required latency. NOTE 2: Maximum UAV speed is same as control mode of direct stick steering in 7.2-1 |  |  |  |  |  |

### 5.X.6	Potential New Requirements needed to support the use case

[PR 5.X.6-1] 5G system shall provide a mechanism for a 3rd party application to provide requirement about the co-ordinately processing for related multiple traffic flows within one service through network exposure capability.

[PR 5.X.6-2] 5G system shall be able to identify the traffic flows which should be processed co-ordinately.

[PR 5.X.6-3] 5G system shall be able to process the multiple traffic flows subject to the same TACMM service during mobility together according to 3rd party application policy.