3GPP TSG-SA1 Meeting #95e	S1-213110r5

Electronic Meeting, 23 August - 2 September 2021	(revision of S1-213110)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.104 | CR | 0078 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Smart Energy Infrastructure Requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, EUTC, China Telecom, ZTE, Vodafone, Huawei, NOVAMINT, Nokia, Nokia Shanghai Bell, Anterix |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEI |  |  |  |  |  | Date: |  |  | 2021-8-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The Smart Energy Infrastructure feature captures the requirements identified by the study in TR 22.867. That study considered smart energy services, e.g. IEC standards, and their communication requirements including capacity, latency, availability, end-to-end QoS, resiliency / redundancy and security. Requirements were identified for operational manageability, e.g. the ability to configure and monitor the real (achieved and up to date) availability of communication services. Emerging smart grid use cases and their functional requirements were also identified, e.g. for on-demand power supply, distributed power supply systems, distribution automation, high accuracy power load measurement and control, meter automation and more. Communication KPIs and their service requirements for enabling micro-grids, DER and specifically distributed generation (DG) that require 5G wireless communication are also included. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new clause is added to 22.104 to capture smart energy requirements for energy system recovery. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Smart Energy Infrastructure feature’s requirements will not be included in the normative standard. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 22.261 CR 0547, TS 22.261 CR 0079 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Requirements added to 22.104 for the SEI feature are specific to smart energy vertical service. Other requirements that are general to the 5G system that were identified as part of the FS_5GSEI study are provided in a separate CRs to 22.261 and 22.104. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the Change---

# 8	Recovery of infrastructure for electrical distribution

## 8.1	Description

The robustness of the infrastructure for electrical power distribution may depend upon the possibility to operate telecommunication networks even during an energy system incident, in which electricity cannot be delivered to some network operator facilities. Through coordination between the network operator and the energy system operator, increases in the ability to recover the energy system operation can be achieved.

## 8.2	Requirements

Subject to regulatory requirements and operator policy, the 5G system shall support a mechanism by which an MNO can identify the ability of the MNO's infrastructure to continue operation despite a lack of electrical supply service, specifying which physical regions would be affected in terms of physical topology and the remaining time in which operation is possible.

NOTE1: This information can facilitate energy distribution system recovery operations.

Subject to regulatory requirements, the 5G system shall support a mechanism by which a third party can, in the event of an energy distribution system service interruption, communicate the energy distribution system recovery status in terms of location and time table to the MNO.

NOTE2: This information can facilitate MNO operations to facilitate energy system recovery.

## ---End of the Change---