3GPP TSG-SA1 Meeting #96-e	S1-214074r2

# Online, , 8th Nov 2021 - 18th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.011 | CR | 0326 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Periodic network selection with shared MCC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KPN, TNO, Qualcomm, Apple, Ericsson, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The existing requirements in TS22.011 for periodic network selection specify that a UE in automatic mode shall only select a higher priority PLMN within the same country. In this context, country is to be interpreted as a single MCC. This implies that a UE that is using a country specific MCC cannot consider (satellite) networks that use a shared MCC (e.g. MCC=901). Similarly, a UE that uses a network with MCC=901 will not consider higher priority (terrestrial) networks with a country specific MCC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modification of the requirement such that shared MCCs can also be considered if the current PLMN has a country specific MCC and that all other MCCs can be considered when the current network uses a shared MCC. The Shared MCC scenario is explained in a note and a reference to ITU-T E.212 for the definition of Shared MCC was added. Editorial changes were made in clause 1.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Switching between satellite networks with shared MCC and terrestial networks will only happen at switch on or when moving out of coverage of the current network. Note that moving out of coverage is unlikely to happen with satellite networks, unless the UE moves indoors. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1.1, 3.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 1.1	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	Void

[2]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[3]	3GPP TS 23.122: "Non Access Stratum functions related to Mobile Station (MS) in idle mode".

[4]	ITU-T Recommendation Q.1001: "General aspects of Public Land Mobile Networks".

[5]	3GPP TS 22.043: "Support of Localised Service Area (SoLSA). Stage 1".

[6]	3GPP TS 22.234: "Requirements on 3GPP system to wireless local area network (WLAN) interworking".

[7]	3GPP TS 22.101: "Service aspects; Service principles".

[8]	3GPP TS 22.278: "Service requirements for the Evolved Packet System (EPS)".

[9]	3GPP TS 36.304: "User Equipment (UE) procedures in idle mode"

[10]	3GPP TS25.304: "User Equipment (UE) procedures in idle mode and procedures for cell reselection in connected mode".

[11]	3GPP TS 25.304: "User Equipment (UE) procedures in idle mode and procedures for cell reselection in connected mode”.

[12]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

[13]	3GPP TS 36.331: "E-UTRA Radio Resource Control (RRC)".

[14]				3GPP TS 22.220: "Service requirements for Home NodeBs and Home eNodeBs".

[15]				3GPP2 C.S0004-A v6.0: "Signaling Link Access Control (LAC) Standard for cdma2000 Spread Spectrum Systems – Addendum 2"

[16]				3GPP TS 22.153: "Multimedia priority service".

[17]				3GPP TS 36.306: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities"

[18]			3GPP TS 43.064: " General Packet Radio Service (GPRS); Overall description of the GPRS radio interface; Stage 2"

[19]	ITU-T Recommendation E.212: "The international identification plan for public networks and subscriptions"

#### 3.2.2.5	Periodic network selection attempts

A UE in Automatic Mode shall make periodic attempts to look for a higher priority PLMN including associated Access Technology of the same country as the currently received PLMN including associated Access Technology or for a higher priority PLMN including associated Access Technology that uses a Shared MCC (e.g. MCC=901). If the currently received PLMN including associated Access Technology uses a Shared MCC, also a higher priority PLMN including associated Access Technology using any non-shared MCC shall be considered. For the ranking of PLMNs the UE shall use the order used in clause 3.2.2.2. In the case that there is no associated Access Technology identifier, the mobile shall assume that all Access Technologies provided by a PLMN are of equal priority. Moreover, periodic network selection shall not lead to change of access technology within the registered PLMN.

In the case that the UE has stored a list of equivalent PLMNs, the UE shall only select a PLMN if it has a higher priority than all the PLMNs, in the list of equivalent PLMNs, which are of the same country as the currently registered PLMN.

NOTE 1:	In the context of this 3GPP TS, the term country is to be interpreted not as a political entity but as a single Mobile Country Code (MCC). For example the USA and India have multiple MCC. Such cases are in fact treated as exceptions in the 3GPP specifications. For all other countries, multiple MCCs may be used, however the specifications have not taken this into account and there could be adverse effects such as the UE being unable to detect that multiple MCCs are within the same country.

NOTE 2:	There are some situations where one MCC represents multiple political entities or where an MCC is independent of political entities. Examples include Shared MCCs as defined in ITU-T E.212 [19], where international satellite operations typically use the Shared MCC with value 901.

In the case that there are multiple EHPLMNs available, the UE shall not attempt to select a higher priority EHPLMN when an EHPLMN has already been selected. The priorities of EHPLMNs are only applicable when the UE is on a VPLMN and multiple EHPLMNs are available.

The UE shall only make reselection attempts while in idle mode. In case of GPRS terminals, the UE shall only make reselection attempts while in Idle or Stand-by mode. In case of terminals operating E-UTRA and/or NR, the UE shall only make reselection attempts while in Idle or RRC-Inactive mode.

In the case of a UE receiving an eMBMS transport service when in idle mode, the UE may postpone the higher priority PLMN search and any reselection attempt until the eMBMS transport service has finished or been stopped.

The interval between attempts shall be stored in the SIM/USIM.

Only the service provider shall be able to select for which of the previous situations, periodic network selection shall be attempted and to set the interval value.

For UEs supporting only Access Technologies other than the following: NB-IoT, GERAN EC-GSM-IoT and Category M1 [17] of E-UTRAN enhanced-MTC the UE shall interpret the interval value to be between 6 minutes and 8 hours, with a step size of 6 minutes.

For UEs only supporting any of the following, or a combination of, NB-IoT, GERAN EC-GSM-IoT [18], and Category M1[13] of E-UTRAN enhanced-MTC, the UE shall interpret the interval value to be between 2 and 240 hours, with a step size of 2 hours between 2 and 80 hours and a step size of 4 hours between 80 and 240 hours.

For UEs supporting a combination of IoT and non-IoT Access Technologies the UE shall interpret the interval value to determine the timer value as above based on the Access Technology currently in use by the UE.

One interval value shall be designated to indicate that no periodic attempts shall be made.

In the absence of a permitted value in the SIM/USIM, or the SIM/USIM is phase 1 and therefore does not contain the datafield, then a default value of 60 minutes, shall be used by the UE except for those UEs only supporting any of the following, or a combination of: NB-IoT, GERAN EC-GSM-IoT [18], and Category M1 [17] of E-UTRAN enhanced-MTC. For those UEs a default value of 72 hours shall be used.

NOTE:	Use of values less than 60 minutes may result in excessive UE battery drain.