3GPP TSG-SA WG1 Meeting #99e 	S1-222107

Electronic Meeting, 22 Aug – 01 Sep 2022

Source:	Huawei, CAICT

pCR Title:	New use case: Sensing for railway intrusion detection

Draft Spec:	3GPP TR 22.837

Agenda item:	7.2

Document for:	Approval

Contact:	     < Fangyuan Zhu (zhufangyuan@huawei.com)>

Abstract: This document proposes a new use case about railway intrusion detection.

1. Introduction

Proper monitoring of railway environment is important to ensure railway safety. The early detection of possible dangers due to intrusion of unauthorized persons and animals on the railroad becomes essential. Traditional mechanism of railway monitoring using fibre optic sensing techniques is costly and requires manual intervention, making it very difficult to meet the increasing demand for railway monitoring.

Figure 1: Railway pedestrian intrusion [1]

Thanks to the 5G NR based sensing, the gNB as transmitter and receiver along the railway can constantly sense the railway situation such as railway intrusion.

2. Reason for Change

This contribution proposes a new use case about railway intrusion detection.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.837 v0.1.0.

4. Reference

1.       Guo B, Shi J, Zhu L, et al. High-speed railway clearance intrusion detection with improved SSD network[J]. Applied Sciences, 2019, 9(15): 2981.

* * * First Change * * * *(All new changes)

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	W. Favoreel, "Pedestrian sensing for increased traffic safety and efficiency at signalized intersections," 2011 8th IEEE International Conference on Advanced Video and Signal Based Surveillance (AVSS), 2011, pp. 539-542, doi: 10.1109/AVSS.2011.6027406.

[3]	Advances in Wildlife Crossing Technologies: https://highways.dot.gov/public-roads/septoct-2009/advances-wildlife-crossing-technologies.

[4]	Protection Detection: Making Roads Safe for Drivers and Wildlife: https://onlinepubs.trb.org/onlinepubs/webinars/201118.pdf.

[5]	F. Liu et al., "Integrated Sensing and Communications: Towards Dual-functional Wireless Networks for 6G and Beyond," in IEEE Journal on Selected Areas in Communications, doi: 10.1109/JSAC.2022.3156632.

[6]	T. S. Rappaport, G. R. MacCartney, M. K. Samimi and S. Sun, "Wideband Millimeter-Wave Propagation Measurements and Channel Models for Future Wireless Communication System Design," in IEEE Transactions on Communications, vol. 63, no. 9, pp. 3029-3056, Sept. 2015, doi: 10.1109/TCOMM.2015.2434384.

[7]	C. Han, Y. Bi, S. Duan and G. Lu, "Rain Rate Retrieval Test From 25-GHz, 28-GHz, and 38-GHz Millimeter-Wave Link Measurement in Beijing," in IEEE Journal of Selected Topics in Applied Earth Observations and Remote Sensing, vol. 12, no. 8, pp. 2835-2847, Aug. 2019, doi: 10.1109/JSTARS.2019.2918507.

[8]	Condition Monitoring of Railway Tracks and Vehicles with Distributed Fiber Optic Sensing: https://www.tugraz.at/institutes/igms/projects/current-projects/condition-monitoring-of-railways-with-distributed-fos/

[9]	Monitoring of Railway Deformations using Distributed Fiber Optic Sensors: https://www.fig.net/resources/proceedings/2016/2016_03_jisdm_pdf/nonreviewed/JISDM_2016_submission_30.pdf

[10]	Rail Industry Safety Induction Handbook: https://railsafe.org.au/__data/assets/pdf_file/0009/32022/Rail-Industry-Safety-Induction-RISI-Handbook-V5.1.pdf

[10]	3GPP TS 22.289: "Mobile Communication System for Railways; Stage1".

* * * Second Change * * * *

5	Use cases

5.4	Use case of sensing for railway intrusion detection

5.4.1	Description

With the rapid development and large-scale operation of high-speed railway technology, the speed of trains has been continuously improved (up to 500 km/h), and the safety management of trains has become more and more important. The early detection of possible dangers due to intrusion of unauthorized persons and animals on the railroad becomes essential [8]. The railway intrusion always happens suddenly and randomly, so the time and location of occurrence are difficult to predict. Traditional mechanism of railway monitoring using fibre optic sensing techniques is costly and requires manual intervention, making it very difficult to meet the increasing demand for railway monitoring [9].

5.4.2	Pre-conditions

Good partnership and cooperation are established between Railway Operator#A and Mobile Operator#B. Requested by Railway Operator#A, the suitable gNBs around/along a railway are selected, which enable Mobile Operator#A to constantly sense the railway including moving objects (e.g. pedestrians). The sensing signal emitted from the gNB arrives at objects and is bounced (reflected) back to the transmitting gNB.

The assumption of this use case is the following:

-	There is at least 300km mountainous train line as depicted in the figure 5.4.2-1[10] owned by Railway Operator#A. The safe place is a place where a person and their equipment cannot be struck by rail traffic, which is used for minimizing damage caused by possible railway accident or crash and for ensuring safe operation of railway. The danger zone is anywhere within 3m horizontally from the nearest track.

-	The size and typical velocity of traffic participant in this use case is described in the Table 5.2.2-1.

Table 5.2.2-1

|  | Size (Length x Width x Height) | Typical velocity |
| --- | --- | --- |
| Pedestrian (Adult) | 0.5m x 0.5m x 1.75m | 5km/h [9] |
| Animal (Sheep/deer) | 1.5m x 0.5m x 1 m | 5km/h [9] |
| Trains | 24m x 3.5m x 3 m | 100km/h - 350km/h |

When the pedestrian/animal standing at the outermost side of safe place starts walking on the danger zone, it means the intrusion happens. The distance that pedestrian/animal move perpendicular to the railway track is more sensitive for road safety, compared to the distance parallel to the railway track.

Figure 5.4.2-1

5.4.3	Service Flows

Figure 5.4.3-1: Railway intrusion detection

- The gNBs owned by Mobile Operator#B are installed around/along the railway. A train of Railway Operator#A starts running into a mountainous area. In order to acquire the sensing information of railway monitoring at the mountainous area, the Railway Operator#A requests sensing service from Mobile Operator#B. The Mobile Operator#B configures the gNBs along the train line to perform sensing

- Suddenly, some pedestrians/animals step out of the guardrail and walk on the danger zone. The gNBs constantly sense the railway. The sensing measurement data is reported from gNBs and further processed into the sensing results in the core network. The Mobile Operator#B exposes the sensing results to the Railway Operator#A according to the required latency requirement. Based on the sensing results, the location of pedestrians/animals can be estimated.

- The Railway Operator#A alarms the train driver and instructs the driver to stop running or drive on another the railway tracks. In addition, the staff working for Railway Operator#A immediately responds to the emergency. The pedestrians/animals leave the danger zone safely and the broken guardrail is repaired. All the passengers who are on the train arrive the destination safely.

5.4.4	Post-conditions

The efficiency of railway management and safety supervision is improved.

5.4.5	Existing features partly or fully covering the use case functionality

None.

5.4.6	Potential New Requirements needed to support the use case

[PR 5.4.6-1] The 5G system shall be able to support the following KPIs:

Table 5.4.6-1: Performance requirements for railway intrusion detection

| Scenario | Distance Accuracy ([95%] confidence level) |  | Velocity Accuracy ([95%] confidence level) |  | Resolution |  | Max allowed Sensing service latency | Refreshing rate |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Horizontal Accuracy | Vertical Accuracy | Horizontal Accuracy | Vertical Accuracy | Horizontal Resolution | Vertical Resolution |  |  |
| Pedestrian/animal intrusion detection on a railway (Note 1) | ≤1.5m (Note 2) | - | - | - | - | - | ˂1.5s (Note 3) | ≥10Hz |
| Note 1: the typical size (Length x Width x Height) of pedestrian is 0.5m x 0.5m x 1.75m and the typical size of animal is 1.5m x 0.5m x 1 m. The length of railway depends on the real environment. Note 2: the width of the danger zone horizontally from the nearest track is within 3m. Note 3: in order to ensure passenger safety if any hazardous situation is detected, this is the latency for 5G system to sense the intrusion and report the sensing information on the presence of a pedestrian/animal to railway management departments. |  |  |  |  |  |  |  |  |

* * * End of Change * * * *