3GPP TSG SA WG 1 Meeting #99e 	S1-222362

Electronic Meeting, 22 August – 1 September 2022	(revision of S1-22xxxx)

Source:	KPN

pCR Title:	Availability scenarios for Ambient IoT

Draft Spec:	3GPP TR 22.840

Agenda item:	7.3

Document for:	Approval

Contact:	toon .norp@ tno .nl

Abstract: This contribution proposes clarification of different power scenarios for Ambient IoT. When and how power is available to an Ambient IoT device has a significant impact on service aspects of Ambient IoT. That is why it is relevant to categorize different scenarios. It is proposed to capture the different scenarios in an Annex to TR22.840.

1. Introduction

Ambient IoT devices can be powered using different methods of power harvesting, e.g. RF power, vibrations/movement, light, heat. It has been agreed that “Specifics of how the device performs energy harvesting are not in the scope of the study”. Nevertheless, the resulting ‘power availability scenarios’ of when the Ambient IoT device has power available have an important impact on service aspects.

Three main power availability scenarios can be distinguished:

-	Continuous power

-	Intermittent power

-	On demand power triggered

These different scenarios are described below.

NOTE: These scenarios are not intended as definitions for device categories. The purpose is to enable discussion of service aspects that are associated with different power availability scenarios.

Continuous Power

In the continuous power scenario, devices have power available continuously or at least for signicant amounts of time (e.g. during daytime when powered by sunlight). The continuous availability of power can be because there is continuous power harvesting (e.g. light / heat / movement), possibly in combination with (limited) energy storage (e.g. in a capacitor) to overcome momentary variations in power harvesting.

The main effect of continuous power is that the processor and communications module in the Ambient IoT device can be continuously active (although in low power mode). The processor can perform tasks like maintaining timers, or log sensor measurements. The communications module can listen to network at regular intervals to determine if there is mobile terminated traffic (e.g. trigger messages) and can transmit data when relevant. Note that for a Ambient IoT device, power optimization for receiving (e.g. DRX and PSM) and sending data (e.g. lower power) should go beyond what is common for NB-IoT and LTE-M.

An exampe continuous power scenario is shown in figure 1

Figure 1: continuous power scenario (example)

As there is continuous availability of power, the communication of the Ambient IoT device does not depend on the power availability. The Ambient IoT device can communicate when needed, e.g. when there is sensor data or a status change to report, and can listen to the network on a regular basis (e.g. based on timers running in the processor). That the Ambient IoT device can listen to mobile terminated traffic is relevant for e.g. triggering and/or provisioning. Note that the network may need to be aware that the Ambient IoT device is only listening to the network on regular intervals.

Intermittent Power

In the intermittent power scenario, devices have power available only intermittently (e.g. a vibration sensor on a bridge that harvests the vibration energy of a passing vehicle).

The main effect of intermittent power is that the Ambient IoT device (including processor and communications module) can only be active for the short periods of time when power is available. E.g. the Ambient IoT device may not be able to listen to the network for mobile terminated traffic for very long periods of time. This has an impact on service aspects such as provisioning.

An exampe intermittent power scenario is shown in figure 2.

Figure 2: intermittent power scenario (example)

When there is no power available, the Ambient IoT device is not active. Assumption is that there is even no processor running that e.g. can maintain timers. The communication module is inactive and cannot transmit, nor receive data. Only when power becomes available, the processor and communication module are activated. The processor can e.g. log measurements or determine a status. When relevant (e.g. there is new data available, or a status has changed), the device can decide to transmit data to the network. When there is still power available after transmission of data, the device can also listen to the network for mobile terminated data. The assumption in this particular exampe is that transmission is prioritized over reception of mobile terminated data. Note that the network will have no information on when the device will be able to receive mobile terminated data. It may be a very long time (e.g. months / years) before the processor and communication module in an Ambient IoT device are activated in the intermittend power scenario.

On demand power triggered

The on demand power triggered scenario is a special version of the intermittent power scenario. Similarly to the intermittent power scenario, in the on demand power triggered scenario the device only has power available intermittently. The difference is that in the on demand power triggered scenario, there is a service user / application owner (e.g. the factory owner) that can determine when to provide power and activate the device or devices.

A typical power source for on demand power triggered is an RF power source that can be switched on/off, but also other power sources can be controlled (e.g. light / sound). From a service point of view, the type of power is not relevant, but it may be a relevant service aspect that a service user / application owner can control when power is provided.

An exampe on demand power triggered scenario is shown in figure 3.

Figure 3: on demand power triggered scenario (example)

In this example a very basic device is considered e.g. in a logistics tracking/tracing use case. The device is completely inactive when there is no power provided. The service user (e.g. the factory owner) can decide when it wants to activate the devices. E.g. in a wharehouse, this may be done once every night to keep inventory.

When power is provided the communications module is activated and, in this example, the device transmits an identifier to indicates its presence. Note that also more elaborated device actions are possible.

In this very simple example, the device does not listen to the network for mobile terminated data. This will have a service impact on e.g. provisioning and other life cycle actions (e.g. terminating the Ambient IoT device).

2. Reason for Change

Power scenarios have an important impact on service aspects of Ambient IoT. Therefore it is important to identify and agree on a classification of the different power scenarios.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.840. An alternative is to include the full text (including the figures in the annex).

* * * First Change * * * *

# Annex A (Informative):
Ambient IoT availability scenarios

The Ambient IOT devices foresee to have different kind of communication pattern that could be dependent on power available for communication e.g harvesting and the availability of storage capability or the specific use case. Following pattern are foreseen:

-	Normal operation: In this scenario, Ambient IoT devices have power available continuously or at least for signicant amounts of time, either because there is continuous power harvesting or possibly in combination with limited energy storage (e.g. in a capacitor) to overcome momentary variations in power harvesting. The main effect of this scenario is that the processor and communications module in the Ambient IoT device can be continuously active. The communications module can listen to network at regular intervals to determine if there is mobile terminated traffic (e.g. trigger messages) and can transmit data when relevant.

-	Device triggered operation: In this scenario, devices have power available only intermittently. The main effect of this scenario is that the Ambient IoT device can only be active for the short periods of time. And it is the ambient IOT device that decides when to communicate with the network. It is possible that the Ambient IoT device is not  able to listen to the network for mobile terminated traffic for very long periods of time. This has an impact on service aspects such as provisioning.

-	On demand operation: in this scenario 5G network will trigger the device to communicate in a relevant manner. This scenario only considers the network to trigger the communication and the ambient IOT device cannot determine when to communicate. Activation of the Ambient IoT device can imply a trigger to perform a specific action (e.g. do measurement) or to communicate (e.g. send an identifier). Activation can also imply to start listening to the network for a trigger message.

These scenarios are not intended as definitions for device categories. The purpose is to enable discussion of service aspects that are associated with different scenarios.

Editor’s Note: FFS to check and possibly find alternative terminology for triggering and activation