3GPP TSG SA WG 1 Meeting #99e 	S1-222379

Electronic Meeting, 22 August – 1 September 2022	(revision of S1-22xxxx)

Source:	KPN

pCR Title:	Traffic scenario for dairy cow stable

Draft Spec:	3GPP TR 22.840v0.1.0

Agenda item:	7.3

Document for:	Approval

Contact:	toon . norp@ tno .nl

Abstract: <provide a short description of the content>

1. Introduction

This traffic scenario provides an example of a continuous power scenario, where the sensor has continuous power available based on harvesting movement and temperature of the cow.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 22.840v0.1.0.

* * * First Change * * * *

## 6.2	Traffic scenario cow stable

### Description

In the dairy industry, there is an increase in the scale of farms. Whilst the number of farms decreases the number of cows per farm increases. The largest dairy farms in the US have over 15000 cows. A more typical dairy farm has around 200 cows.

Figure 6.2-1: A typical dairy farm

Because of the increase of the number of cows, there is a lot of automation in dairy farming. For example cows are milked using a milk robot, and manure is removed with robots.

Figure 6.2-2: Dairy farm automation with milk robot (left) and manure robot (right)

In order to identify individual cows (e.g. in the milk robot), the cows have tags. These tags also perform measurements of the cows vital signs (e.g. temperature, movement) to e.g. determine the fertility cycle of the cow and monitor health.

Figure 6.2-3: Cow sensor (sensor is just behind the ear)

It is also possible to create a cow sensor in the form of a capsule (or bolus) that can be swallowed by cows and remains in the first of the cow’s four stomachs. The advantage of this is that the sensor is smaller, providing more freedom to the cow, and that more internal data from within the cow can be measured.

Many cows spend their whole life in the stable. However, data in the Netherlands shows that more than 80% of cows also can graze outside in the meadows around the farm.

### Assumptions

We assume a typical farm with 200 cows. Every cow is equipped with a sensor.

The dimensions of the stable varies with the interior design of the stable, but 30 m x 60 m is a realistic dimension. For simplicity we assume that cows can go everywhere within the stable. The assumption is that there is an indoor base station on the ceiling in the middle of the stable.

The sensors provide measurement data every hour, but can also provide immediate alerts in case the cow is in distress (e.g. during calving). Assumption is that each measurement (identifier, plus 4 measured values) can be provided within 500 bits.

The cow sensor can harvest energy from the movement and temperature of the cow. A continuous power scenario is assumed where the sensor continuously has power available.

Maximum range for the sensor to base station is approximately 35 meters. Note that when the cows are outside, the distance to outside base stations can be much larger (kilometres). It is not assumed that the sensors can communicate over such long range. Even though the sensor may detect the base station, it should not attempt to connect to the base station when it is out of range.

A typical size of a capsule / bolus for cows is approximately 30 mm by 100 mm. For smaller ruminants, such as sheep or goats, this size is not suitable and smaller capsules will have to be made.

### Potential Functional Requirements

Ambient IoT devices shall not transmit (creating interference) when the network is outside communication range.

### Potential Key Performance Requirements

[PR 6.1-001] The 5G system shall be able to provide Ambient IoT service with the following KPIs:

Table 6.1-1: KPIs for dairy stable scenario

| Scenario | Communication Service Availability | Message Size | User experienced Data rate | Communication range | Device density |  | Service area dimension |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Cows in dairy stable | [99,9%] | [500] bits (note 1) | [5] kbit/s | [35] m Indoors | [< 1] /km2 (note 3) |  | [1 800] m2 (note 4) |
| NOTE 1: An identifier and four measurement values (e.g. temperature, movement, …). NOTE 2: This value is calculated as the instant data rate for transmitting 500 bits within 100 ms transmission time. The need for data transmission is infrequent (e.g. once per hour). NOTE 3: 200 cows in the stable NOTE 4: Assuming a 30 m x 60 m stable. |  |  |  |  |  |  |  |