3GPP TSG SA WG 1 Meeting #99e 	S1-222030r3

Electronic Meeting, 22 August – 1 September 2022	(revision of S1-222zzz)

Source:	Samsung, Intel

pCR Title:	22.837 pCR - Transparent Sensing Use Case

Draft Spec:	3GPP TR 22.837-010

Agenda item:	7.2

Document for:	Approval

Contact:	                     Erik Guttman  <erik.guttman@samsung.com>

Suresh Srinivasan <suresh.srinivasan@intel.com>

Abstract: Introduces a use case to TR 22.837 for sensing that is received by the 5GS itself.

1. Introduction

By opaque sensing we describe sensing data sent 'over the top' such that the 3GPP system is unaware of the information in the communication. Transparent Sensing is by contrast sensing data such that the 3GPP system is aware of the content in the communication. In location services, transparent sensing is the essential enabler for the service. This use case generalizes this concept to support transparent sensing to support diverse 3GPP services. One such service is

2. Reason for Change

Use case to consider the possibility of transparent sensing and the value possible through service enablers

3. Conclusions

<Conclusion part (optional)>

4. Proposal

r1		adds NR sensing to the use case, following comments by Alice. Corrects the style of the EN, spelling, grammar.

r2		includes changes as discussed on the CC on 23.08.22 for FS_Sensing

1/ remove the definition of opaque sensing

2/ align the terminology with ‘sensor measurement’ and ‘sensor result’

3/ update the title to be ‘Use Case on…’

4/ add an EN for the requirement for text on ‘surfaces’

It is proposed to agree the following changes to 3GPP TR 22.837 v010.

First Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	W. Favoreel, "Pedestrian sensing for increased traffic safety and efficiency at signalized intersections," 2011 8th IEEE International Conference on Advanced Video and Signal Based Surveillance (AVSS), 2011, pp. 539-542, doi: 10.1109/AVSS.2011.6027406.

[3]	Advances in Wildlife Crossing Technologies: https://highways.dot.gov/public-roads/septoct-2009/advances-wildlife-crossing-technologies.

[4]	Protection Detection: Making Roads Safe for Drivers and Wildlife: https://onlinepubs.trb.org/onlinepubs/webinars/201118.pdf.

[5]	F. Liu et al., "Integrated Sensing and Communications: Towards Dual-functional Wireless Networks for 6G and Beyond," in IEEE Journal on Selected Areas in Communications, doi: 10.1109/JSAC.2022.3156632.

[6]	T. S. Rappaport, G. R. MacCartney, M. K. Samimi and S. Sun, "Wideband Millimeter-Wave Propagation Measurements and Channel Models for Future Wireless Communication System Design," in IEEE Transactions on Communications, vol. 63, no. 9, pp. 3029-3056, Sept. 2015, doi: 10.1109/TCOMM.2015.2434384.

[7]	C. Han, Y. Bi, S. Duan and G. Lu, "Rain Rate Retrieval Test From 25-GHz, 28-GHz, and 38-GHz Millimeter-Wave Link Measurement in Beijing," in IEEE Journal of Selected Topics in Applied Earth Observations and Remote Sensing, vol. 12, no. 8, pp. 2835-2847, Aug. 2019, doi: 10.1109/JSTARS.2019.2918507.

[8] 		Liu, A., Huang, Z., Li, M., Wan, Y., Li, W., Han, T.X., Liu, C., Du, R., Tan, D.K.P., Lu, J. and Shen, Y., 2022. A survey on fundamental limits of integrated sensing and communication. IEEE Communications Surveys & Tutorials, 24(2), pp.994-1034.

[11] 		https://medium.com/desn325-emergentdesign/s-l-a-m-and-optical-tracking-for-xr-cfabb7dd536f

[12] 		Dwivedi, S., Shreevastav, R., Munier, F., Nygren, J., Siomina, I., Lyazidi, Y., Shrestha, D., Lindmark, G., Ernström, P., Stare, E. and Razavi, S.M., 2021. Positioning in 5G networks. IEEE Communications Magazine, 59(11), pp.38-44.

[M]	3GPP TR 22.856, "Localized Mobile Metaverse Services".

[N]	3GPP TR 29.238, "Uplink Streaming".

[Q]	“Velodyne™ LiDAR VPL-16 User Manual,” 63-9243 Rev. E, Velodyne™ LiDAR, https://velodynelidar.com/wp-content/uploads/2019/12/63-9243-Rev-E-VLP-16-User-Manual.pdf  <accessed 9.8.22>

Next Change

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

sensing measurement: obtaining sensing measurement data about a target object.

sensing result: the information about a target object after processing, such as being present and object dimension, which is related to a particular sensing service.

transparent sensing: sensing measurements are communicated such that they can be discerned and interpreted by the 5GS, e.g. the data is communicated using a standard protocol to an interface defined by the 5GS.

Next Change

## 5.Q	Use Case on Transparent Sensing Use Case

### 5.Q.1	Description

In general, a UE senses using either or combination of the sensors such as camera, NR-based sensing and Lidars, Radars. In NR-based sensing, the UE and BS senses for stationary and moving Objects around UE – using time-difference-of-arrival (TDoA), angle-of-arrival (AoA), angle-of-departure (AoD) measurements, RSSI etc.. as shown in Figure 5.Q.1-1 [8]. Transparent sensing is a use case in which sensing data is captured by the UE and communicated so that the 5GS is aware of the sensing information. From this information, service enablers can be defined. One example of such information is location data, whose corresponding service enabler is Location Based Services.

Figure 5.Q.1-1: BS and UE sensing Objects [8]

In this use case, sensing data is made available to the 5GS, and the requirements for this exposure are considered. The data so obtained can be used for diverse purposes. One such purpose is Localization (identifying both a three dimensional position and orientation.) Transparent Sensing data used for Localization is described in TR 22.856 [M]

Figure 5.Q.1-1: Opaque and Transparent Sensing Data

The distinguishing characteristic of this use case is that the sensing data is provided to the 5GS itself, not merely by the 5GS to an AF out of scope of 3GPP standardization.

### 5.Q.2	Pre-conditions

A UE has access to one or more sensors. In this use case. the UE has access to four sensors: NR-based sensing, 3D LiDAR, an RGB Camera and a Smart Phone Camera. The sensors' physical configuration is known (e.g. the cameras are 10 cm apart). The NR-based sensing capabilities of the UE and its connected BS are used to capture information about the nearby environment by the UE.

A mobile network MN supports the acquisition of sensing data. We term this support by the network a 'sensing data sink.'

Editor's Note: A sensing data sink is a stage 2 notion. What can we instead write? The 5GS supports a service that acquires sensing data to either process it and expose the processed information?

### 5.Q.3	Service Flows

The user U activates a mechanism to enable NR-based sensing at UE and MN and provide sensing data to the 5GS. This process is analogous to activating or enabling a location tracking service.

MN acquires sensing data provided by U's UE, for a period of time.

The user U deactivates the mechanism to provide sensing and sensing data to the 5GS

### 5.Q.4	Post-conditions

The sensing data acquired by the 5GS is processed in order to enable other services. The processed information can for example provide 'Spatial Localization' information that can be exposed to authorized third parties, as discussed in 22.856 [M]. "Spatial Localization Use Case".

### 5.Q.5	Existing feature partly or fully covering use case functionality

Positioning in 5G Networks been proposed in 3GPP release-16, it specifies positioning signals and measurements for the 5G NR. In release-16, 5G Positioning architecture extends 4G positioning architecture by adding Location Management Function (LMF) and Transmission reception points (TRP).  5G- along with enables- provides new positioning methods based on multi-cell round-trip time measurements, multiple antenna beam measurements, multiple to enable downlink angle of departure (DL-AoD) and uplink angle of arrival (UL-AoA) [11, 12, 13]. Current 5G system supports positioning of the device-based but not device-free – Objects that do not radiate EM signals. [8, 11, 12]

The 5GS already supports transport of sensor data. The table below provides indicative performance requirements for media used for sensor information communication.

| Sensor Type | Uplink KPI | Remarks |
| --- | --- | --- |
| 3D Lidar | 30 Mbps | An example 3D LiDAR: 16 channel, 0.3M data points, dual return mode 2 bytes distance, 1byte [Q] |
| Industrial RGB Camera | 16 ~ 800 Mbps | 2,592 x 2,048 x 10bits x 2.5 Hz x 6 EA, compression ratio 2% |
| Smart Phone Camera | 4 ~ 200 Mbps | 2,160 x 2,880 x 8bits x 1 Hz x 4 EA, compression ratio 2% |

Figure 5.Q.5-1: Performance Requirements (already possible to fulfill with the 5GS)

### 5.Q.6	Potential New Requirements needed to support the use case

[PR.5.Q.6 - 001] Subject to user consent and national or regional regulation, based on operator policy, the 5GS shall support a mechanism to receive uplink sensing measurements from a UE that is authorized to provide uplink sensing measurements to the network.

NOTE: 	This requirement assumes there is some functionality in the 5GS to discern and interpret the acquired sensing measurements.

[PR.5.Q.6 - 002] Subject to user consent and national or regional regulation, based on operator policy, the 5GS shall support a mechanism to expose information derived from processed sensing measurements to authorized third parties.

[PR.5.Q.6 - 003] Subject to user consent, network operator policy and national or regional regulation, the 5GS shall support a mechanism to enable the UE to perform NR-based sensing to capture information about the nearby environment.

[PR.5.Q.6 - 004] Subject to user consent, network operator policy and national or regional regulation, the base station of the 5GS shall support a mechanism to sense information about UE’s nearby enviroment.

[PR.5.Q.6 - 004] The sensing of BS and UE shall meet the following KPI: (Common Table for FFS)

End of Changes