3GPP TSG SA WG 1 Meeting #99e 	S1-222033r1

Electronic Meeting, 22 August – 1 September 2022	(revision of S1-222zzz)

Source:	Samsung

pCR Title:	Pseudo-CR on Use Case of Spatial Anchor Enabler

Draft Spec:	3GPP TR 22.856-010

Agenda item:	7.4 (FS_Metaverse)

Document for:	Approval

Contact:	Erik Guttman

erik.guttman@samsung.com

Abstract: This pCR contains a new use case for the FS_Metaverse study that proposes a new standardized service enabler for spatial anchoring of AR services and content.

1. Introduction

This pCR contains a new use case for the FS_Metaverse study that proposes a new standardized service enabler for 3GPP to allow spatial locations to be associated with services and content, and for that association to be accessed by those who notice and then access AR services and content.

2. Reason for Change

This pCR introduces a new use case for spatial anchoring.

3. Conclusions

Non-applicable.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.856-010.

r1	contains the following changes

- correction of typos (thanks Lei for pointing them out)

- addition of an FFS for privacy for requirement 2

Begin Changes

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

spatial anchor: an association between a location in space (three dimensions) and service information that can be used to identify and access services, e.g. information to access AR media content.

Service information: this information is out of scope of standardization but could contain, e.g. a URL, media data, media access information, etc. This information is used by an application to access a service.

Next Change

## 5.A	Spatial Anchor Enabler Use Case

### 5.A.1	Description

In use case 5.1 "Localized Mobile Metaverse Service Use Case," we introduced the term spatial anchor to describe an association between space and service information. This use case elaborates the concept of the spatial anchor to enable diverse metaverse services, including those described in use case 5.1. This use case focuses on the creation, management and use of spatial anchors.

The overall purpose of this enabler is to make it possible to create AR content and share it with others. The 'spatial anchor producer' determines what to share and its location, and any constraints (e.g. who to share the spatial anchor with) and additional information, most importantly the 'resource' associated with the anchor (e.g. AR media, service to access, etc.).

The 'spatial anchor consumer' is able to recognize anchors associated with locations, and use the spatial anchor to obtain the associated information.

Functionally, a spatial anchor has the following model:

- 	Spatial Anchor: information that can be provided by a content producer to a content consumer. How this is done is out of scope of this use case.

- 	Precise spatial location information: where the produced content is located, including the content's extent, orientation, etc.

- 	Service information: this information is out of scope of standardization but could contain, e.g. a URL, media data, media access information, etc. This information is used by an application to access a service.

The spatial anchor enabler will benefit retail environments. Here, a cheese seller has extensive information about her wares that she will share with customers.

Figure 5.A.1: Spatial Anchors - created by a user to share with other users

In this use case there are several actors that are relevant.

- 	Leka the cheese seller is the content producer. She creates content and also anchors them to her wares. (She is very adept at putting the cheese back in the same places, and moving the anchors when this is not possible.)

- 	I am the customer.

- 	Warez the application service provider stores and updates Leka's content, generates its presentation, supports any interactive features, etc.

-	FineNet is the network operator that enables anchored services for any content producer, customer and application service provider.

### 5.A.2	Pre-conditions

Leka makes use of a UE that has a subscription with FineNet. She has a sensor that can be used in combination with the UE to indicate precise locations. In figure 5.A.1-1, the sensor can identify the location of the tip of the cheese fork she holds.

Leka obtains services from Warez, where she stores information related to her inventory. She also has arranged in advance what information to display to customers and in what format.

I wear glasses that provide AR experience and communicate by means of my UE. I have a mobile subscription with FineNet also.

### 5.A.3	Service Flows

Leka places wares on display. She indicates the location of wares that are associated with inventory information so that the association of location (listed below as 'pose' including position in 3 dimensional space, orientation and possibly more spatial data) and service information are captured by the 5GS. This inventory information is captured also by the mobile network operator as spatial anchors. This is shown on the top half of Figure 5.A.3-1.

Figure 5.A.3-1: Spatial Anchor Enabler Service Flow

NOTE:	The text in parentheses in the figure above are examples.

1. Creating, modifying, removing spatial anchors

To add new wares to display, Leka captures the location of the item with her cheese for (which includes sensors.) She associates a new spatial anchor with this location and product information (e.g. by scanning the bar code on the cheese). The Warez inventory management system also generates AR content on demand: this application is external to 3GPP standards. Leka's UE accesses the mobile service that establishes the association that comprises the spatial anchor, the physical location and the service information.

When Leka moves wares, or the wares are sold, Leka can adjust the spatial location of items or remove them entirely (e.g. when the cheese has been sold) by registering the new location or removal with the Warez inventory management system. The 'location' and 'service information' can be updated over time.

Leka can also update the information that will be displayed as AR to the customer by the media server offered by the Warez inventory management system, for example the description of the cheese, its price, etc. This interaction is out of scope of 3GPP standards.

2. Accessing and using Spatial Anchors

I enter the store and examine what is on display. I capture the scene with sensors and share this information with the Spatial Mapping and Localization Service Enabler (5.X). This allows me to identify my localization information, including orientation, precisely.

I put on (activating) my AR glasses. By means of my UE with access over FineNet, share the location and orientation information (the area of interest) with the 5G system.

The 5G system uses the localization information to identify all applicable spatial anchors in the area of interest. These are returned to the UE and the AR glasses. This function is illustrated in the lower half of Figure 5.A.3-1.

The service information suffices to access the media server offered by Warez. The location information indicates the location of each spatial anchor.

I now perceive the spatial anchors as shown in Figure 5.A.1.

As Leka indicates the Halloumi in her counter, and my gaze focuses on that location (known to a very high degree of precision), the AR glasses use the service information associated with the spatial anchor to activate the Halloumi cheese media.

I now perceive the AR information panel associated with the Halloumi cheese.

### 5.A.4	Post-conditions

I am able to observe the AR content associated with the wares on display, as shown in Figure 5.A.1-1. As wares are moved or removed from the display, the content shifts as well. The display of AR content is the result of the service information (i.e. how to access the media) and the location information (i.e. where the media is placed, oriented, etc.)

At any time Leka can add new wares and associated AR content, update the content that is displayed, etc.

I perceive AR content associated with the items in the shop and happily buy the cheese that meets my needs.

### 5.A.5	Existing feature partly or fully covering use case functionality

### 5.A.6	Potential New Requirements needed to support the use case

[PR-5.A.6-1]	Subject to operator policy, the 5G system shall enable an authorized third party to establish an association between a physical location (in three dimensional space, an orientation, etc.) and service information, where the service information is provided to the 5G system and the spatial anchor is either provided or determined by the 5G system.

[PR-5.A.6-2]	Subject to operator policy, the 5G system shall enable an authorized third party to obtain all of the spatial anchors located in a given three dimensional area.

NOTE 1:	How the authorized third party identifies which three dimensional area to request spatial anchors in is not in scope of the 3GPP standard. Spatial localization and mapping information could be used to identify areas of interest.

[PR-5.A.6-3]	Subject to operator policy, the 5G system shall enable a third party to request the service information associated with the precise location of a specific spatial anchor. Making use of this service and location information, the third party can access an application server to obtain AR media.

NOTE 2:	How the service and location information is used by the third party to access an application server and the AR media itself is out of scope of this requirement.

Editor's Note: It is FFS whether some spatial anchors and associated spatial location and service information may be considered 'confidential' and require specific handling to maintain the privacy of the authorized third party who added the spatial anchor.

[PR-5.A.6-4]	Subject to operator policy, the 5G system shall provide an authorized third party a means to manage the spatial anchor(s), e.g. add, remove or modify spatial anchors, determine privacy and security aspects, etc.

End of Changes