3GPP TSG SA WG 1 Meeting #99e 	S1-222034r3

Electronic Meeting, 22 August – 1 September 2022	(revision of S1-222zzz)

Source:	Samsung, Interdigital

pCR Title:	Pseudo-CR on Spatial Mapping and Localization Service Enabler Use Case

Draft Spec:	3GPP TR 22.856-010

Agenda item:	7.4

Document for:	Approval

Contact:	Erik Guttman

erik.guttman@samsung.com

Abstract: This P-CR proposes a use case for a new 'spatial localization' service enabler.

1. Introduction

Please consider the use case below.

2. Reason for Change

This is a use case for the FS_Metaverse study, applying concepts of environment mapping which is in some ways analogous to location based services

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.856 version 0.1.0.

r1		changes

- corrected the reference

- corrected the definition of Localization

- added definition of Spatial Mapping Service, and Spatial Localization Service

- added a NOTE and EN to requirement 1 to clarify

- added clarification to the section describing mapping

- changed wording of the requirements to align with the new terminology

r2		changes

- a word was added in 5.A.1 'spatial mapping considerations' for clarity

r3		changes

- add Interdigital as a supporting company

- [ZTE] use correct sensing requirements terminogy, add clarification and references to 23.837

- [ZTE] explain why the output of Spatial Localization Service and Spatial Mapping Service may be an authorized 3rd party (in NOTEs)

- [Interdigital] Add as supporter

- [Interdigital] Reword text in various parts of the document to remove normative language and clarify intent

- [Interdigital] reformatting for clarity, fix a typographic error

- [Interdigital] add a reference to 23.837

First Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.228: "Service requirements for the Internet Protocol (IP) Multimedia core network Subsystem (IMS)".

[3]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services".

[4]	3GPP TS 22.101: "Service principles".

[5]	3GPP TS 22.261: "Service requirements for the 5G system".

[6] 	Talaba D, Horvath I, Lee K H, "Special issue of Computer-Aided Design on virtual and augmented reality technologies in product design," Computer-Aided Design 42:361 - 363.

[7] 	Wang X, Tsai J J-H, "Collaborative Design in Virtual Environments," In: ISCA 48:1-2.

[8] 	Benford S, Greenhalgh C, Rodden T, "Collaborative virtual environments." Communications of the ACM 44: 79–85.

[9]	M. Eid, J. Cha, and A. El Saddik, "Admux: An adaptive multiplexer for haptic-audio-visual data communication", IEEE Tran. Instrument. and Measurement, vol. 60, pp. 21–31, Jan 2011.

[10]	K. Iwata, Y. Ishibashi, N. Fukushima, and S. Sugawara, "QoE assessment in haptic media, sound, and video transmission: Effect of playout buffering control", Comput. Entertain., vol. 8, pp. 12:1–12:14, Dec 2010.

[11]	N. Suzuki and S. Katsura, "Evaluation of QoS in haptic communication based on bilateral control", in IEEE Int. Conf. on Mechatronics (ICM), Feb 2013, pp. 886–891.

[12]	E. Isomura, S. Tasaka, and T. Nunome, "A multidimensional QoE monitoring system for audiovisual and haptic interactive IP communications", in IEEE Consumer Communications and Networking Conference (CCNC), Jan 2013, pp. 196–202.

[13]	A. Hamam and A. El Saddik, "Toward a mathematical model for quality of experience evaluation of haptic applications", IEEE Tran. Instrument. and Measurement, vol. 62, pp. 3315–3322, Dec 2013.

[14]	O. Holland et al., "The IEEE 1918.1 "Tactile Internet" Standards Working Group and its Standards," Proceedings of the IEEE, vol. 107, no. 2, Feb. 2019.

[G]	Lee, Donghwan et. al., “Large-scale Localization Datasets in Crowded Indoor Spaces,” NAVER LABS, NAVER LABS Europe, CVPR 2021.

[H]	3GPP TR 22.837, " Study on Integrated Sensing and Communication."

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

Next Change

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Spatial Map: A collection of information that corresponds to space, including information gathered from sensors concerning characteristics of the forms in that space, especially apperance information.

Localization: A known location in 3 dimensional space, including an orientation, e.g. defined as pitch, yaw and roll.

Spatial Mapping Service: A service offered by a mobile network operator that gathers sensor data in order to create and maintain a Spatial Map that can be used to offer customers Spatial Localization Service.

Spatial Localization Service: A service offered by a mobile network operator that can provide customers with Localization.

Next Change

## 5.A	Spatial Mapping and Localization Service Enabler Use Case

### 5.A.1	Description

For the localized mobile metaverse use case 5.1, the service provider or operator needs to provide and use spatial map information, i.e. a 3D map of indoor or outdoor environment. This use case considers how a spatial map can be created and employed, both as service enablers. The creation and maintenance of the spatial map is referred to as Spatial Mapping Service and the employment of the map to identify the customer's Localization is termed Spatial Localization Service.

Figure 5.A.1-1: Spatial Location Services Enabler

Specifically, this use case proposes two spatial localization service enablers.

1)	Spatial Mapping

Sensor data gathered transparently is processed in order to identify the static and transient forms. For example, in Figure 5.A.1-1

(A), the Rubens room in the Louvre, sensor data captures information.

(B) This information is processed to identify the static and transient information, to establish an up to date spatial map.

(C) In combination with location information (for the sensors) and other information (e.g. architectural specifications of the Louvre), a spatial map can be achieved in which not only the forms but also their locations in 3D space are known.

2)	Localization

Given that a spatial mapping exists,

(A) sensor data for a user (from devices communicating by means of a UE) can be captured,

(B) compared with the spatial map,

(C) used to identify in 3D space the position, viewing direction, angle, etc. of the user.

Spatial Mapping Considerations

Examples when spatial mapping could be useful:

- 	A government conducting a digital city project can build a 3D spatial map of outdoor environment of an entire city or public spaces such as outdoor parks or indoor offices of their government building.

- 	A navigation service provider (or Spatial Localization Service) can build a 3D spatial map of outdoor environment of entire roads or public spaces. An operator’s partner or operator also can build a 3D spatial map of outdoor environment.

-	A customer wanting mapping of their indoor environment, e.g. the interior of a commercial space such as the cheese shop as described in 5.X.

It is a hard task to perform the mapping of the entire city using a vehicle by traversing various roads and spaces. It also requires lots of time and effort (for data conversion, etc.) if the work is performed in offline. If the multiple capturing devices are used in parallel, the spatial map data in the same location could be synthesized over the different cameras and input devices to generate the spatial map.

The mobile capturing device, vehicle or robot equipped with multiple stereo/mono RGB cameras and multiple LiDAR to capture various qualities of images and depth information of the environments. As an example in [G], a mobile indoor robot is equipped with two LiDARs, 6 industrial cameras and 4 smartphone cameras. Based on the example, we can derive the uplink bandwidth requirements for one mobile indoor mapping robot. There is a corresponding use case for 'transparent sensing' in TR 22.837.

Editor's Note: Provide a complete reference to 22.837 [H] if / once S1-222030 is implemented.

Localization Considerations

The environment mapping can be used for providing a visual positioning service, for enhancing the accuracy of location service or for helping the metaverse contents management system for spatial internet.

In this use case, the UE provides uplink sensor information that can be interpreted, along with a spatial map, to identify Localization. This is analogous to the process used by the UE to provide sensor data that the 5G system can use to determine UE location for Location Services.

### 5.A.2	Pre-conditions

For Spatial Mapping

Mobile operator or service provider can determine the target area for the spatial mapping. The target area can be divided into multiple areas where each can be mapped with multiple capturing devices (e.g. capturing indoor robot, vehicles).

Each capturing device is equipped with multiple sensors, e.g. mono or stereo RGB camera or one or more multiple LiDAR cameras. The intrinsic parameter of cameras equipped with the device are pre-configured in the device.

Each capturing device has a capability with high resolution positioning system, the positioning technique also can derive the altitude information.

An indoor robot can be equipped with non-3GPP based high resolution positioning system such as UWB. To derive the exact the position of the capturing device, each capturing device also can utilize the structure-from-motion technologies.

The capturing devices communicate via a UE that can access the mobile network of the MNO that supports spatial mapping. The capturing devices can either sense the spatial area to be mapped, or move about sufficiently that the capturing devices can be used to acquire sensor information corresponding to the area.

For Localization

A set of sensors are accessible by a UE. These could be built into the terminal equipment or communicate with it in some way that is out of scope of this use case, e.g. using a cable, a personal area network, etc.

The UE can access a mobile network of MNO M that offers localization services.

5.A.3	Service Flows

For Spatial Mapping

1.	A capturing mobile device attaches to the mobile network and becomes authorized to deliver sensor information for the purposes of spatial mapping.

2.	The capturing mobile device starts the mapping operation.

a.	The capturing mobile device arranges for its sensors to provide information as needed by the 5G system. This could require some configuration of the capturing mobile device, e.g. the sensors or to control the uplink communication of sensor information.

3.	The capturing mobile device navigates, e.g. with a pre-defined route, within the selected target region in order to provide a sufficiently complete set of sensor information for the space to be mapped.

4.	The capturing mobile device uploads the captured RGB camera images and LIDAR depth images to the 5G system with the positioning information of the mobile device.

5.	The mapping server collects and analyzes the information provided to cumulative create or update a spatial map.

For Localization

1.	A mobile device requiring localization attaches to a mobile network and becomes authorized to obtain Localization services.

2.	The mobile device uses sensors to capture information corresponding to the point and direction that must be localized.

3.	The mobile device sends via uplink communication the sensor information to the 5G system.

4.	The 5G system uses the sensor information and a spatial map to determine the localization, that is, the corresponding positioning information and sensor pose.

5.A.4	Post-conditions

For Spatial Mapping

The spatial mapping enabler re-structures the data provided by mobile capture devices to create a spatial map. This information could be combined with other information available, e.g. floor plans of buildings, survey data, satellite images, etc. Over time sufficient information will be captured to allow the spatial mapping enabler to distinguish between static and dynamic objects in the environment.

For Localization

The result of the localization service is available as a service enabler. This could be provided to the UE for use by applications or exposed to a third party by means of an API by the 5GS. This information can be used for many purposes, especially for media based applications that require localization to control the rendering of AR or MR content. Localization can be done over time, e.g. to track a user's movement.

The localization enabler can enhance location services as it can identify with some degree of precision a location by means of diverse sensors compared with known location data in the spatial map.

### 5.A.5	Existing feature partially or fully covering use case functionality

Location Services provide location information which can be used to support services that are triggered or informed by the user's location.  This information is not at the degree of accuracy needed for some applications, e.g. AR. Location Services also do not work in all environments, e.g. indoor.

The 5GS supports uplink media and uplink sensor data communication.

### 5.A.6	Potential New Requirements needed to support the use case

#### 5.A.6.1	Requirements for Spatial Mapping

[PR5.A.6.1-1]	Subject to consent by the owner of the space to be mapped and relevant regional and national regulation, the 5GS shall support mechanisms to authorize Spatial Mapping Service.

NOTE: 	Some locations are not appropriate to map, e.g. for regulatory reasons, or where they require user consent. This is the reason for the requirement for 'consent by the owner of the space to be mapped' condition in the above requirement. For example, secure facilities such as prisons, military bases, etc. would never consent to be mapped. On the other hand, commercial spaces such as markets would have strong incentive to consent to be mapped.

Editor's Note: It is FFS whether and how an MNO can identify whether Spatial Mapping Service authorization is required, and how this can be obtained.

[PR5.A.6.2-2]	Subject to operator policy and relevant regional and national regulation, the 5GS shall support mechanisms for an authorized UE to provide sensor measurement data that can be used to produce or modify a spatial map.

[PR5.A.6.1-3]	Subject to operator policy, user consent and relevant regional and national regulation, the 5GS shall support mechanisms to receive and process sensor measurement data to produce or modify a spatial map.

Editor's Note: The corresponding use case 'Transparent Sensing' is added in TR 22.837 [H]. How to properly reference this use case is FFS.

[PR5.A.6.1-4]	Subject to operator policy and relevant regional and national regulation, the 5GS shall support mechanisms to expose a spatial map or derived localization information from that map to authorized third parties.

NOTE 1:	The spatial map and derived localization information supports services that produce AR and MR media, e.g. as described in clause 5.1.

Editor's Note:	The requirements for the precision of spatial positioning of the sensors that provide sensor data and the spatial map are FFS.

#### 5.A.6.2	Requirements for Localization

[PR5.A.6.2-1]	Subject to operator policy and relevant regional and national regulation, the 5GS shall support mechanisms to authorize Spatial Localization Service.

[PR5.A.6.2-2]	Subject to operator policy and relevant regional and national regulation, the 5GS shall support mechanisms for an authorized UE to provide sensor data that can be used to for Spatial Localization Service.

Editor's Note: The corresponding use case 'Transparent Sensing' is added in TR 22.837 [H]. How to properly reference this use case is FFS.

[PR5.A.6.2-3]	Subject to operator policy and relevant regional and national regulation, the 5GS shall support mechanisms to expose Spatial Localization Service information to authorized third parties.

NOTE 2:	The Spatial Localization Service information supports services that produce AR and MR media, e.g. as described in clause 5.1.

Editor's Note:	The requirements for the precision of spatial positioning of the sensors that provide sensor data and the spatial map are FFS.

End of Changes