3GPP TSG-SA WG1 Meeting #99e 	S1-222045r1

Electronic Meeting, 22 Aug - 1 Sep 2022	(revision of S1-22xxxx)

Title:	Use case of store and forward operation with discontinuous feeder link for 	delay-tolerant IoT - Inter-satellite

Agenda Item:	7.8

Source:	China Telecom

Contact:	Heng Wang (wangh26@chinatelecom.com)

Abstract: This document proposes a use case and related potential requirements to be included in  FS_5GSAT_Ph3 TR 22.865 version 0.0.0.

**************** First Change ******************

References

[x1]	3GPP TS 22.822: "Study on using Satellite Access in 5G ".

************* End of First Change ***************

**************** Second Change ******************

x.y	Store and forward operation with discontinuous feeder link for delay-tolerant IoT - Inter-satellite

x.y.1	Description

To expand the market of delay-tolerant IoT devices, store and forward operations are necessary to be developed to sustain the user plane data during the feeder link disconnection between the satellite and the terrestrial gateway. Based on the earlier studies[x1], there are many use cases can be further improved with such mechanisms.

Regardless of a simple scenario describing a relative static location relationship of a IoT device, a satellite and a terrestrial gateway, the serving satellite may change to another one during the time when the feeder link is unavailable. As shown in Fig.1, a mobile IoT device may move from the coverage of one satellite to the other(e.g. containers tracing and tracking) , or as shown in Fig.2, a NGEO satellite may fly away and the other one will come and turn to serving a static IoT device. Under such circumstances, the serving satellite may forward the stored user plane date to the next serving satellite through Inter-Satellite Links, and the next serving satellite may help forward the data to the gateway. Meanwhile, if the feeder link of the next satellite is also unavailable, it will continue the store operation until the recovery of its feeder link. In this way, for every single IoT device, there will be only one satellite for its data storage in the overall satellite system. And the mobile operators will be easier to manage and maintain the data rather than dealing with the separate data which is belong to one device but among different satellites.

Furthermore, considering the limited data storage in satellite and the large amount of IoT devices, a maximum storage for each IoT device should be pre-configured based on the application data characteristics, user subscriptions and overall performance of satellite communication system.

Fig.1 Serving satellite change during the feeder link disconnection - IoT device moving

Fig.2 Serving satellite change during the feeder link disconnection - satellite moving

x.y.2	Pre-conditions

A delay-tolerant device has a subscription with the terrestrial operator TerrA, and it is tagged on one container for tracing and tracking.

TerrA has agreement with the satellite operator SatA for satellite access.

SatA maintains multiple serving satellites for the satellite access of TerrA’s subscribers all over the world, including Adam and Bob.

The container will be shipped from the Harbour A to the Harbour B across the Pacific.

x.y.3	Service Flows

- 1. After the cargo ship leaves the Harbour A, the device can send some packets through the satellite access during the shipping time.

- 2. Due to some reasons, the feeder link between the serving satellite Adam and terrestrial gateway is interrupted or couldn’t be used for a time.

- 3. Based on the configuration of store and forward operations, Adam will go on to receive the packets from the device, and store these packets until the feeder link recovers.

- 4. However, during the period of feeder link is unavailable, the cargo ship approaches the border of coverage of Adam and will head to the coverage of another satellite Bob. So, based on the movement of the cargo ship, the serving satellite will change.

- 5. During the period of the change, Adam sends the stored packets to Bob through the inter-satellite link and Bob will forwards the packets to the gateway if its feeder link is available.

- 6. Particularly, Bob will continue storing the packets if its feeder link is also unavailable.

x.y.4	Post-conditions

1. Those packets will be finally sent to the application server by the network behind the gateway, e.g. transportation network, core network, internet. And the application will parse some information from the packets.

x.y.5	Existing features partly or fully covering the use case functionality

Editor's Note:	it is FFS whether other existing features will be identified.

x.y.6	Potential New Requirements needed to support the use case

[P.R.x.y-001] Subject to user consent and operator policies, a 5G system with satellite access shall be able to support store data received from delay-tolerance IoT devices while the feeder link is unavailable.

[P.R.x.y-002] Subject to user consent and operator policies, a 5G system with satellite access shall be able to support forward the stored data received from delay-tolerance IoT devices through Inter-Satellite Links.

[P.R.x.y-003] Subject to user consent and operator policies, a 5G system with satellite access shall be able to define the maximum amount of data storage per delay-tolerance IoT device in the overall satellite system.

Editor's Note:	it is FFS whether other potential new requirements or corresponding KPIs can be identified.

************* End of Second Change ***************