3GPP TSG-SA WG1 Meeting #99e 	S1-222049r1

Electronic Meeting, 22 Aug – 1 Sep 2022	(revision of S1-22xxxx)

Source:	China Unicom, Charter Communications, ZTE

pCR Title:	Pseudo-CR on Use Case of Service Continuity and QoS Requirements

Draft Spec:	3GPP TR 22.851 V0.1.0

Agenda item:	7.5

Document for:	Approval

Contact:	Qun Wei, weiqun5@chinaunicom.cn

Abstract: Introduce the potential service continuity and QoS requirements for the non-N2 shared network in TR22.851V0.1.0.

1. Introduction

The use case of non-N2 sharing network is an important use case in the network sharing. The contribution describes the  use case and potential requirements of service continuity and QoS for non-N2 sharing network scenario.

2. Use Cases

Update the “Use Cases” section 5 of the TR 22.851V0.1.0.

This use case was provided in the last meeting, but not captured in the TR. Instead an Editor’s NOTE was introduced which needs be resolved.

It is proposed to clarify the NOTE to add new service flows and define new requirements.

3. Conclusions

It is proposed to put the use case into the TR 22.851.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.851.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x1]	3GPP TS 22.261: "Service requirements for the 5G system".

[x2]	3GPP TS 22.101: "Service aspects; Service principles".

[x3]	3GPP TS 22.071: "Location Services (LCS); Service description; Stage 1".

[x4]	3GPP TS 29.513: "5G System; Policy and Charging Control signalling flows and QoS parameter mapping; Stage 3".

* * Next Change * * * *

5 Use cases

5.A 	Use case of Service Continuity and QoS

5.A.1	Description

The requirements to the services provided by the Participating Operator and the Hosting RAN operator for a UE moving between their service areas needs to be clearly defined. This service considerations are based on their user subscriptions and charging requirements from the Participating Operator. The service principle is not expected to be significantly different from the MOCN access sharing. The business here includes not only the operator’s name displayed in the UE UI, but also the service logics provided by both the Participating Operator and the Hosting RAN operator for the services, such as voice, SMS and data communications for the UE.

5.A.2	Pre-conditions

Assumptions,

1. OP 1 is the Hosting RAN operator of NG-RAN access network.

2. The core network of OP 2 does not have direct connection with OP1’s shared NG-RAN.

3. There is connection between the OP1’s CN and OP2’s CN.

4. UE 1 belongs to OP 1. UE 2 and UE N belong to OP 2.

5. OP3 is the Hosting RAN operator of NG-RAN and E-UTRAN. And OP3 and OP2 have a sharing agreement with MOCN.

6. OP2 is the emergency call service provider. OP2’s core network routes the emergency call to PSAP according to the location information transmitted by the shared network, such as Tracking Area Code.

5.A.3	Service Flows

OP1 IMS

network

OP1 network

with E-UTRAN

and shared NG-RAN

OP2 IMS

network

OP2 network

with E-UTRAN

and NG-RAN

connection between the OP1’s CN and OP2’s CN

UE 2 of

OP2

UE 1

of OP1

UE N

of OP2

Figure 5.A.3-1: Basic service scenario without direct connections between the shared access and the core networks of the participating operators

OP1 IMS

network

OP1 network

with E-UTRAN

and shared NG-RAN

OP2 IMS

network

OP2 network

with E-UTRAN

and NG-RAN

connection between the OP1’s CN and OP2’s CN

UE 2 of

OP2

UE 1

of OP1

UE N

of OP2

OP3 shared E-UTRAN

and shared NG-RAN

connection between the OP3’s RAN and OP2’s CN

Area 1

Area 2

Area 3

Figure 5.A.3-2: Emergency call routing with multiple sharing scenarios

1. UE 2 successfully registers to OP 2’s PLMN via the shared NG-RAN.

2. UE 1 successfully registers to OP 1's PLMN.

3. UE N successfully registers to OP 2's PLMN, via OP2’s wireless access network.

4. UE 2 initiates a 5G voice call to user N and succeeds under the shared network.

- 	OP 2 and OP1 do not need to expose the IMS network to each other, e.g., UE2 may also access services provided by their home environment in the same way even if the UE2 moves to the coverage of shared NG-RAN of OP1, and it is not necessary to assume that the IMS network of OP1 is involved in this scenario.

-	When UE 2 moves to OP 2's 4G area from the shared network, the call continues.

- 	When UE 2 moves to OP 2's 5G area from the shared network, the call does not drop.

- UE 1 initiates 5G voice call to UE N. The call succeeds. When UE 1 moves to OP 1's 4G network, the call does not drop.

- According to national regulations, a user accesses the shared network may also need to use emergency call in different sharing scenarios. For example, in the scenario of Fig. 5.A.3-2,

- UE 2 successfully registers to OP 2’s PLMN via the shared NG-RAN of OP1, and the geographical location of UE2 is in area 1. The shared network of OP1 may need to transmit the user location information (e.g., 11100F) to the core network of the OP2 as the service provider.

- At this time,OP2’s core network also maintain the location information related with non-shared E-UTRAN and NG-RAN of its own, for example a location information “1110” of its own E-UTRAN, which is very similar to “11100F”, but the corresponding geographic location of “1110” is in the area 2.

- In addition, OP2’s core network also have the location information of OP3’s shared network, because OP2 and OP3 have a sharing agreement with MOCN. The location information “1110D6” of OP3’s NG-RAN may be very similar to “11100F”. But the corresponding geographic location of the “1110D6” of OP3 is in the area 3.

- UE2 initiates an emergency call under the shared network of OP1, it is most reasonable for the core network of OP2 to route the emergency call to PSAP within area 1.

The challenges to the core network of OP2 arises, because it needs to choose one PSAP from the area 1, 2 and 3 to route the emergency call of the UE. (corresponding to PR 5.A.6-002)

- The network sharing partners may set a specific sharing allocation model for a network sharing method communicating with. It is necessary to understand the number of the users, and how long users using a certain shared network method will take. Thus the charging call records need to be synchronized between the partners’ network. The collection of charging information associated the sharing method that the UE accesses with shall be possible. (corresponding to PR 5.A.6-005)

- Service capabilities consist of bearers defined by QoS parameters and the mechanisms needed to realise services. Interoperability and interworking need to be ensured across different operators' networks according to different service capabilities. It is challenging to maintain the user experience for a UE using an active communication and moving between two networks communicating via no direct connection between the shared RAN and the Participating operator’s Core Network, because of different QoS, problem of QoS communication, different resource requirements for slices configured for industrial customers, and different implementation mechanisms for services (e.g., voice of VoNR or EPS Fallback, emergency call deployments). Consistent user experience with diverse service capabilities (e.g., QoS, QoE, network slice, and mechanisms needed to realise services) is indeed required to ensure interoperability and interworking. (corresponding to PR 5.A.6-004）

5.A.4	Post-conditions

The service of UE 2 succeeds, when UE 2 moves between shared access network and Participating Operator’s network.

5.A.5	Existing feature partly or fully covering use case functionality

SA1 has performed various studies on services in previous releases. For the definition of service continuity and the description of user experience see TS 22.261 [x1]. The description of service capabilities is stated in TS 22.101 [x2]. Some typical requirements for service in network sharing are:

The provision of services and service capabilities that is possible to offer in a network shall not be restricted by the existence of the network sharing It shall be possible for a core network operator to differentiate its service offering from other core network operators within the shared network.

It shall be possible to control the access to service capabilities offered by a shared network according to the core network operator the user is subscribed to.

The 3GPP System shall support service continuity for UEs that are moving between different Shared RANs or between a Shared RAN and a non-shared RAN.

Chapter 10 of TS 22.101[x2] defines the emergency call service. According to National regulations, the requirement of UE’s location of emergency call service is different in deployment scenarios, examples are:

National regulations may require wireless networks to provide the emergency caller's location. This requirement typically overrides the caller's right to privacy with respect to their location being revealed, but remains in effect only as long as the authorities need to determine the caller's location. The interpretation of the duration of this privacy override may also be different, subject to national regulation. For example, some countries require location to be available from the wireless network only while the call is up, while others may allow PSAP's to unilaterally decide how long the location must be made available.

Therefore, the requirement for providing location availability is to allow the network to support providing a mobile caller's location to the authorities for as long as required by the national regulation in force for that network.

Note:	See TS 22.071 [x3] for location service requirements on emergency calls.(TB check)

In addition to the charging requirements of 5G system introduced in Chapter 9 of TS 22.261 [x1], the following requirement is included in TS 22.101 [x2]:

Charging and accounting solutions shall support the shared network architecture so that end users can be appropriately charged for their usage of the shared network, and network sharing partners can be allocated their share of the costs of the shared network resources.

And Further Charging requirements related are specified in TS 29.513 [x4].

5.A.6	Potential New Requirements needed to support the use case

[PR 5.A.6-001] The 5G system shall be able to maintain the service continuity (e.g. for IMS services) for UEs moving between a Participating Operator’s E-UTRAN or NG-RAN, and the shared NG-RAN of a Hosting Operator, with indirect connections between the shared access and individual core networks.

[PR 5.A.6-002] Multiple sharing methods are expected to have minimum impact to the user experience of routing emergency call to PSAP.

[PR 5.A.6-003] It shall be possible for a core network operator to differentiate its services and service capabilities of one network sharing method offering from other network sharing method when communicating with the shared network.

[PR 5.A.6-004] The 5G system shall be able to ensure a consistent user experience, when the UE using an active communication moves between the networks of Participating and Hosting Operator’s with diverse service capabilities (e.g., QoS, QoE, network slice, and mechanisms needed to realise services).

[PR 5.A.6-005] The 5G core network shall support collection of charging information associated the sharing method that the UE accesses with.