3GPP SA WG1 Meeting #99e 	S1-222092r3

Electronic Meeting, 22 August – 1 September 2022	(revision of S1-222092)

# Source:	Peraton Labs, CISA ECD, T-Mobile US, Verizon, AT&T

# Title:	New WID on MPS for Messaging services

# Document for:	Approval

# Agenda Item:	4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: MPS for Messaging services

Acronym: MPS4msg

Unique identifier:

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No | X |  |  |  |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

Dependency on non-3GPP (draft) specification: None

# 3	Justification

In addition to Multimedia Priority Service (MPS) for MMTEL voice/video calls and MPS for DTS specified in TS 22.153, authorized MPS Service Users also need priority for messaging services in periods of network congestion during which normal commercial messaging services are degraded.

# 4	Objective

The objective is to define normative stage 1 requirements to support MPS for Messaging services.

The Rel-19 scope of MPS for Messaging is:

- End to end priority treatment of point to point messages and messages to/from the Message Service Center when MPS for Messaging is authorized and activated for an originating UE with a subscription for MPS using IMS Messaging for the messaging service,

- Priority treatment of messages to/from the Message Service Center when MPS for Messaging is authorized and activated for a UE or IoT device with a subscription for MPS using SMS or MMS for the messaging service, and

- End to end priority treatment or point to point messages and messages to/from the Message Service Center when MPS for Messaging is authorized and activated for an originating UE or IoT device with a subscription for MPS using MSGin5G for the messaging service.

NOTE  1:	MPS for Messaging may make use of commercial messaging service offerings provided by the operator. However, the messaging application services (e.g., Rich Communication Service (RCS) using IMS Messaging, SMS, and MMS) and inter system aspects are outside 3GPP scope.

NOTE  2:	End to end priority can be provided by the 3GPP system when both the originating UE and the destination UE are using IMS Messaging service.  When the originating UE is using IMS Messaging service but the destination UE is not using IMS Messaging service, priority treatment is provided by the 3GPP system only to/from the Message Service Center.

NOTE 3:	When the MPS subscribed UE or IoT device is using SMS/MMS, priority treatment is provided by the 3GPP system for the messages to/from the Message Service Center. The SMS and MMS platforms and inter system aspects are outside of 3GPP scope.

NOTE 4:	End to end priority can be provided by the 3GPP system when both the originating UE or IoT device and the destination UE or IoT device are using MSGin5G.  When the originating UE or IoT device is MSGin5G but the destination UE or IoT device is not using MSGin5G, priority treatment is provided by the 3GPP system only to/from the Message Service Center.

NOTE 5:	SMS options in scope are SMS via NAS and SMS over IP (i.e., SMS over MAP and SMS over SGs are not in scope).

NOTE 6:	MSGin5G is only applicable to 5GS.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 22.153 | Text modifications and addition of new requirements on MPS for Messaging | SA#98 |  |

# 6	Work item Rapporteur(s)

Singh, Ray, Peraton Labs, rsingh@peratonlabs.com

# 7	Work item leadership

SA1

# 8	Aspects that involve other WGs

None.

# 9	Supporting Individual Members

Supporting IM name

Peraton Labs

CISA ECD

T-Mobile US

AT&T

Verizon

DISH network