3GPP TSG SA WG 1 Meeting #99e 	S1-222097r1

Electronic Meeting, 22 August – 1 September 2022	(merging with S1-222107)

Source:	NTT DOCOMO, NTT, Huawei, CAICT

pCR Title:	Pseudo-CR on Use case of sensing for railway intrusion detection

Draft Spec:	3GPP TR 22.837 version 0.1.0

Agenda item:	7.2

Document for:	Approval

Contact:	Atsushi Minokuchi, minokuchi@nttdocomo.com

Yutaka Musaka, yutaka.musaka.nw@nttdocomo.com

Yoshifumi Morihiro, morihiro@nttdocomo.com

Shinya Otsuki, shinya.otsuki.ma@hco.ntt.co.jp

Tomoki Murakami, tomoki.murakami.nm@hco.ntt.co.jp

Fangyuan Zhu (zhufangyuan@huawei.com)

Abstract: This contribution proposes a new use case for FS_Sensing.

1. Introduction

Proper monitoring of railway environment is important to ensure railway safety. The early detection of possible dangers due to intrusion of unauthorized persons and animals on the railroad becomes essential. Traditional mechanism of railway monitoring using fibre optic sensing techniques is costly and requires manual intervention, making it very difficult to meet the increasing demand for railway monitoring.

Figure 1: Railway pedestrian intrusion [1]

Thanks to the 5G NR based sensing, the gNB as transmitter and receiver along the railway can constantly sense the railway situation such as railway intrusion.

2. Reason for Change

This contribution proposes a new use case about railway intrusion detection.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.837 version 0.1.0.

4. Reference

1.       Guo B, Shi J, Zhu L, et al. High-speed railway clearance intrusion detection with improved SSD network[J]. Applied Sciences, 2019, 9(15): 2981.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	W. Favoreel, "Pedestrian sensing for increased traffic safety and efficiency at signalized intersections," 2011 8th IEEE International Conference on Advanced Video and Signal Based Surveillance (AVSS), 2011, pp. 539-542, doi: 10.1109/AVSS.2011.6027406.

[3]	Advances in Wildlife Crossing Technologies: https://highways.dot.gov/public-roads/septoct-2009/advances-wildlife-crossing-technologies.

[4]	Protection Detection: Making Roads Safe for Drivers and Wildlife: https://onlinepubs.trb.org/onlinepubs/webinars/201118.pdf.

[5]	F. Liu et al., "Integrated Sensing and Communications: Towards Dual-functional Wireless Networks for 6G and Beyond," in IEEE Journal on Selected Areas in Communications, doi: 10.1109/JSAC.2022.3156632.

[6]	T. S. Rappaport, G. R. MacCartney, M. K. Samimi and S. Sun, "Wideband Millimeter-Wave Propagation Measurements and Channel Models for Future Wireless Communication System Design," in IEEE Transactions on Communications, vol. 63, no. 9, pp. 3029-3056, Sept. 2015, doi: 10.1109/TCOMM.2015.2434384.

[7]	C. Han, Y. Bi, S. Duan and G. Lu, "Rain Rate Retrieval Test From 25-GHz, 28-GHz, and 38-GHz Millimeter-Wave Link Measurement in Beijing," in IEEE Journal of Selected Topics in Applied Earth Observations and Remote Sensing, vol. 12, no. 8, pp. 2835-2847, Aug. 2019, doi: 10.1109/JSTARS.2019.2918507.

[x]	Eradication of elephant mortality and injury due to railway accidents through automatic tracking and alert system in IEEE Conference Publication, IEEE Xplore

[y]	Impact of wild animals (deer and bears) on train operations 210616_KO_Animal2.pdf (jrhokkaido.co.jp) (in Japanese)

[z]	Rail Industry Safety Induction Handbook: https://railsafe.org.au/__data/assets/pdf_file/0009/32022/Rail-Industry-Safety-Induction-RISI-Handbook-V5.1.pdf

* * * Next Change * * * * (all new)

5.x	Use case of sensing for railway intrusion detection

5.x.1	Description

Extensive railway deployment and the changing wildlife habitat area due to the changing global environment has led to increase of crash of wildlife to trains. Once a crush happens, its recovery costs, takes time, and impairs convenience [x, y]. Such a crush should be avoided, but it appears difficult to proactively predict wildlife's intrusion onto railway track. It's different from e.g., weather forecast. Passively detecting wildlife's intrusion onto railway track appears an option to take. Monitoring with cameras serves the same purpose. However, this requires LOS (i.e., line of sight) and a dense deployment of cameras, which is not necessarily efficient.

5.x.2	Pre-conditions

gNBs are deployed near and along a railway track which enable the mobile operator to constantly sense the railway including intruder (e.g. pedestrians and animal).For sensing, signaling transmitted by a gNB is influenced or bounced by objects around the railway and then monitored by the gNB and other gNBs. Sensing result is being notified to a railway operator by the mobile operator. The railway operator knows locations of trains.

The assumption of this use case is the following:

-	There is at least 300km train line as depicted in the figure 5.x.2-1[z] owned by railway operator. The safe place is a place where a person and their equipment cannot be struck by rail traffic, which is used for minimizing damage caused by possible railway accident or crash and for ensuring safe operation of railway. The danger zone is anywhere within 3m horizontally from the nearest track.

-	The typical size and velocity of intruder and train in this use case are described in the Table 5.x.2-1.

Table 5.x.2-1

|  | Size (Length x Width x Height) | Velocity |
| --- | --- | --- |
| Intruder | Pedestrian(Adult): 0.5m x 0.5m x 1.75m | 5km/h |
|  | Animal(Sheep/deer): 1.5m x 0.5m x 1 m | 5km/h |
| Trains | 24m x 3.5m x 3 m | 100km/h - 350km/h |

When the intruder standing at the outermost side of safe place starts walking on the danger zone, it means the intrusion happens. The distance that intruder move perpendicular to the railway track is more sensitive for road safety, compared to the distance parallel to the railway track.

Figure 5.x.2-1

5.x.3	Service Flows

Figure 5.x.3-1 Railway intrusion detection

- gNBs are deployed near and along a railway track. In order to acquire the sensing information of railway, railway operator requests sensing service from mobile operator. The mobile operator configures the gNBs along the train line to perform sensing. Suddenly, an intruder (e.g. pedestrian or animal) is walking on the danger zone.

- The sensing measurement data is reported from gNBs and further processed into the sensing results by the core network. The mobile operator exposes the sensing results to the railway operator. Based on the sensing results, the location of the intruder can be estimated..

3.	Trains running on the railway track measure their own location and velocity. These trains inform those information to a controller of the railway operator.

4.	The controller identifies a train that is affected by an intruder based on the sensing results from mobile operator and train's location and velocity.

5.	The controller orders the train to slow down or stop. In addition, the staff working for railway operator immediately responds to the emergency. The intruder leaves the danger zone safely.

5.x.4	Post-conditions

The controller judges the intruder is gone and safety can be ensured. The controller permits the train to start again or speed up.

5.x.5	Existing feature partly or fully covering use case functionality

TBD.

5.x.6	Potential New Requirements needed to support the use case

[PR. 5.x.6 - 001] The 5G system shall enable the core network to collect and aggregate sensing measurement data from gNBs, and expose a suitable API to provide the information regarding sensing results to a third party.

[PR. 5.x.6 - 002] The 5G system shall be able to support the following KPIs:

Table 5.4.6-1: Performance requirements for railway intrusion detection

| Scenario | Distance Accuracy ([95%] confidence level) |  | Velocity Accuracy ([95%] confidence level) |  | Resolution |  | Max allowed Sensing service latency | Refreshing rate |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Horizontal Accuracy | Vertical Accuracy | Horizontal Accuracy | Vertical Accuracy | Horizontal Resolution | Vertical Resolution |  |  |
| Intrusion detection on a railway (Note 1) | ≤1.5m (Note 2) | - | - | - | - | - | ˂1.5s (Note 3) | ≥10Hz |
| Note 1: the typical size (Length x Width x Height) of pedestrian is 0.5m x 0.5m x 1.75m and the typical size of animal is 1.5m x 0.5m x 1 m. The length of railway depends on the real environment. Note 2: the width of the danger zone horizontally from the nearest track is within 3m [z]. Note 3: in order to ensure passenger safety if any hazardous situation is detected, this is the latency for 5G system to sense the intrusion and report the sensing information on the presence of an intrusion(e.g. pedestrian/animal) to railway management departments. |  |  |  |  |  |  |  |  |

* * * End of Changes * * * *