3GPP TSG SA WG 1 Meeting #99e 	S1-222108r4

Electronic Meeting, 22 August – 1 September 2022	(revision of S1-222108r3)

Source:	Nokia, Nokia Shanghai Bell, Huawei?, ZTE

pCR Title:	Pseudo-CR on harmonised KPIs for sensing scenarios

Draft Spec:	3GPP TR 22.837

Agenda item:	7.2

Document for:	Approval

Contact:	Hideaki Takahashi (hideaki.takahashi@nokia.com)

Abstract: This contribution proposes harmonized KPIs applicable to all use cases for integrated sensing and communications.

1. Proposal

It is proposed to agree the following changes to 3GPP TR 22.837 and for every use case to use the same KPI table.

* * * First Change * * * *

# 7	Consolidated potential requirements and KPIs

## 7.1	Consolidated potential requirements

## 7.2	Consolidated potential KPIs

Table 7.2-X	Performance requirements for integrated sensing and communication scenarios

| Scenario | Sensing service area (indoor, outdoor, etc.) | Positioning accuracy by sensing (95% confidence level) |  | Velocity estimation accuracy by sensing (95% confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [min] | Missed detection/ false alarm by sensing [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Horizontal [m] | Vertical [m] | Horizontal [km/h] | Vertical [km/h] | Size resolution (horizontal/ vertical) [m x m] | Velocity resolution (horizontal/ vertical) [km/h x km/h] |  |  |  |