3GPP TSG SA WG 1 Meeting #99e 	S1- 222141

Electronic Meeting, 22 August – 1 September 2022	(revision of S1-22xxxx)

Title:	Use case: Enhanced Positioning Service using Satellite Access

Agenda Item:	7.8

Source:	CATT, China Telecom

Contact:	Qing Wan (wanqing1@cictmobile.com)

Abstract: This document proposes a use case for enhanced positioning and navigation service to the users through 3GPP satellite access and potential requirements for TR22.865 v0.0.0 (FS_5GSAT_ph3).

* * * First Change * * * *

References

[x1]	Hamza Benzerrouk , “Iridium Next LEO Satellites as an Alternative PNT in GNSS Denied Environments”, June 17, 2019 (https://insidegnss.com)

[x2]	“The human cost of disasters: an overview of the last 20 years (2000-2019)”，https://www.undrr.org/publication/human-cost-disasters-overview-last-20-years-2000-2019

[x3]	3GPP TS 22.261: "Service requirements for the 5G system ".

* * * Next Change * * * *

X.1 Enhanced Positioning Service using Satellite Access

X.1.1	Description

During natural disasters, the recovery of communication services and the acquisition of the survivors’ location are important to aid the rescue activities. Normally, satellite communication networks and standalone GNSS are utilized to serve communication and positioning independently. With the integration of satellite access in 5G systems, it’s possible to provide both communication and positioning services by 5G system together to address the cases that only GNSS cannot provide reliable positioning service (e.g. poor GNSS signal, limited visible satellites). Meanwhile, the positioning performance like accuracy can be improved with the assistance of 5G satellites (e.g. LEO), network information, etc.[x1]

Indonesia is famous for its extraordinary natural landscapes attracting millions of tourists around the world. Meanwhile, it is widely recognized as one of the most disaster-prone countries in the world according to data released by the United Nations International Disaster Reduction Agency (UN-ISDR)[x2]. Any a disaster (e.g. earthquake, tsunamis) will impact tens of millions of people, who may get support from the sustainable and reliable communication service and positioning service of 5G systems.

X.1.2	Pre-conditions

Bali, Indonesia is covered by terrestrial access network of Operator TerrA and satellite access network of Operator SatA, which share the core network of TerrA deployed in Jakarta with mutual agreement. 5G communication service and positioning service have been launched all through Indonesia.

GNSS (e.g. GPS, BeiDou) are allowed to use in Indonesia, but are independent from 5G satellite constellation.

It is assumed that UEs served by TerrA network are capable of 5G satellite access and integrated with different types of GNSS receivers.

X.1.3	Service Flows

A tsunami strikes Bali and has destroyed most infrastructures and terrestrial access networks. The core network is still in service without damage.

The awake survivors will initiate an emergency call or send an emergency message to report personal information, injuries, and surrounding conditions to Indonesia Rescue Center for rescue requests through SatA access network. During the interaction, the precise location information (e.g. accurate latitude) of the survivors is requested to report to Rescue Center from UE or/and the network within the requested response time of local regulatory requirements. The location of the survivors and rescue personnel will be sent to the survivors as well for preparation.

All powered-on UEs will autonomously update registration in TerrA’s network using satellite access. The network identifies the areas where the devices are located and authorizes Rescue Center to fetch real-time devices’ location and tracing log during the Rescue UAV or Helicopter searching for the survivors in a coma or not able to report emergency information actively.

X.1.4	Post-conditions

The locations of the awake survivors are identified as compliant with regulatory requirements.

The location information of powered-on devices is available in Rescue vehicles.

X.1.5	Existing features partly or fully covering use case functionality

Regarding TS 22.261[x3] as below, UE using satellite access shall have the capability to offer the location, and 5G system needs to determine the location for service, but not consider the situation that the location cannot be decided by UE alone.

A UE supporting satellite access shall be able to provide or assist in providing its location to the 5G network.

A 5G system with satellite access shall be able to determine a UE's location in order to provide service (e.g. route traffic, support emergency calls) in accordance with the governing national or regional regulatory requirements applicable to that UE.

The legacy requirements for positioning service defined in TS 22.261 [x3] are not well adapted to all types of UE with satellite access considering the satellite characteristics (e.g. latency).

The 5G system shall provide 5G positioning services in compliance with regulatory requirements.

The 5G network shall be able to request the UE to provide its position-related-data on request—together with the accuracy of its position—triggered by an event or periodically and to request the UE to stop providing its position-related data periodically.

The 5G system shall support mechanisms to determine the UE’s position-related data for period when the UE is outside the coverage of 3GPP RAT-dependent positioning technologies but within the 5G positioning service area (e.g. within the coverage of satellite access).

The 5G System with satellite access shall be able to negotiate the positioning methods according to the operator's policy or the application’s requirements or the user's preferences and shall support mechanisms to allow the network or the UE to trigger this negotiation.

X.1.6	Potential New Requirements needed to support the use case

[PR X.1.6-001] Subject to regulatory requirements and operator policy, the 5G system with satellite access shall provide 5G positioning service to UEs only using satellite access by 3GPP RAT-dependent positioning technologies.

[PR X.1.6-002] The 5G system with satellite access shall support mechanisms for the network to select positioning method regarding application’s requirements, access type (e.g. NR, Satellite access), UE positioning capability and availability, and user preference.

* * * End of Changes * * * *