3GPP TSG-SA WG1 Meeting #99e 	S1-222143r5

Electronic Meeting, 22 August – 1 September 2022	(revision of S1-22xxxx)

Title:	New use case: Ambient IoT in Smart Agriculture

Agenda Item:	7.3

Source:	vivo

Contact:	        Huan Li <huan.li@vivo.com>

Abstract: This document provides a Text Proposal for the use case about using Ambient IoT services in sensor networks for smart agriculture, smart product line and smart home.

# Text Proposal to TR 22.840

**************** Start of Change ******************

## 5.x	Use Ambient IoT service in  Smart Agriculture

### 5.x.1	Description

Ambient power-enabled IoT devices can be used in Smart Agriculture to monitor the environment and control the facilities such as irrigation system and temperature control system.

Farm A establishes a smart greenhouse for tomato planting. Some sensing Ambient power-enabled IoT devices are placed in the smart greenhouse to monitor the air temperature and humidity, carbon dioxide concentration, light, soil temperature, humidity and PH. Some operation Ambient power-enabled IoT devices are placed in the smart greenhouse to control the window and irrigation system. A pico cell or a mobile phone (with subscription of Operator O) is placed in the smart greenhouse to communicate with the Ambient IoT devices.

### 5.x.2	Pre-conditions

Pre-conditions of Farm A:

Farm A establishes a smart greenhouse for tomato planting. Some sensing Ambient power-enabled IoT devices are placed in the smart greenhouse to monitor the air temperature and humidity, carbon dioxide concentration, light, soil temperature, humidity and PH. Some operation Ambient power-enabled IoT devices are placed in the smart greenhouse to control the window and irrigation system. A pico cell or a mobile phone (with subscription of Operator O) is placed in the smart greenhouse to communicate with the Ambient IoT devices.

Pre-conditions of Factory B:

Factory B establishes a smart product line. Some sensing Ambient power-enabled IoT devices are placed in the smart product line to monitor the position, liquid level, humidity, temperature and pressure. Some operation Ambient power-enabled IoT devices are placed in the smart product line to control the manipulators. A pico cell or a mobile phone (with subscription of Operator O) is placed in the smart product line to communicate with the Ambient IoT devices.

Pre-conditions of smart home:

Alice establishes a smart home network in her apartment. Some sensing Ambient power-enabled IoT devices are placed in her home to monitor the temperature, humidity and brightness in her home. Some operation Ambient power-enabled IoT devices are placed to control the light and air conditioner. Alice’s mobile phone (with subscription of Operator O) is used to communicate with the Ambient IoT devices.

The mobile phone or the pico cell placed in the smart greenhouse is configured by Operator O to perform the 5GS Ambient IoT service.

### 5.x.3	Service Flows

- The mobile phone or the pico cell placed in the smart greenhouse is configured by Operator O to perform the 5GS Ambient IoT service.

- Operator O’s 5GS authenticates Ambient IoT devices in Farm A. The communication between the Ambient IoT device and 5GS are transferred through the mobile phone or the pico cell configured to perform 5GS Ambient IoT service.

- Periodically, or upon request from the application server of Farm A, the mobile phone or the pico cell placed in the smart greenhouse obtains the environmental monitoring information from the sensing Ambient IoT devices and sends to the application server of Farm A.

- Based on the received information and preconfigured logic, the application server requests 5GS to send control sigalling to the operation Ambient IoT devices to control their operations, e.g. open or close the window of the greenhouse, start or close the irrigation system.

- The 5GS transfers the control signalling from the application server to Ambient IoT devices through the mobile phone or the pico cell.

### 5.x.4	Post-conditions

Thanks to the Ambient IoT service provided by the 5G system, Smart Agriculture automatically takes care of the plants.

### 5.x.5	Existing features partly or fully covering the use case functionality

None.

### 5.x.6	Potential New Requirements needed to support the use case

### 5.x.6.1 Service requirements for use of Ambient IoT devices in  Smart Agriculture

[PR.5.x.6.1-001] The 5G system shall support communication with Ambient IoT device.

[PR.5.x.6.1-002] The 5G system shall be able to support direct communication between authorized UE(s) and Ambient IoT devices.

[PR.5.x.6.1-003] The 5G system shall be able to authenticate an Ambient IoT device.

### 5.x.6.2 KPIs for the use of Ambient IoT devices in Smart Agriculture

[PR.5.x.6.2-001] The 5G system shall be able to provide Ambient IoT service with following KPIs:

Table 5.x.6.2-1 Ambient_IoT KPI for  Smart Agriculture

| Scenario | Ambient IoT communication service latency | Ambient IoT service availability | Max. instantaneous device power consumption | Payload Size (UL) | Service bit rate: user-experienced data rate | communication range |
| --- | --- | --- | --- | --- | --- | --- |
| Smart Agriculture | [2] s | 99.9% | [100-200] µW | <[1000] bits | [< 1 kbit/s] | [10-50] m |

Editor’ Note: It is FFS whether “Max. instantaneous device power consumption” is included or not, which depends on SA1’s final decision.

************* End of Change ***************